| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F1101 |
| --- | --- |

| Title: | Description of Core Experiment 1 (CE1) on sub-PU processing |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Shinya Shimizu | Email: | shimizu.shinya@lab.ntt.co.jp |
| Source: | JCT-3V CE1 Coordinators |  |  |

_____________________________

# Abstract

This document describes Core Experiment 1 (CE1) of the development of 3D extensions for HEVC. This Core Experiment investigates sub-PU level processing which performed in inter-view motion prediction and view synthesis prediction.

# Participants

| Participant | Contact | Email | Proponent |
| --- | --- | --- | --- |
| NTT | S. Shimizu | shimizu.shinya@lab.ntt.co.jp | P |
| MediaTek | Y.-L. Chang Y.-W. Chen Y.-W. Huang | yulin.chang@mediatek.com yiwen.chen@mediatek.com yuwen.huang@mediatek.com |  |
| Qualcomm | X. Zhao L. Zhang Y. Chen H. Liu | xinzhao@qti.qualcomm.com lizhang@qti.qualcomm.com cheny@qti.qualcomm.com hongbinl@qti.qualcomm.com | P |
| Sharp | T. Ikai | Ikai.tomohiro@sharp.co.jp |  |
| Zhejiang University | Y. Zhang P. Lu | felixzyc@gmail.com yiqiu8932@gmail.com |  |

(P = proponent, C = cross checker)

# Tools under Investigation

The tools to be investigated in this Core Experiment require sub-PU based processing. There are two coding tools; Sub-PU level inter-view motion prediction (SPIVMP) and view synthesis prediction (VSP). SPIVMP performs motion compensation for each 8x8 sub-PU, whereas VPS performs disparity compensation for each either 8x4 or 4x8 sub-PU.

The tools to be investigated can be categorized into the following 2 categories. All separable tools should be evaluated separately according to the following categorization. Reports on combined tools are also encouraged.

- Default motion vector for SPIVMP (JCT3V-F0127)

- Sub-PU level deblocking/motion parameter storing (JCT3V-F0135, JCT3V-F0176)

## JCT3V-F0127: CE3 related: Simplifications to sub-PU level inter-view motion prediction

In this contribution, it is proposed to derive default motion parameter from the block in base view which corresponds to the center pixel of the current PU, and to assign the default motion parameter to the sub-PU where motion parameter is unavailable in the base view.

- Complexity impacts, especially the number of memory access, will be studied.

## JCT3V-F0135: CE3 related: HEVC compatible de-blocking for sub-PUs

This contribution proposes to apply de-blocking filter to sub-PU boundary by introducing transform unit boundaries before the de-blocking process.

- The main objective is to reduce blocking artifacts in sub-PU boundaries and to improve subjective quality. Subjective evaluation will be performed.

## JCT3V-F0176: CE1-related: Motion parameters stored at VSP-coded blocks

This contribution proposes to store the finally used disparity parameters for each sub-PU within the VSP-coded PU.

- This is similar to the approach of 3D-AVC. Text and implementation will be crosschecked.

# Core Experiment Conditions

## Software

Experiments will use the HTM 9.0 software, as recommended in JCT3V-F1100. Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

## Test Sequences

Test sequences will be used according to common test conditions (CTC) [1].

## Coding Conditions

The experiments are conducted under CTC [1].

## Evaluation Criteria

Evaluation will be done according to CTC [1].

  - Coding Performance Measurements: PSNR values shall be computed for the decoded texture views relative to original texture views and for the synthesized views relative to the synthesized views based on uncompressed texture and depth. The 4-point BD-rate according to common test conditions is used to report the overall simulation results.

  - Complexity measurements: For the complexity measurement, the reference software and the reference software with the proposed method implemented will be executed on the same machine with the same configuration and the computational time will be measured. A time ratio will then be calculated between the reference software and the reference software with the proposed method implemented.

Thorough complexity analysis (in terms of number of computation, memory access, memory usage, etc) compared to the HTM v9.0 (and traditional MC) shall also be reported.

For the sub-PU level de-blocking proposal, subjective evaluation will be performed during the meeting. The stereo AVI files for the synthesized stereo pairs specified in [1] shall also be prepared.

## Timeline

2013/11/08		Finalize CE description
2013/12/16		Make source code and simulation results available for all CE1 participants

2014/01/03		Register and upload contributions to the JCT-3V document repository

2014/01/11		Start the 7th JCT-3V meeting

# Reference

- D. Rusanovksyy, A. Vetro, and K. Müller, "Common Test Conditions for 3D Video Extensions Development," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-F1100, Geneva, CH, 2013.