| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F1105 |
| --- | --- |

| Title: | Description of Core Experiment 5 (CE5) on Depth Intra Modes |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Hongbin Liu | Tel: Email: | +86 10 57760696 hongbinl@qti.qualcomm.com |
| Source: | JCT-3V Core Experiment Coordinator |  |  |

_____________________________

# Abstract

This document describes Core Experiment 5 on depth intra modes to be performed for the 7th JCT-3V meeting in San Jose, United States.

# Introduction

The goal of this Core Experiment (CE) is to investigate methods for depth intra modes proposed to the 6th JCT-3V meeting. Tools under test will be evaluated according to their impact on both compression efficiency and complexity.

# Participants

| Participant | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| MediaTek Inc. | Yi-Wen Chen Jian-Liang Lin Yu-Wen Huang Yu-Lin Chang Kai Zhang | yiwen.chen@mediatek.com jl.lin@mediatek.com yuwen.huang@mediatek.com yulin.chang@mediatek.com kai.zhang@mediatek.com | X | X |
| Qualcomm | Hongbin Liu Ying Chen Li Zhang | hongbinl@qti.qualcomm.com cheny@qti.qualcomm.com lizhang@qti.qualcomm.com | X | X |
| Samsung | Jin Young Lee Min Woo Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | X | X |
| Tsinghua University | Weiran Li Yun He | liweiran0@gmail.com | X | X |
| Hisilicon Technologies | Xiaozhen Zheng Yongbing Lin Xu Chen Jianhua Zheng | zhengxiaozhen@huawei.com xiaozhenzheng@gmail.com linyongbing@huawei.com anderson.chen@huawei.com zhengjianhua@huawei.com | X | X |
| HHI | Philipp Merkle | philipp.merkle@hhi.fraunhofer.de |  | X |
| RWTH Aachen University | Fabian Jäger | jaeger@ient.rwth-aachen.de |  | X |
| Zhejiang University | Peng Lu | yiqiu8932@gmail.com |  | X |

(P=proponent, C=cross checker)

# Tools Under Test

## Category 1: Segment-wise DC coding (SDC)

## JCT3V-F0126: Generic SDC for all Intra modes in 3D-HEVC

This contribution proposes to generalize SDC to all HEVC intra modes and additional depth intra modes. In the generalized SDC, one DC residual is coded for each partition within a prediction unit, and transform and quantization is skipped.

## JCT3V-F0149: Simplified depth inter mode coding

The contribution proposes to apply inter SDC only to 2Nx2N partition. In addition, to find more accurate residual values, the proposed method tests three candidates, including DC, DC-1, and DC+1, in inter SDC coding.

## JCT3V-F0170: Simplification on SDC planar coding

This contribution proposes two simplification methods to reduce the storage cost of prediction sample block in SDC planar mode for the case of 64x64 coding block. In method 1, 2:1 sub-sampling is used to reduce the prediction sample block to size 32x32. In method 2, the first 32 samples from the left-column or the top-row neighbouring samples are used to derive a 32x32 prediction sample block. In both methods, the SDC residual is added to the 32x32 prediction sample block to generate a 32x32 reconstruction sample block, and each sample value in the 32x32 reconstruction sample block is used to fill the corresponding 2x2 block in 64x64 reconstruction block.

Test the case where 64x64 planar in SDC is disabled.

## JCT3V-F0113: Bin reduction for SDC residual coding

This contribution proposes to constrain the DC residual magnitude of intra SDC within a pre-defined range (e.g., 5) so that the bin number for coding the syntax element associated with the SDC residual could be reduced.

Proponents ask to also test the following cases: the constraint will be tested on the residual coding of intra and inter SDC within a pre-defined range N. (N= 1, 5, 7, 11 or any non-negative integer which needs to be tested).

## Category 2: Depth Modeling Mode (DMM)

## JCT3V-F0153: Simplification of DMM pattern generation and signaling

This contribution proposes an alternative DMM pattern generation and signaling method. A prediction block can have two types of partitions: rectangle partition and non-rectangle partition. For rectangle partition, it can be further classified to horizontal pattern and vertical pattern. For non-rectangle case, the partitions can be categorized by the partition line’s slope and orientation (left-down or right-up). Five slope values including 1:1, 2:1, 1:2, 4:1 and 1:4 are allowed. For both rectangle partition and non-rectangle partition, an offset is encoded to indicate the distance between the partition line and central partition line. The central partition line has the same slope and orientation as the partition line, and divides the prediction block into two partitions with equal size.

Test more slopes.

# Core Experiment Conditions

## Software

Core Experiment CE5 will use the 3D-HTM 9.0 software that is described in the common test conditions (CTC), JCT3V-F1100. Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

## Test Sequences, Bit Rates and Coding Conditions

The CE will use test sequences, configuration and conditions that are described in the common test conditions (CTC), JCT3V-F1100. Moreover, proponents and cross checkers are required to provide simulation results for the following two configurations:

- Random access configuration as specified in JCT3V-F1100.

- All-intra configuration (configuration files to be provided by CE coordinator).

## Evaluation of CE Results

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are to be used in this core experiment. A corresponding Excel sheet for reporting the simulation results will be provided by the CE coordinator.

Coding Performance Measurements: PSNR values shall be computed for the decoded texture views relative to original texture views and for the synthesized views relative to the synthesized views based on uncompressed texture and depth. The 4-point BD-rate according to common test conditions is used to report the results for random access as well as all-intra configuration.

Complexity measurements: For the complexity measurement, the reference software and the reference software with the proposed method implemented will be executed on the same machine with the same configuration and the computational time will be measured. A time ratio will then be calculated between the reference software and the reference software with the proposed method implemented.

## Timeline

2013/11/08		Final CE description available
2013/12/16		Source code, simulations results and draft text available to all participants
2014/01/03		Document registration and upload deadline
2014/01/11-2014/01/17   The 7th JCT-3V meeting