| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0005 |
| --- | --- |

| Title: | JCT-3V AHG Report: MV-HEVC and 3D-HEVC Software Integration (AHG5) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-3V |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Gerhard Tech (Fraunhofer HHI) Li Zhang (Qualcomm) YuLin Chang (Mediatek) Krzysztof Wegner (Poznan Univ. of Tech.) | Email: | gerhard.tech@hhi.fraunhofer.de lizhang@qti.qualcomm.com yulin.chang@mediatek.com kwegner@multimedia.edu.pl |
| Source: | AHG |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AhG on 3D-HEVC Software Integration that have taken place between the 6th JCT-3V meeting in Geneva and the 7th JCT-3V meeting in San José. Activities focused on the integration of tools adopted at the 6th meeting into a common code base.

# Mandates

| Title | Chairs | Mtg |
| --- | --- | --- |
| MV-HEVC and 3D-HEVC Software Integration (AHG5) (jct-3v@lists.rwth-aachen.de) - Coordinate development of the HTM software and its distribution to JCT-3V members - Produce documentation of software usage for distribution with the software - Prepare and deliver HTM 9.0 software version and the reference configuration encodings according to JCT3V-F1100 based on common conditions suitable for use in most core experiments (expected within four weeks after the meeting). - Prepare and deliver HTM 9.1 software that include additional items not integrated into the 9.0 version (expected within one week after the 9.0 software release). - Prepare and deliver HTM 9.2 software that include 3D-HEVC HLS not integrated into the 9.1 version (expected within two weeks after the 9.1 software release). - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with MV-HEVC Draft and 3D-HEVC Test Model editing AhG to identify any mismatches between software and text. | G. Tech L. Zhang (co-chairs) Y. Chang K. Wegner (vice chairs) | N |

# HTM tool integration

Development of the software was co-ordinated with the parties needing to integrate changes.

The distribution of the software was announced on the JCT-3V e-mail reflector and the software was made available through the SVN server:

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/

Anchor bitstreams have been created and uploaded to:

ftp.hhi.fraunhofer.de; login: mpeg3dv_guest; path: /MPEG-3DV/HTM-Anchors/

Multiple versions of the HTM software were produced and announced on the JCT-3V email reflector. The following sections give a brief summary of the integrated tools and achieved coding gains.

## Versions HTM-9.0 / HTM-9.0r1

Starting point for development of HTM-9.0 was HTM-8.2. Development of HTM-9.0 was conducted in three parallel tracks each performing sequential integration. Development of each branch has been supervised by one software coordinator. Software of all three tracks was merged by the software coordinators. Afterwards F0110 has been integrated.

After release of HTM-9.0, a bug critical under CTC has been reported. Therefore an additional version HTM-9.0r1 has been issued.

### Integrated items

Track 1 (VSP, RP, coordinated by Li Zhang):

- JCT3V-F0123/JCT3V-F0108 CE4: Results on improved advanced residual prediction

- JCT3V-F0161 CE4: Coding of weighting factor of advanced residual prediction

- JCT3V-F0105 CE4: ARP reference picture selection and its availability check

- JCT3V-F0109/JCT3V-F0120 CE1: A simplified block partitioning method for VSP

- JCT3V-F0102 CE1: VSP partitioning for AMP

Track 2 (Merge, coordinated by Yulin Chang):

- JCT3V-F0125 CE3: Inter-view motion vector prediction for depth coding

- JCT3V-F0111 CE1: Simplified view synthesis prediction

- JCT3V-F0104 CE3: Removal of redundancy on VSP, ARP and IC

- JCT3V-F0150 CE3: MPI candidate in depth merge mode list construction

- JCT3V-F0093 CE3: Results on simple merge candidate list construction for 3DV

Track 3 (Depth Intra + IC Flags, coordinated by Gerhard Tech):

- JCT3V-F0132/JCT3V-F0171 CE5: Unification of delta DC coding for depth intra modes

- JCT3V-F0159 CE5: Fast depth LUT application method to intra modes for depth data

- JCT3V-F0160 Non-CE: Illumination compensation flag coding

- JCT3V-F0147 CE5: DMM simplification and signalling

- JCT3V-F0151 Removal of IC in depth coding and IC flag signalling in 3D-HEVC

Others

- JCT3V-F0110 CE3: Sub-PU level inter-view motion prediction

- Minor fixes.

Fix in HTM-9.0r1

- Disabled sub-PU level inter-view motion prediction.

### Coding performance

MV-HEVC: The coding results for MV-HEVC are identical to results obtained from version HTM-8.2.

3D-HEVC: HTM-9.0r1 vs. HTM-8.2 (CTC, three view configuration)

|  | video video rate | video total rate | synth total rate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | -3,2% | -2,9% | -2,6% | 89,3% | 99,1% | 100,9% |
| Kendo | -2,5% | -2,5% | -2,1% | 90,7% | 108,2% | 101,7% |
| Newspaper_CC | -1,9% | -1,8% | -1,5% | 87,7% | 113,1% | 101,3% |
| GT_Fly | -2,2% | -2,0% | -1,9% | 98,3% | 109,2% | 103,4% |
| Poznan_Hall2 | -3,0% | -2,8% | -2,8% | 89,4% | 104,2% | 103,8% |
| Poznan_Street | -1,5% | -1,5% | -1,4% | 89,3% | 113,2% | 97,7% |
| Undo_Dancer | -1,1% | -1,1% | -1,2% | 99,7% | 109,0% | 102,0% |
| Shark | -1,6% | -1,4% | -1,4% | 99,3% | 108,3% | 102,2% |
| 1024x768 | -2,5% | -2,4% | -2,1% | 89,2% | 106,8% | 101,3% |
| 1920x1088 | -1,9% | -1,8% | -1,7% | 95,2% | 108,8% | 101,8% |
| average | -2,1% | -2,0% | -1,8% | 93,0% | 108,1% | 101,6% |

### History of revisions

The following sections give a brief summary of average rate savings and runtimes of the different software revisions produced during the integration the period. Results are not cross verified. Simulations were performed using CTC for the three view case.

#### Track 1 (VSP, RP)

#### Track 2 (Merge, and others)

#### Track 3 (Depth Intra + IC Flags)

#### Merge of tracks

## Version HTM-9.1

MV-HEVC 6 HLS has been integrated in a single track. After integration proponents have been asked for review and further integrations.

### Integrated items

- H_MV_LAYER_WISE_STARTUP Basic version of layer wise startup.

- H_MV_FIX_SKIP_PICTURES Fix related to skipping pictures.

- H_MV_6_HRD_O0164_15 Multilayer HRD (without SEI)

- H_MV_6_HRD_O0217_13 Sub-DPB based DPB operations, Specify a separate DPB capacity for each layer – no sharing of capacity across layers – each layer has its own parameters (max pictures, max latency, max reordering). Specify distinct parameters for each "output layer set"

- H_MV_6_SHVC_O0098_36 Scaled ref layer offset Modify signalling of scaled reference layer offsets to allow signalling of any lower layer, rather than just a direct reference layer, in order to enable alignment of auxiliary pictures.

- H_MV_6_GEN_0153_28 Output highest layer add a flag in the VPS to indicate if startup process should output the highest available layer if the target output layer is not available. (only flag)

- H_MV_6_PS_0109_25 To change default_one_target_output_layer_flag to a two-bit default_one_target_output_layer_idc, and reserve the values 2 and 3

- H_MV_6_O0226_37 Mod Tile WPP, Modifications to the VUI indicators of tile and WPP alignment related syntax elements, from the r1.

- H_MV_6_RALS_O0149_11 Prop2, A new slice_reserved_flag is taken into use to indicate if a base-layer IDR picture initiates the layer-wise start-up process. (not thoroughly tested, since coding IDR refresh is currently broken in HM 12.0 )

- H_MV_6_PSEM_O0142_3 Conditional extension syntax, a structure to be used to switch whatever extensions we define in SPS, not necessarily committing to having these extensions be separate for each extension, but the current plan unless decided otherwise is to use one flag for range extensions syntax presence and one flag for SHVC+MV-HEVC extension syntax presence

- H_MV_6_FIX_GET_VIEW_ID, Fix getViewId

- H_MV_6_LAYER_ID_32 Layer id 63 reserved

- H_MV_6_PS_O0118_33 visual signal info in vui per layer, Add visual signal information (video_format, video_full_range_flag, colour_primaries, transfer_characteristics, matrix_coeffs) per layer to the VPS VUI, from v2 version of JCTVC-O0118.

- H_MV_6_POC_31_35_38 Various related to POC

- H_MV_6_MISC_O0062_31 POC LSB present, to add a flag in the VPS for each EL to control whether these LSBs are present or not (for IDR pictures), and when not present, the LSBs are inferred to be equal to 0.

- H_MV_6_ILDDS_ILREFPICS_27_34 Various inter layer reference picture derivation

  - O0225 Change derivation of NumActiveRefLayerPics to consider max_tid_il_ref_pics.

  - O0120 Modify inter-layer reference picture list default construction to incorporate max temporal sub-layers per layer syntax elements in VPS extension

  - O0225 signalling of max_tid_il_ref_pics per layer

- H_MV_6_ILDSD_O0120_26  Add syntax elements to signal max temporal sub-layers for each layer in the VPS, with a gating flag

- H_MV_6_PS_O0223_29 Add a flag in VPS VUI to indicate cross layer pic type alignment.  Move cross_layer_irap_aligned_flag to VPS VUI and make presence condition on added flag

- H_MV_6_PS_0092_17 Restrict sharing of SPS and PPS across layers to avoid creating problems during sub-bitstream extraction

- H_MV_6_PS_REP_FORM_18_19_20 Various Representation format

  - Change rep format syntax element length to 8 bit as decided later in trac.

  - O0179 Add flag in rep_format( ) syntax structure to control sending of chroma and bit depth related parameters

  - O0096 Modify the SPS syntax for layers with nuh_layer_id > 0 to signal a reference to a rep_format index in the VPS, rather than signalling explicit representation format data in the SPS

  - FIX on N0092 Rep. Format Inferences.

- H_MV_6_PS_O0096_21 Add a gating flag in VPS extension to condition the presence of direct dependency type, with a default type signalled

- H_MV_6_PS_O0109_22 view_id_len, Modify the VPS extension syntax and semantics to replace view_id_len_minus1 with view_id_len, always signal that syntax element, add a constraint that (1<<view_id_len) >= NumViews, and modify view_id_val semantics to infer value of 0 when not present

- H_MV_6_PS_O0109_23 Modify the semantics of profile_ref_minus1[ i ] to replace “shall be less than i” with “shall be less than or equal to i”

- H_MV_6_PS_O0109_24 Move the vps_vui_present_flag to precede vps_vui_offset, and make vps_vui_offset conditional on that flag, from JCTVC-O0109

### Coding performance

MV-HEVC:HTM-9.1 vs. HTM-9.0r1 (CTC, three view configuration)

|  | video 0 video 0 rate | video 1 video 1 rate | video 2 video 2 rate | video total rate |
| --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,1% | 0,1% | 0,0% |
| Kendo | 0,0% | 0,1% | 0,1% | 0,0% |
| Newspaper_CC | 0,0% | 0,1% | 0,1% | 0,0% |
| GT_Fly | 0,0% | 0,0% | 0,0% | 0,0% |
| Poznan_Hall2 | 0,0% | 0,1% | 0,1% | 0,0% |
| Poznan_Street | 0,0% | 0,1% | 0,1% | 0,0% |
| Undo_Dancer | 0,0% | 0,0% | 0,0% | 0,0% |
| Shark | 0,0% | 0,0% | 0,0% | 0,0% |
| 1024x768 | 0,0% | 0,1% | 0,1% | 0,0% |
| 1920x1088 | 0,0% | 0,1% | 0,1% | 0,0% |
| average | 0,0% | 0,1% | 0,1% | 0,0% |

3D-HEVC: HTM-9.1 vs. HTM-9.0r1 (CTC, three view configuration)

Marginal changes as above for MV-HEVC.

## Version HTM-9.2

3D-HEVC 6 HLS has been integrated in a single track.

### Integrated items

- JCT3V-F0136 Comments on camera parameters in 3D-HEVC

- JCT3V-F0045 Constraints on camera parameter signaling

- JCT3V-F0082 On slice-level camera parameter signaling

- JCT3V-F0044 HEVC compatible slice segment header in 3D-HEVC

- JCT3V-F0131/JCT3V-F0139/ Depth Lookup Table Coding for 3D-HEVC

- Move DLT signaling (DLTs for multiple layers) from VPS to PPS

- JCT3V-F0138 3D-HEVC HLS: Inter-view Updating Mechanism for Coding of DLT

### Coding performance

MV-HEVC: No changes.

3D-HEVC: Insignificant decrease of bitrate. ( < 0.01%)

## Branch HTM-9.0r1-F0122

This branch is based on HTM-9.0r1 and includes F0122. Hence, MV-HEVC configuration can be used with VSO enabled.

# Open issues

- Some minor mismatches related to 3D-HEVC HLS.

- MV-HEVC / 3D-HEVC SEI messages missing.

- Some missing items related to MV-HEVC HLS (CLRAS, POC alignment, Auxiliary pictures)

- Other minor issues are listed in the bug tracking system.

# Recommendations

The recommendations of the 3D-HEVC Software integration group are:

- Develop reference software HTM version 10 based on HTM-9.2 and improve its quality

- When a proposal is adopted to discuss how to enable it in the HTM software (e.g. encoder parameter / parameter set flag, or always on).

- Continue to identify bugs and discrepancies with text, and address them.

- Fix open issues.

- Discuss on how to address open issues.