| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0021 |
| --- | --- |

| Title: | CE1: Summary report on Sub-PU processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shinya Shimizu (NTT) 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, Japan | Tel: Email: | +81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

This document is the summary report of Core Experiment 1 (CE1) on sub-PU processing. This Core Experiment investigates simplification and deblocking process for sub-PU level inter-view motion prediction and view synthesis prediction, which perform sub-PU based motion/disparity compensation predictions. Experiments are related to 3D-HTM.There are 3 CE proposals and 5(7) CE related proposals.

# Participants

| Participant | Contact | Email | Proponent |
| --- | --- | --- | --- |
| NTT | S. Shimizu | shimizu.shinya@lab.ntt.co.jp | P/C |
| MediaTek | Y.-L. Chang Y.-W. Chen Y.-W. Huang | yulin.chang@mediatek.com yiwen.chen@mediatek.com yuwen.huang@mediatek.com | P/C |
| Qualcomm | X. Zhao L. Zhang Y. Chen H. Liu | xinzhao@qti.qualcomm.com lizhang@qti.qualcomm.com cheny@qti.qualcomm.com hongbinl@qti.qualcomm.com | P/C |
| Sharp | T. Ikai | Ikai.tomohiro@sharp.co.jp |  |
| Zhejiang University | Y. Zhang P. Lu | felixzyc@gmail.com yiqiu8932@gmail.com |  |
| Ericsson | A. Norkin | andrey.norkin@ericsson.com |  |
| Samsung | J.Y. Lee M.W. Park | Jinyoung79.lee@samsung.com m.w.park@samsung.com | P |
| Kyung Hee University | Y.S. Heo | medist@khu.ac.kr | P |

(P = proponent, C = cross checker)

# Tools under Investigation

In this Core Experiment, the processes in the tools which require sub-PU based processing are investigated are investigated. There are two coding tools; sub-PU level inter-view motion vector prediction (SPIVMP) and view synthesis prediction (VSP). SPIVMP performs motion compensation for each 8x8 sub-PU, whereas VPS performs disparity compensation for each either 8x4 or 4x8 sub-PU.

The following two experiments were set up:

- Default motion vector for SPIVMP

  - JCT3V-G0120

  - JCT3V-G0084 (titled CE3 related)

  - JCT3V-G0147 (related)

- Sub-PU level deblocking/motion parameter storage

  - JCT3V-G0118

  - JCT3V-G0148

  - JCT3V-G0068 (related)

There are many CE-related contributions, which are roughly categorized into the following two categories:

- VSP with residual prediction

  - JCT3V-G0059

  - JCT3V-G0076 (titled CE4 related)

- Others

  - JCT3V-G0069

  - JCT3V-G0077

## List of contributions

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| CE contributions |  |  |  |
| Qualcomm | JCT3V-G0118 | CE1: De-blocking friendly sub-PU processing in 3D-HEVC | Proposal |
| NTT | JCT3V-G0152 | CE1: Crosscheck on De-blocking friendly sub-PU processing in 3D-HEVC (JCT3V-G0118) | Crosscheck |
| Qualcomm | JCT3V-G0120 | CE1: Simplifications to sub-PU level inter-view motion prediction | Proposal |
| MediaTek | JCT3V-G0203 | 3D-CE1: Crosschecking for qualcomm's simplification for sub-PU level inter-view motion prediction | Crosscheck |
| NTT | JCT3V-G0148 | CE1: Motion parameters stored for VSP-coded blocks | Proposal |
| Qualcomm | JCT3V-G0218 | CE1: Crosscheck on Motion parameters stored for VSP-coded blocks (JCT3V-G0148) | Crosscheck |
| CE-related contributions (including related other CE-related contributions) |  |  |  |
| MediaTek | JCT3V-G0059 | 3D-CE1 related: VSP combining inter-view ARP | Proposal |
| NTT | JCT3V-G0185 | 3D-CE1 related: Crosscheck on VSP combining inter-view ARP (JCT3V-G0059) | Crosscheck |
| MediaTek | JCT3V-G0068 | 3D-CE1 related: Deblocking for sub-PU edge | Proposal |
| NTT | JCT3V-G0155 | 3D-CE1 related: Crosscheck on Deblocking for sub-PU edge (JCT3V-G0068) | Crosscheck |
| MediaTek | JCT3V-G0069 | 3D-CE1 related: Restriced bi-prediction for sub-PU | Proposal |
| NTT | JCT3V-G0157 | 3D-CE1 related: Crosscheck on Restriced bi-prediction for sub-PU (JCT3V-G0069) | Crosscheck |
| Samsung | JCT3V-G0076 | 3D-CE4 related: Residual Prediction for View Synthesis Prediction | Proposal |
| NTT | JCT3V-G0154 | 3D-CE4 related: Crosscheck on Residual Prediction for View Synthesis Prediction (JCT3V-G0076) | Crosscheck |
| Samsung | JCT3V-G0077 | 3D-CE1 related: Crosscheck on Restriced bi-prediction for sub-PU (JCT3V-G0069) | Proposal |
| NTT | JCT3V-G0153 | 3D-CE1 related: Crosscheck on MCP Size and DV for Sub-PU Prediction (JCT3V-G0077) | Crosscheck |
| UESTC, HiSlilicon | JCT3V-G0084 | CE3 related: Modifications to motion hole filling in sub-PU level inter-view motion prediction | Proposal |
| Qualcomm | JCT3V-G0214 | CE1 related: Crosscheck on Modifications to motion hole filling in sub-PU level inter-view motion prediction (JCT3V-G0084) | Crosscheck |
| KHU | JCT3V-G0147 | 3D-CE1.h related: Simplification of sub-PU level temporal interview motion prediction | Proposal |
| Qualcomm | JCT3V-G0219 | CE1 related: Crosscheck on Simplification of sub-PU level temporal interview motion prediction (JCT3V-G0147) | Crosscheck |

# Summary of results

## Test (a): [CE] Default motion vector for SPIVMP

In this category, default motion parameters for SPIVMP are investigated. The default motion parameters are utilized on motion hole filling as well as the pruning process of merge candidates. All contribution is aiming to improve the parallel processing capability.

In order to distinguish motion parameters for motion hole filling with those for the pruning process, the former is called as default motion parameters (DMP) and the latter is called as representative motion parameters (RMP). In addition, motion parameters on the block which corresponds to center pixel of the current PU in a reference picture is called as center motion parameters (CMP), and motion parameters with zero motion vectors and zero reference index for both L0 and L1 lists is called as zero motion parameters (ZMP).

TABLE 1: Summary of results on Test (a)

| Doc. | Availability of SPIVMP candidate | DMP | Update of DMP | RMP | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | Dec. time | Cross-check | WD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HTM | At least one sub-PU has available MP | Last available MP | Yes | First available MP | - | - | - | - | - |
| G0120 | Always | CMP or ZMP | No | CMP or ZMP | 0.07% | 0.08% | 103.5% | G0203 | Yes |
|  | At least one sub-PU has available MP | CMP or ZMP | No | First available MP | 0.01% | 0.02% | 99.8% | G0203 | No |
| G0084 | At least one sub-PU has available MP | ZMP | Yes | First available MP | 0.01% | 0.02% | 101.2% | G0214 | Yes |
|  | At least one sub-PU has available MP | (Do)NBDV | Yes | First available MP | 0.01% | 0.00% | 100.6% | G0214 | No |
| G0147 | CMP is available | CMP | No | CMP | 0.00% | -0.01% | 98.9% | G0219 | Yes |

## Test (b): [CE] Sub-PU level deblocking/motion parameter storage

In the current WD, the disparity parameters utilized to fetch depth map are stored in the motion storage in for VSP while sub-PU based motion parameters are stored for SPIVMP. The impact by storing sub-PU based disparity parameters for VSP is investigated. In addition, the impact by applying deblocking filter for sub-PU edges is also investigated.

TABLE 2: Summary of results on Test (b)

| Doc. | Store sub-PU based DVs for VSP | Deblocking filter on sub-PU edges | Type of sub-PU edge | Deblocking edges located other than 8x8 grid | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | Dec. time | Cross-check | WD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HTM | No | No | n/a | No | - | - | - | - | - |
| G0118 | Yes* | Yes (SPIVMP and VSP) | TU | No | -0.29% | -0.18% | 102.8% | G0152 | Yes |
| G0148 | Yes | No | n/a | No | -0.11% | -0.08% | 96.0% | G0218 | Yes |
|  | Yes | Yes (VSP) | TU | No | -0.11% | -0.07% | 99.7% | G0218 | No |
| G0068 | Yes | Yes (SPIVMP and VSP) | PU | No | -0.31% | -0.19% | 102.9% | G0155 | Yes |

* The implementation of G0118 for sub-PU based DV storage for VSP has a minor bug; clipped disparity vectors are stored although such clipping process is not specified in the proposed text and such clipping process might be inappropriate to be specified.

In order to apply deblocking filter to sub-PU edges, G0118 proposes to modify transform tree structure before the deblocking process while G0068 proposes to add a derivation process of sub-PU boundary to the deblocking filter process.

As for the proposed draft text on storing sub-PU based DVs for VSP, G0118 proposes minimum changes relative to the current WD although the view synthesis prediction process in H.8.5.3.3.8 is invoked twice. The draft text proposed by G0068 also requires small changes, but it requires to invoke a prediction sample construction process during motion parameter derivation process. On the other hand, G0148 requires many changes, but the prediction sample construction processes are unified between SPIVMP and VSP.

## Test (c): [CE-related] VSP with residual prediction

In the current WD, VSP merge candidates become available when iv_res_pred_weight_idx is equal to 0; no combination of VSP and ARP is allowed. Such combination of VSP and ARP is investigated in this category.

The difference between G0059 and G0076 is an interpolation filter utilized for VSP, i.e. main depth-based disparity compensated prediction, when ARP is applied. G0059 proposes to use the 7/8-tap interpolation filter in HEVC while G0079 proposed to use a bi-linear interpolation filter as used in the 3D-HEVC residual predictions. For the residual prediction process, both G0059 and G0079 propose to use a bi-linear interpolation filter.

It is reported that there are minor impact on decoding time; however, one of the major concerns is the decoder complexity since a tri-prediction is performed for each 8x4 or 4x8 block. In both proposals, the number of the referred samples is smaller than the normal 8x8 bi-prediction, which requires to access 548 samples (450 luma + 98 chroma) per 8x8 block.

TABLE 3: Summary of results on Test (c)

| Doc. | Interpolation filter for VSP when ARP is applied | # of accessed samples per sub-PU (8x4 or 4x8) | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | Dec. time | Crosscheck | WD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G0059 | HEVC interpolation filter | 228 (main pred.: 108, resi. pred.: 120) | -0.08% | -0.07% | 100.4% | G0185 | Yes |
| G0076 | Bi-linear filter | 172 (main pred.:52, resi, pred.: 120) | -0.06% | -0.06% | 100.2% | G0164 | Yes |

## Test (d): [CE-related] Others

### JCT3V-G0069: Vertical Filtering for View Synthesis Prediction

In HEVC, the bi-prediction for small PU-size, 8x4 or 4x8, is prohibited. This contribution proposes to disable the bi-prediction for sub-PU whose block size is smaller than 8x8 in SPIVMP or VSP in order to align with the base HEVC design. Under the CTC, such bi-prediction is utilized only for the SPIVMP on 16x4/4x16 PUs.

TABLE 4: Simulation results of JCT3V-G0069

| Doc. | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | Dec. time | Crosscheck | WD |
| --- | --- | --- | --- | --- | --- |
| G0069 | 0.00% | -0.02% | 100.2% | G0157 | Yes |

### JCT3V-G0077: MCP Size and DV for Sub-PU Prediction

This contribution contains three SPIVMP-related aspects; 1) MCP size, 2) disparity vector utilized for the motion parameter derivation, and 3) clean-up of sub-PU size syntax element.

In the current WD, a block size of sub-PU can be 12x8 or 8x12 which is not allowed in the base HEVC. In order to align with HEVC, this contribution proposes two different solutions; 1) dividing 12x8(8x12) PU into 8x8 and 4x8 (8x4 and 8x8) blocks, and 2) allowing SPIVMP only to 2Nx2N partition.

This contribution also proposes to set a vertical component of disparity vector utilized in SPIVMP to 0.

The last proposal is to clean-up the sub-PU size syntax, log2_sub_pb_size_minus2, in VPS. It is proposed to change it into log2_sub_pb_size_minus3 with moving from VPS to SPS.

TABLE 5: Simulation results of JCT3V-G0077

| Doc. | SPIVMP on block whose size is not 2Nx2N | 12x8/8x12 MCP | Vertical comp. of DV for SPIVMP | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | Dec. time | Cross-check | WD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HTM | Yes | Yes | Any | - | - | - | - | - |
| G0077 | Yes | No | Any | -0.01% | -0.01% | 107.6% | G0153 | Yes |
|  | No | No | Any | 0.00% | -0.02% | 100.1% |  | Yes |
|  | Yes | No | 0 | -0.03% | -0.04% | 107.4% |  | Yes |
|  | No | No | 0 | -0.03% | -0.06% | 100.1% |  | Yes |

# Recommendations

- To review the relevant input contributions

- To conduct subjective viewing for deblocking related proposals if necessary

- ...