| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0022 |
| --- | --- |

| Title: | CE2: Summary report on motion prediction for depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang Ying Chen | Tel: Email: Tel: Email: | +886-3-5670766 ext. 25314 yulin.chang@mediatek.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

A summary of proposals and cross-check results of CE2 is reported.

# Introduction

A summary of proposals and cross-check results of CE2 is reported.

## Mandates

# Mandates

## CE2

The goals of this core experiment are as follows:

- To further study the coding performance and implementation for the disparity-to-depth conversion for the disparity derived depth candidate [JCT3V-F0107]

- To further study the coding performance for additional texture merging candidate [JCT3V-F0107]

- To further investigate into the coding performance of the Sub-PU based MPI [JCT3V-F0128]

- To further study the benefits of disparity derivation in different ways in terms of processing pipeline [JCT3V-F0195]

- To further investigate ways to improve the inter-view motion vector prediction for depth coding. [JCT3V-F0195]

- To encourage study of the interactions of the above technical proposals.

## Participants

Participants with proposals at this meeting in CE2 are as follows:

## CE2

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Kai Zhang Jicheng An Yu-Lin Chang Yu-Wen Huang | jl.lin@mediatek.com yiwen.chen@mediatek.com kai.zhang@mediatek.com jicheng.an@mediatek.com yulin.chang@mediatek.com yuwen.huang@mediatek.com | P/C |
| Qualcomm | Li Zhang Ying Chen | lizhang@qti.qualcomm.com cheny@qti.qualcomm.com | P/C |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp | C |
| Zhejiang University | Yichen Zhang Peng Lu | felixzyc@gmail.com yiqiu8932@gmail.com | C |
| NTT | S. Sugimoto S. Shimizu | sugimoto.shiori@lab.ntt.co.jp shimizu.shinya@lab.ntt.co.jp | C |
| Samsung | Jin Young Lee Min Woo Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | C |
| HHI | Gerhard Tech | gerhard.tech@hhi.fraunhofer.de | C |
| Orange Labs | J. Jung | joelb.jung@orange.com | C |

# List of input documents

## CE2

A total of 17 input contributions were gathered in this category, among which were 9 proposals. Among the 9 proposals, three of them are CE contributions, and six of them are non-CE contributions. The input contributions can be roughly listed as following table. Please note that the CE contributions are in blue blocks.

| Doc. No. | Contributions | Proponent(s) | Cross-checker(s) |
| --- | --- | --- | --- |
| JCT3V-G0046 | 3D-CE2: results on interview motion prediction by DoNBDV for depth coding | MediaTek | G0093 (Samsung) |
| JCT3V-G0047 | 3D-CE2 related: Simplification on interview motion prediction for depth coding | MediaTek | G0196 (Samsung) |
| JCT3V-G0055 | 3D-CE2 related: A texture-partition-dependent depth partition for 3D-HEVC | MediaTek | G0210 (Orange) |
| JCT3V-G0063 | 3D-CE2: results on additional merging candidates for depth coding | MediaTek | G0195 (Samsung) |
| JCT3V-G0074 | 3D-CE2 related: Simplification of DV Derivation for Depth Coding | Samsung | G0170 (Sharp) |
| JCT3V-G0098 | 3D-CE2 related: Performance evaluation on depth Merge mode candidate | Samsung | G0227 (MediaTek) |
| JCT3V-G0119 | CE2: Sub-PU based MPI | Qualcomm | G0212 (HHI) |
| JCT3V-G0126 | CE2 related: Disparity derivation for depth coding | Qualcomm |  |
| JCT3V-G0127 | CE2 related: Simplification of merge candidate list in depth coding | Qualcomm | G0197 (Samsung) |

# Summary of proposals & results

## CE2 results

The following table describes the best coding gain in each CE contribution.

| Doc. No. | Contributions | Video /total BR | Synthesized /total BR |
| --- | --- | --- | --- |
| Test 1: New merging candidates for depth coding |  |  |  |
| JCT3V-G0063 | 3D-CE2: results on additional merging candidates for depth coding (DDD Candidate only) | 0.01% | -0.20% |
|  | 3D-CE2: results on additional merging candidates for depth coding (Additional texture candidate) | -0.05% | -0.08% |
|  | 3D-CE2: results on additional merging candidates for depth coding (Both two candidates) | -0.03% | -0.27% |
| Test 2: Sub-PU based MPI |  |  |  |
| JCT3V-G0119 | CE2: Sub-PU based MPI | -0.24% | -0.23% |
| Test 3: Interview motion vector prediction for depth coding |  |  |  |
| JCT3V-G0046 | 3D-CE2: results on interview motion prediction by DoNBDV for depth coding | -0.01% | -0.03% |

### Test 1: New merging candidates for depth coding

JCT3V-G0063 is the CE2 results for new merging candidates for depth coding.

- JCT3V-G0063 [MediaTek] 3D-CE2: results on additional merging candidates for depth coding (Cross-checked in JCT3V-G0195 [Samsung])

There are two parts in this contribution:

- A disparity derived depth (DDD) candidate was proposed to derive a depth prediction value converted from the disparity vector of a collocated texture block. The derived depth value is used to represent all prediction samples of the current block when the proposed candidate is selected.

- An additional texture merging candidate was proposed to inherit the motion parameters from the below-right position of the collocated texture PU. It is also proposed to add an additional pruning to further improve the coding efficiency.

#### Coding results of the CE contribution G0063

| G0063 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Part 1 | 0.00% | 0.00% | 0.00% | 0.01% | -0.20% | 99.3% | 97.3% |
| Part 2 | -0.05% | -0.04% | -0.01% | -0.05% | -0.08% | 99.7% | 99.6% |
| Part 1+Part 2 | -0.05% | -0.04% | -0.01% | -0.03% | -0.27% | 98.8% | 97.1% |

### Test 2: Sub-PU based MPI

- JCT3V-G0119 [Qualcomm] CE2: Sub-PU based MPI (Cross-checked in JCT3V-G0212[HHI])

This contribution proposes to extend the sub-PU level inter-view motion prediction method to the Motion Parameter Inheritance (MPI) merge candidate in 3D-HEVC. For each depth PU, 8x8 size sub-PUs can grab different motion vectors from the co-located 8x8 blocks in texture to form a MPI candidate. Experimental results show that the proposed method achieves additional 0.2% BD-rate saving

#### Coding results of the CE contribution G0119

| G0119 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.06% | 0.06% | 0.01% | -0.24% | -0.23% | 100.1% | 103.3% |

### Test 3: Interview motion vector prediction for depth coding

- JCT3V-G0046 [MediaTek] 3D-CE2: results on interview motion prediction by DoNBDV for depth coding (Cross-checked in JCT3V-G0093[Samsung])

In the current 3D-HEVC, the interview motion vector prediction for depth coding uses a disparity vector derived from neighboring reconstructed depth pixels to retrieve the corresponding motion vector in the reference view. The decoding pipeline needs to wait for the reconstruction of the neighboring depth pixels to proceed the derivation of the disparity vectors. This procedure may produce bubble cycles in the decoder pipeline.

It is proposed to use depth oriented neighboring block disparity vector (DoNBDV) in the interview motion vector prediction for depth coding in this contribution. The experimental results reportedly show that no coding loss is observed for coded and synthesized views.

#### Coding results of the CE contribution G0046

| G0046 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.00% | 0.00% | 0.00% | -0.01% | -0.03% | 102.57% | 101.33% |

Three CE-related contributions JCT3V-G0047, JCT3V-G0074, JCT3V-G0126 propose some simplification methods related to the disparity derivation part of interview motion prediction for depth coding..

## CE2 related contributions

The CE2-related contributions are roughly classified into three categories according to their purpose: simplifcation of disparity vector derivation in depth coding, simplification of merge candidate list in depth coding, and coding efficiency. The following table shows the classfication and their coding results..

| Doc. No. | Contributions | Video /total BR | Synthesized /total BR |
| --- | --- | --- | --- |
| Simplification of disparity vector derivation in depth coding |  |  |  |
| JCT3V-G0047 | 3D-CE2 related: Simplification on interview motion prediction for depth coding | -0.02% | -0.06% |
| JCT3V-G0074 | 3D-CE2 related: Simplification of DV Derivation for Depth Coding | 0.00% | 0.04% |
| JCT3V-G0126 | CE2 related: Disparity derivation for depth coding | -0.01% | -0.02% |
| Simplification of merge candidate list in depth coding |  |  |  |
| JCT3V-G0098 | 3D-CE2 related: Performance evaluation on depth Merge mode candidate (remove DV, Shift IVMV and Shift DV candidates) | -0.01% | 0.05% |
| JCT3V-G0127 | CE2 related: Simplification of merge candidate list in depth coding (Remove shifting candidate) | 0.00% | 0.02% |
|  | CE2 related: Simplification of merge candidate list in depth coding (Remove DV and shifting candidates) | -0.01% | 0.05% |
| Coding efficiency |  |  |  |
| JCT3V-G0055 | 3D-CE2 related: A texture-partition-dependent depth partition for 3D-HEVC | 0.04% | -0.16% |

### Simplification of disparity vector derivation in depth coding

- Comparison of the disparity vector derivation of the CE and CE-related contributions.

| Doc. No. | DV derivation |
| --- | --- |
| JCT3V-G0046 | Use NBDV to do DoNBDV |
| JCT3V-G0047 | Use depth value 128 converted DV to do DoNBDV |
| JCT3V-G0074 | Use depth value 128 converted DV as final DV |
| JCT3V-G0126 | Use collocated depth block(zero DV) to do DoNBDV |

- JCT3V-G0047 [MediaTek] 3D-CE2 related: Simplification on interview motion prediction for depth coding (Cross-checked in JCT3V-G0196 [Samsung])

In the current 3D-HEVC, the interview motion vector prediction for depth coding uses a disparity vector derived from neighboring reconstructed depth pixels to retrieve the corresponding motion vector in the reference view. The decoding pipeline needs to wait for the reconstruction of the neighboring depth pixels to proceed the derivation of the disparity vectors. This procedure may produce bubble cycles in the decoder pipeline.

It is proposed to use depth oriented neighboring block disparity vector (DoNBDV) with an initial disparity vector converted from a constant depth value 128 in the interview motion vector prediction for depth coding in this contribution. The experimental results reportedly show 0.1% coding gain for synthesized views.

- JCT3V-G0074 [Samsung] 3D-CE2 related: Simplification of DV Derivation for Depth Coding (Cross-checked by JCT3V-G0170 [Sharp])

In the current 3D-HEVC, the disparity vector for depth coding is derived using neighboring reconstructed pixel values. However, the current method has a potential problem with pipeline processing because of dependency with the neighboring blocks.

Two aspects are proposed:

- Derive the disparity vector for depth coding only using the depth value 128, which means the middle value of the depth value range. With the proposed method, the dependency of the neighboring blocks can be completely removed. The proposed method reportedly provides no coding loss.

- Additionally, this contribution presents the results of proposed method with the removal of DV candidates from the depth merge list. This also reportedly provides no coding loss, this is marked as Removal1 in the following coding results. And when shifted inter-view candidate is additionally removed, it reportedly provides 0.06% loss for synthesized views as marked by Removal2 in the following coding results.

- JCT3V-G0126 [Qualcomm] CE2 related: Disparity derivation for depth coding (Cross-checked by JCT3V-G0xxx)

In 3D-HEVC, inter-view motion prediction for depth blocks is enabled. To support that, currently a disparity vector is generated by accessing neighboring reconstructed depth pixels of the current coding unit. Since the current method was considered to potentially introduce some delay, this proposal provides a simple method to fix this problem.

A co-located depth block in the reference depth view is accessed and the maximum value of the four corner pixels is used to generate the disparity vector, therefore there is no need to access the spatial neighboring pixels of the current block. Simulation results show that the proposed method doesn’t provide any loss (-0.01% coding gain instead).

#### Coding results of the CE-related contribution relating to simplification on interview motion prediction for depth coding

|  | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G0047(based on G0046) | 0.00% | 0.00% | 0.00% | -0.02% | -0.06% | 101.7% | 98.9% |
| G0074 | 0.00% | 0.00% | 0.00% | 0.00% | 0.04% | 99.8% | 100.5% |
| G0126 | 0.00% | 0.00% | 0.00% | -0.01% | -0.01% | 99.3% | 100.6% |
| G0126(based on G0046) | 0.00% | 0.00% | 0.00% | -0.01% | -0.02% | 97.1% | 100.1% |

#### Additional coding results of the CE-related contribution G0074

|  | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G0074 | 0.00% | 0.00% | 0.00% | 0.00% | 0.04% | 99.8% | 100.5% |
| G0074(Removal1) | 0.00% | 0.00% | 0.00% | -0.01% | 0.04% | 100.1% | 101.5% |
| G0074(Removal2) | 0.00% | 0.00% | 0.00% | 0.00% | 0.06% | 99.9% | 102.8% |

### Simplification of merge candidate list in depth coding

- JCT3V-G0098 [Samsung] 3D-CE2 related: Performance evaluation on depth Merge mode candidate (Cross-checked in JCT3V-G0227 [MediaTek])

This contribution evaluates coding performance of depth Merge candidates. Currently, five 3D candidates, which consist of MPI, IVMV, DV, Shift IVMV and Shift DV, are added in the Merge lists in addition to HEVC candidates. However, it is observed that DV, Shift IVMV, and Shift DV have no coding gain at all. Also, IVMV has very small coding performance of less than 0.1%. Hence, this contribution proposes to remove these three candidates (DV, Shift IVMV, and Shift DV) in the depth Merge list construction and suggests evaluating the performance of the IVMV candidate in CE.

#### Coding results of the CE-related contribution G0098

|  | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G0098 | 0.00% | 0.00% | 0.00% | -0.01% | 0.05% | 99.5% | 99.7% |

- JCT3V-G0127 [Qualcomm] CE2 related: Simplification of merge candidate list in depth coding (Cross-checked in JCT3V-G0197 [Samsung])

Current 3D-HEVC supports inter-view depth motion prediction with merge candidates generated in the same way as in inter-view texture motion prediction plus the MPI (motion parameter inheritance) candidate. However, the characteristics of the depth are quite different from texture. Therefore the complexity and efficiency trade off achieved for texture coding in 3D-HEVC is not optimal for depth coding. Given that there are too many merge candidates for depth blocks (even more than texture), it is necessary to remove the candidates which provide very minor coding benefits.

In this contribution, two candidates are verified to be inefficient: the first one is the shifting candidate (from shifting a spatial disparity motion vector and accessing an additional bottom-right block of the reference block), and the second one is the disparity vector candidate. It is reported that the coding loss for removing the first candidate is only 0.02% and the coding loss for removing both candidates is only 0.05%.

#### Coding results of the CE-related contribution G0127

|  | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Removing shifting cand. | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 102.5% | 100.6% |
| Removing DV and shifting cands. | 0.00% | 0.00% | 0.00% | -0.01% | 0.05% | 103.0% | 102.1% |

### Coding efficiency

- JCT3V-G0055 [MediaTek] 3D-CE2 related: A texture-partition-dependent depth partition for 3D-HEVC (Cross-checked in JCT3V-G0210 [Orange])

In the current 3D-HEVC, the depth quad-tree predictive coding restricts the possible PU partitions of a depth CU according to the PU partition of its collocated CU.

In this contribution, the restrictions are modified as follows. Vertical/horizontal partitions are allowed for a depth CU only when the collocated texture CU is coded with vertical/horizontal or NxN partition. The experimental results reportedly show 0.2% BD-rate reduction on synthesis views without encoding and decoding time increase.

#### Coding results of the CE-related contribution G0055

| G0055 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 98.0% | 98.0% |

# Closing Remarks

It is recommended the CE be continued and investigate proposed designs in further details.