| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0024 |
| --- | --- |

| Title: | CE4 Summary Report: Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | Li Zhang Jian-Liang Lin | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com +886-3-5670766 ext. 25555 jl.lin@mediatek.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of the Core Experiment 4 (CE4) on Residual Prediction. During the 6th JCT-3V meeting, five contributions have been proposed to further improve the residual prediction in 3D-HEVC in terms of good trade-off between coding efficiency and complexity and they were considered for further study in a Core Experiment. The goal of this CE is to evaluate them in the context of the 3D-HTM 9.0r1.

In summary, 9 companies and universities were involved in the CE with 4 CE proposals and 3 CE-related proposals.

# Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Qualcomm | Li Zhang Ying Chen | lizhang@qti.qualcomm.com cheny@qti.qualcomm.com | P/C |
| MediaTek | Jian-Liang Lin Jicheng An Kai Zhang Yu-Lin Chang Yu-Wen Huang | jl.lin@mediatek.com jicheng.an@mediatek.com Kai.zhang@mediatek.com yulin.chang@mediatek.com yuwen.huang@mediatek.com | P/C |
| NTT | Shiori Sugimoto Shinya Shimizu | sugimoto.shiori@lab.ntt.co.jp shimizu.shinya@lab.ntt.co.jp | P |
| Samsung | M. W. Park Jin Young Lee | m.w.park@samsung.com jinyoung79.lee@samsung.com | P/C |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp | P/C |
| LG | Jin Heo | jin78.heo@lge.com | P/C |
| Zhejiang University | Yichen Zhang Peng Lu | felixzyc@gmail.com yiqiu8932@gmail.com | C |
| Industrial Technology Research Institute | Jih-Sheng Tu Ching-Chieh Lin | SuriseTu@itri.org.tw JackLin@itri.org.tw | P |
| National Cheng Kung University | Bo-Syun Li, Chun-Fu Chen | n26014972@mail.ncku.edu.tw n28991146@mail.ncku.edu.tw | C |

# Tools under investigation

## Subset A (complexity reduction)

### Simplification of inter-view ARP [JCT3V-G0158]

In this proposal, it is claimed to simplify the inter-view ARP by utilizing the following aspects:

- The temporal motion information used in inter-view ARP is derived using the same way as shifted temporal inter-view merging candidate derivation process.

- The residual and residual predictor are calculated as temporal residuals from current view, and the reference view, respectively.

- The disparity vector from DoNBDV process is updated to the coded disparity motion vector associated with current block.

Two more aspects which are considered as outside of the CE scope are described in 4.1.2.

The crosscheck report JCT3V-G0223 is missing.

## Subset B (coding performance)

### Results on improved advanced residual prediction [JCT3V-G0064]

In this contribution, two aspects are proposed to improve the coding efficiency of the advanced residual prediction. The first part was in the scope of CE:

Motion vector candidate lists for ARP (MV-Cand ARP): three motion vector candidates (disparity motion vectors for inter-view ARP and temporal motion vectors for temporal ARP) are derived from temporal/spatial neighbouring blocks. Spatial neighbouring blocks are the top and left blocks of current block while temporal neighbouring blocks are those used in NBDV or the TMVP process. For temporal ARP, the DoNBDV results of current block and spatial neighbouring blocks may be included. The best motion vector candidate index is selected according to RDO criterion and signaled in the bitstream.

This part was crosschecked by Sharp and Qualcomm, and detail information can be referred to JCT3V-G0167 and JCT3V-G0222, respectively.

The second part is considered as outside of the CE scope which will be described in section 4.2.1.

### Further improvements on advanced residual prediction [JCT3V-G0121]

The techniques to improve the coding efficiency of ARP in this proposal include two aspects in the CE:

- Block-level ARP: block-level ARP is proposed by splitting one prediction unit into several blocks to perform ARP separately. Furthermore, the residual of chroma components is not coded if the current PU is coded with ARP;

- Disable IC when ARP is enabled: the ic_flag is not signalled when the ARP is enabled (i.e., ARP weighting factor is unequal to 0); (same as #1 in JCT3V-G0072)

The second part was crosschecked by Samsung, and detail information can be referred to JCT3V-G0220. The whole proposal was crosschecked by MediaTek and LG, and detail information can be referred to JCT3V-G0226 and JCT3V-F0247.

## Subset C (Interaction with other tools)

### Results on IC and ARP Flags Signaling [JCT3V-G0072]

In this contribution, three aspects are proposed:

- Disable the IC when ARP is enabled, (i.e., iv_res_pred_weight_idx is greater than 0. (same as #2 in JCT3V-G0121)

- Disable ARP when illumination compensation (IC) is enabled, i.e., ic_flag is equal to 1;

This proposal was crosschecked by LG and detail information can be referred to JCT3V-G0075.

# Experiments and results for CE proposals

Note for JCT3V-G0158, both the anchor and proposed method are evaluated on top of HTMv9.0r1 with a ‘bug-fix’.

Table 1: Summary of CE results compared to 3D-HTM

| Topics | Proposals |  |  |  | Simulation Results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Video 1 |  | Video 2 |  | Video PSNR / Video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc. time | Dec. time |
| Simplified Inter-view ARP | JCT3V-G0158* | 0.3% |  | 0.3% |  | 0.1% | 0.1% | 0.1% | 101% | 102% |
| MV-Cand ARP with 3 weighting factors | JCT3V-G0064 | -0.4% |  | -0.4% |  | -0.2% | -0.2% | -0.1% | 110% | 99% |
| MV-Cand ARP with 2 weighting factors | JCT3V-G0064 | -0.5% |  | -0.5% |  | -0.2% | -0.2% | -0.1% | 103% | 100% |
| Block-level ARP | JCT3V-G0121 | -0.5% |  | -0.7% |  | -0.2% | -0.2% | -0.2% | 104% | 102% |
| disable IC when ARP is enabled | JCT3V-G0072 | -0.1% |  | -0.1% |  | -0.1% | -0.1% | 0.0% | 98% | 99% |
|  | JCT3V-G0121 | -0.1% |  | -0.1% |  | -0.1% | -0.1% | 0.0% | 98% | 100% |
| disable ARP when IC is enabled | JCT3V-G0072 | -0.1% |  | -0.1% |  | 0.0% | 0.0% | 0.0% | 98% | 99% |

# Related proposals

## Subset A (complexity reduction)

### Simplification of inter-view ARP [JCT3V-G0158]

In this proposal, two more aspects are further proposed in addition to the technique described in 2.1.1:

- Removal of the constraint of DV-MCP: It is proposed that in NBDV derivation process, the DV-MCP information of spatial neighbouring blocks are always checked even they are not coded as skip mode.

- Combination of inter-view and temporal ARP: The DVs from NBDV and DoNBDV processes are both updated to the disparity motion vector associated with current block, if available. The updated disparity vectors are used in ARP process.

### ARP simplification [JCT3V-G0033]

This contribution proposes to disable ARP for 4x4 chroma blocks. Simulation results show that the proposed method introduces 0.1% coding loss for non-base texture views and minor coding loss for coded texture views (0.03%).

This proposal is crosschecked by Zhejiang University and detail information can be referred to JCT3V-G0041.

## Subset B (coding performance)

### Results on improved advanced residual prediction [JCT3V-G0064]

In this proposal, it is further proposed to add two more candidates for inter-view ARP:

Aligned Temporal DV (ATDV): on top of the techniques described in section 2.2.1, a new temporal motion vector candidate, i.e., ATDV, is obtained from the aligned block, which is located by a scaled MV to the collocated picture. Two collocated pictures used in the NBDV derivation may be used to derive the DMV candidates. ATDV is checked before DV candidates from neighbouring blocks when it is used. Note that this item was not concluded as part of CE.

It was crosschecked by Sharp, and detail information can be referred to JCT3V-G0167.

### An alternative disparity vector derivation method for ARP [JCT3V-G0095]

This contribution proposes a disparity motion vector candidate list for temporal ARP. The list includes two candidates, one is the disparity vector from NBDV process and the other is the disparity vector from DoNBDV process. The best candidate is signalled in the bitstream.

This proposal is crosschecked by NCKU and detail information can be referred to JCT3V-G0187.

### Residual Prediction for View Synthesis Prediction [JCT3V-G0076]

It is proposed to change the initialization table for the contexts used by coding weighting factors.

This proposal is crosschecked by NTT and detail information can be referred to JCT3V-G0154.

# Experiments and results for CE related proposals

Note for JCT3V-G0158, both the anchor and proposed method are evaluated on top of HTMv9.0r1 with a ‘bug-fix’.

Table 2: Summary of CE related results compared to 3D-HTM

| Topics | Proposals |  |  |  | Simulation Results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Video 1 |  | Video 2 |  | Video PSNR / Video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc. time | Dec. time |
| Simplified Inter-view ARP + DV-MCP | JCT3V-G0158* | 0.3% |  | 0.2% |  | 0.1% | 0.1% | 0.1% | 101% | 101% |
| Simplified Inter-view and temporal ARP + DV-MCP | JCT3V-G0158 | 0.3% |  | 0.2% |  | 0.1% | 0.1% | 0.1% | 101% | 102% |
| ARP Removal for 4x4 chroma blocks | JCT3V-G0033 | 0.1% |  | 0.1% |  | 0.0% | 0.0% | 0.0% | 100% | 100% |
| MV-Cand ARP + ATDV with 2 weighting factors | JCT3V-G0064 | -0.7% |  | -0.7% |  | -0.2% | -0.2% | -0.2% | 103% | 98% |
| NBDV and DoNBDV adaptation | JCT3V-G0095 | -0.3% |  | -0.3% |  | -0.1% | -0.1% | -0.1% | 100% | 100% |
| Extension of ARP to BVSP | JCT3V-G0076 | -0.3% |  | -0.2% |  | -0.1% | -0.1% | -0.1% | 100% | 99% |