| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, USA, 9 – 17 Jan. 2014 | Document: JCTVC-P0041v3 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, USA, 11 – 17 Jan. 2014 | Document: JCT3V-G0031v3 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On picture order count |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Adarsh K. Ramasubramonian Ye-Kui Wang Ying Chen Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA Ming Li Ping Wu ZTE Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 | Tel: Email: | 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-658-5801 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 86-25-88014644 li.ming42@zte.com.cn ping.wu@zte.com.cn |
| Source: | Qualcomm Incorporated, ZTE |  |  |

_____________________________

# Abstract

This contribution presents an improved design for the signalling and derivation of picture order count in SHVC and MV-HEVC. It is proposed that POC reset be indicated by a two-bit indication, to fully utilize the fact that there would never be POC LSB reset only. Additional a POC LSB is signalled in order to provide better error resilience to the POC derivation process and for support of missing-collocated-picture scenarios. Finally, the MSB value of the picture order count is also signalled for CRA pictures.

The main changes compared to the solution in JCTVC-O0213v4 are as follows:

- In output order conformant decoders, it is proposed to output all earlier pictures in the DPB upon receiving a POC reset picture.

  - It is asserted that by doing this, the problem raised at the 2nd POC conference call, for the solution in JCTVC-O0213v3, about possibility of having erroneous order of output of pictures is addressed.

- Revert the timing of decrement of POC of earlier pictures in the DPB to that of described in v3 of this proposal, that is, POC decrement of earlier pictures in the DPB is done in a layer-specific manner.

  - It is asserted that the combination of outputting all earlier picture in the DPB upon receiving a poc reset picture and decrementing POC of earlier pictures in the DPB only of pictures in the same layer as the current layer addresses the problem raised at the 3rd POC conference call, for the solution in JCTVC-O0213v4, about incorrect POC value decrement in case of down-switching and up-switching and with picture loss.

- Propose to signal POC MSB information in slice header extension when current picture is a CRA or BLA picture.

  - The signalled POC MSB information is used for derivation of POC MSB when current picture is a CRA picture with NoRaslOutputFlag equal to 0 for any conditions and derivation of previous POC MSB for pictures when POC reset is applied at CRA picture.

  - This provides two benefits:

    - The first is to allow correct derivation of POC in some use-cases such as switching down and up, and pseudo-single-loop-decoding where in the base layer only CRA pictures are used for inter-layer prediction and present.

    - The second is to allow correct derivation of POC for trick-mode with CRA pictures, including changing from CRA-based trick-mode to normal playback mode or reduced speed-up ratio e.g adding TemporalId-zero pictures.

The text changes are included in the attachment of this document, in relative to JCT3V-F1004v6.

In v2 JCTVC-P0041/JCT3V-G0031, the document template/header was corrected, without change marks. The spec text changes and other parts remain unchanged as in v1.

In v3 JCTVC-P0041/JCT3V-G0031, an example is added in section 3, with change marks. The spec text changes and other parts remain unchanged as in v2.

# Introduction

The SHVC and MV-HEVC specifications allow access units that contain both IRAP pictures and non-IRAP pictures. In the previous meeting, it was proposed that the POC reset information be signalled in the slice header in the form of poc_reset_flag, and this flag would be used to reset the POC of the current picture to 0, and also decrement the POC of the pictures in the DPB with the same value of nuh_layer_id as the current picture by a value that is equal to the POC of the current picture before reset. The flag is signalled in the reserved bits of the slice segment header, and signalled not just for the enhancement layer, but also for the base layer.

# Problem Statement

The presence of the POC reset information in the base layer only indicates that the POC derivation process would be different in case of an operating point containing only the base layer, and other operating points. This POC signalling has the following shortcomings:

- When an access unit auA contains an enhancement layer picture that is an IRAP picture with NoRaslOutputFlag equal to 1, and a base layer picture that is non-IRAP (assume two layer bitstream for simplicity), and either the value of the signalled slice_pic_order_cnt_lsb of the EL picture is non-zero, or the derived POC MSB value of the base layer when decoding starts from any preceding IRAP access unit is non-zero, then in order that POC of all the picture in an access unit be aligned, the POC has to be reset at the base layer, and additionally in the enhancement layer when the signalled slice_pic_order_cnt_lsb at the EL picture is non-zero. Under this circumstance, the POC LSB of base layer pictures following auA in decoding order will be assuming the reset value. Consider the example shown in Figure 1, with MaxPicOrderCntLsb equal to 128 for both the base and the enhancement layers. The bitstream has two layers, and the EL picture in AU 241 is an IRAP picture with NoRaslOutputFlag equal to 1. The POC, therefore, is reset for the base layer pictures. In a multi-layer codec, the signalling results in aligned POC values for pictures in an access unit. However, when the bitstream would be decoded by a version 1 HEVC decoder, the POC derived for the BL pictures would be incorrect in their relative ordering with other pictures.

Figure 1: POC reset when an AU contains a non-IRAP BL picture and an IRAP EL picture with NoRaslOutputFlag equal to 1. The numbers in green indicate the POC LSB and MSB after POC is reset at AU 241. The numbers in red indicate that the relative POC numbers are incorrect.

- When the picture containing POC reset information is lost, the derivation of the POC for succeeding pictures in the same layer may be incorrect.

# Proposal

A summary of the various aspects of the proposal are presented below:

- Instead of using two bits, one for POC MSB reset and one for POC LSB reset, a two-bit indication is used, to fully utilize the fact that there would never be only POC LSB reset.

- The two bits, and possibly additional information for support of missing-collocated-picture scenarios as well as for resolving the error resilience issue, are signalled in the slice header extension instead of in the up-to-seven reserved slice header bits.

- A POC resetting period is specified based on the POC resetting period ID. Each non-IRAP picture that belongs to an access unit that contains at least one IRAP picture must be the start of a POC resetting period in the layer containing the non-IRAP picture. In that access unit, each picture would be the start of a POC resetting period in the layer containing the picture. POC resetting and update of POC values of same-layer pictures in the DPB are applied only for the first picture within each POC resetting period.

- For derivation of the delta POC value used for updating the POC values of the same-layer pictures in the DPB as well as for derivation of the POC MSB of the POC value of the current picture, a POC LSB value is conditionally signalled in the slice segment header.

- A loss of a picture at the start of a POC resetting period can be overcome by having one or more same-layer pictures that follow that picture in decoding order and that are assigned the value 3 of the two-bit indication. The same process for support of missing-collocated-picture scenarios apply for resolving the error resilience issue.

- Signal POC MSB value for CRA pictures in order to

  - Derive correct rest the POC values in some use cases

  - To enable trick mode play with CRA pictures

- In output-order conformant decoders, output all pictures in the DPB before a POC resetting period at the beginning of the first access unit of the POC resetting period.

## Some examples

During the 1st, 2nd, and 3rd POC conference calls, several use cases were considered important to be supported. Some of the use-cases and how the proposed solution solves these issues are described in this section.

### Example 1

Figure 2: Example 1

Figure 2 illustrates a bitstream that is received by a decoder. The pictures that are marked with dotted lines are pictures that are not received by the decoder – possibly due to a switch-up and switch-down operation performed on the bitstream by a middle box. The solution described JCTVC-O0213v3 would not work with the above example because the POC reset in the AU that contains the CRA picture would only reset the pictures of the base layer. Then the picture order count of the preceding pictures would not be aligned within an AU, and this could result in erroneous order of output of pictures.

The proposed solution outputs all the pictures in the DPB that are marked as "needed for output" at the beginning of the first access unit of a POC resetting period. As a result even if POC reset AU containing the CRA picture only resets the POC of the BL pictures, it would not cause any erroneous order of output of pictures due to the POC mismatch.

### Example 2

Figure 3: Example 2

Figure 3 illustrates a bitstream that is received by a decoder. The pictures that are marked with dotted lines (without red cross) are pictures that are not received by the decoder – possibly due to a switch-up and switch-down operation performed on the bitstream by a middle box. When the base layer picture at the switch-up point is lost, the solution described JCTVC-O0213v4 would not work because the value needed to decrement the POC value of the base layer pictures during reset would be unknown to the IDR picture. The result would once again be erroneous order of output of pictures.

The proposed solution outputs all the pictures in the DPB that are marked as "needed for output" at the beginning of the first access unit of a POC resetting period. In the above example, this would be initiated at the IDR picture. Since the proposed solution resets the POC only within each layer, the POC would be reset for the base layer only at the subsequent AU, where the picture would have poc_reset_idc equal to 3.

## Signalling MSB values for CRA pictures

Figures 4, 5, and 6 describe why it is beneficial to signal the MSB of the POC for the CRA pictures.

Figure 4: Example 3, where POC MSB is signalled for CRA pictures

Figure 5: In the example in the figure, the POC MSB is needed for CRA pictures in order to decrement the POC of reference pictures in EL that may occur before the switch-down point.

Figure 6: In the example in the figure, POC MSB is not needed for IDR pictures

Due to coding restrictions on IDR pictures, IDR pictures and their associated pictures cannot use pictures preceding the IDR picture, in decoding order, for reference. And since the pictures preceding that POC reset AU are output at the beginning of decoding of that AU and those that belong to the same layer would be removed from the DPB soon, there is no need to reset the POC of those preceding pictures, and hence no need to signal the MSB for the IDR pictures.

Figure 7: Example of bitstream with only CRA pictures, where the signaling of MSB for the CRA pictures would allow the correct decoding of POC values for the CRA pictures.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).