| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0043 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0032 |
| --- | --- |

| Title: | Version 1/MV-HEVC/SHVC HLS: Access unit boundary detection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution discusses problems related to access unit boundary detection and contains the following three proposals (one with two alternatives):

- It is proposed to clarify that the decoders shall use access unit delimiter NAL units with any value of nuh_layer_id in the determination of the start of a new access unit.

- Regarding the presence of the access unit delimiter NAL unit when there is no base layer picture present, either of the following alternatives is proposed:

- It is proposed to require the presence of the access unit delimiter NAL unit when there is no base layer picture present in the access unit.

- It is proposed to allow indication of access unit boundaries by external means. When external means are not in use, it is proposed to require the presence of the access unit delimiter NAL unit when there is no base layer picture present in the access unit.

- It is proposed to require the presence of first_slice_segment_in_pic_flag as the first syntax element in all VCL NAL units with nuh_layer_id equal to 0.

# Problems

It is asserted that the access unit (AU) boundary detection has the following problems currently:

- The current AU boundary specification specifies one coded picture to be an access unit.

It is specified that the first VCL NAL unit of a coded picture after the last VCL NAL unit of the previous coded picture starts a new access unit. The intent in SHVC/MV-HEVC is to allow several coded pictures, each having different values of nuh_layer_id, in the same access unit.

- Version 1 decoders must be able to detect boundaries of AUs that do not contain an HEVC base layer picture.

It is allowed to have access units where the base layer picture is not present for example to enable a base layer @ 30 Hz and a spatial or quality enhancement layer @ 60 Hz.

If there is no NAL unit that starts a new access unit (e.g. an access unit delimiter) present and also if there is no base layer picture present in the access unit (AU), it is asserted that HEVC v1 decoders may consider the following coded enhancement layer pictures as a part of the previous access unit, while SHVC/MV-HEVC decoders are intended to consider them as part of a new access unit. Consequently, it is asserted that the HRD parameters for AU-based CPB operation may become ambiguous and may be interpreted differently by HEVC v1 decoders and SHVC/MV-HEVC decoders.

A similar issue occurs in hybrid codec scalability, when the AVC base layer pictures are either not present in the HEVC bitstream or are encapsulated in NAL units that are not interpreted to start a new access unit.

- It should be clarified if version 1 decoders shall consider NAL units with nuh_layer_id greater than 0 in the AU boundary determination.

# Related subclauses and their proposed interpretation

The first NAL unit of an access unit is specified in subclause 7.4.2.4.4 as follows:

The first of any of the following NAL units after the last VCL NAL unit of a coded picture specifies the start of a new access unit:

–	access unit delimiter NAL unit (when present),

–	VPS NAL unit (when present),

–	SPS NAL unit (when present),

–	PPS NAL unit (when present),

–	Prefix SEI NAL unit (when present),

–	NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44 (when present),

–	NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC55 (when present),

–	first VCL NAL unit of a coded picture (always present).

This specification has an obvious problem if used as such for MV-HEVC/SHVC, because the intent is to allow several coded pictures, each having different values of nuh_layer_id, in the same access unit.

The following is specified in subclause 7.4.2.2:

For purposes other than determining the amount of data in the decoding units of the bitstream (as specified in Annex C), decoders shall ignore (i.e., remove from the bitstream and discard) all NAL units with values of nuh_layer_id not equal to 0.

This sentence would suggest that version 1 decoders shall consider all NAL units regardless of their value of nuh_layer_id in determining the amount of data in the decoding units. An access unit could be regarded as a form of a decoding unit. Moreover, determining the data belonging to an access unit requires the decoder to determine the first NAL unit from the list above. Hence, version 1 decoders should consider NAL units with nuh_layer_id greater than 0 as candidates to be the first NAL unit of an access unit as per the list above.

Subclause F.7.4.2.2 specifies the following regarding access unit delimiters:

When nal_unit_type is equal to AUD_NUT, the value of nuh_layer_id shall be equal to the minimum of the nuh_layer_id values of all VCL NAL units in the access unit.

# Design principles for AU boundary detection

It is suggested that the following design principles are used for fixing the asserted problems in AU boundary detection:

- AU boundary detection should be specified based on NAL unit header syntax elements (nal_unit_type, nuh_layer_id) and first_slice_segment_in_pic_flag.

It is asserted that AU boundary detection is needed in some operations performed by middle-boxes, such as selective forwarding of NAL units. In such operations it is often desirable to omit the forwarding of an entire access unit. For middle-boxes it would be desirable to enable AU boundary detection without performing any actual decoding processes.

- It is undesirable to specify AU boundary detection such that it would require decoding of slice segment header beyond first_slice_segment_in_pic_flag.

Motivation:

- Decoding of a slice segment header requires the have the parameter sets correctly activated. In other words, a middle-box should perform the parameter set activation process for correct parsing of the slice segment header. This brings undesirable complexity for middle-boxes.

- The AU boundary detection should be generic enough to support reserved VCL NAL unit types to be taken into use in future extensions. Consequently, the first VCL NAL unit in an AU may be of a previously unused VCL NAL unit type. It would be undesirable to constrain the slice segment header syntax for the new VCL NAL unit types extensively, while requiring the presence of first_slice_segment_in_pic_flag as the first syntax element is considered acceptable in our opinion.

# Overview of the proposal

The contribution includes three proposals (the second one of them with two alternatives):

- It is proposed to clarify that the decoders shall use access unit delimiter NAL units with any value of nuh_layer_id in the determination of the start of a new access unit.

- The presence of the access unit delimiter NAL unit when there is no base layer picture present in the access unit is proposed to be required with either of the following two alternatives:

- It is proposed to require the presence of the access unit delimiter NAL unit when there is no base layer picture present in the access unit.

OR

- It is proposed to allow indication of access unit boundaries by external means. When external means are not in use, it is proposed to require the presence of the access unit delimiter NAL unit when there is no base layer picture present in the access unit.

It is believed that option b could be realized in most systems. For example, in the file format (ISO/IEC 14496-15) the sample size indicated in the file format structures is used to determine access unit boundaries and in many other systems presentation time stamps can be used for access unit boundary determination. The asserted downside of option b is that whenever a bitstream relying on access unit boundary detection performed by external means is transferred to a domain where the external means are not available and access units without a base-layer picture exist in the bitstream, transcoding to insert access unit delimiters has to be done.

- To allow future flexibility to take reserved VCL NAL unit types into use in the base layer and to keep on using the same access unit boundary detection, it is proposed to require the presence of first_slice_segment_in_pic_flag as the first syntax element in all VCL NAL units with nuh_layer_id equal to 0.

It is asserted that the proposal enables access unit boundary detection by:

- Access unit delimiter NAL unit (with any value of nuh_layer_id), or

- The first VCL NAL unit of a coded picture with nuh_layer_id equal to 0 is a VCL NAL unit with nuh_layer_id equal to 0 and first_slice_segment_in_pic_flag equal to 1, which may be immediately preceded, in decoding order, by an access unit delimiter NAL unit, VPS NAL unit(s), SPS NAL unit(s), PPS NAL unit(s), prefix SEI NAL unit(s), NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44, or NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC55, the first one of which is the first NAL unit of an access unit.

# Proposed specification text

Alternative a is presented with red and the additional parts of alternative b are presented with purple.

- 7.4.2.4.4	Order of NAL units and coded pictures and their association to access units

This subclause specifies the order of NAL units and coded pictures and their association to access unit for CVSs that conform to one or more of the profiles specified in Annex Error: Reference source not found that are decoded using the decoding process specified in clauses Error: Reference source not found through Error: Reference source not found.

An access unit consists of one or more coded pictures with different values of nuh_layer_id and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in subclause Error: Reference source not found.

The first access unit in the bitstream starts with the first NAL unit of the bitstream.

If the start of a new access unit is specified by external means not specified in this Specification, the first NAL unit of a new access unit following NAL units after the last VCL NAL unit of a coded picture shall be one of the NAL units in the following list.

Otherwise, the first of any of the following NAL units after the last VCL NAL unit of a coded picture specifies the start of a new access unit:

–	access unit delimiter NAL unit (when present),

–	VPS NAL unit (when present),

–	SPS NAL unit (when present),

–	PPS NAL unit (when present),

–	Prefix SEI NAL unit (when present),

–	NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44 (when present),

–	NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC55 (when present),

–	first VCL NAL unit of a coded picture with nuh_layer_id equal to 0 (always present when no access unit delimiter NAL unit is present).

NOTE – The first VCL NAL unit of a coded picture with nuh_layer_id equal to 0 is a VCL NAL unit with nuh_layer_id equal to 0 and first_slice_segment_in_pic_flag equal to 1.

When an access unit contains no VCL NAL unit with nuh_layer_id equal to 0 and external means to specify a new access unit are not in use, an access unit delimiter NAL unit shall be present in the access unit.

Decoders shall use access unit delimiter NAL units with any value of nuh_layer_id in the determination of the start of a new access unit.

# Patent rights declaration(s)

The author is not aware of any current or pending patent rights relating to the technology described in this contribution.