| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0041 |
| --- | --- |

| Title: | CE4 related: Crosscheck of Sharp's proposal on ARP simplification (JCT3V-G0033) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Lu and Lu Yu Zhejiang University Zheda Road 38 Hangzhou, Zhejiang, China | Tel: Email: | +86 15968882677 yiqiu8932@gmail.com |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This document reports the cross check results on ‘JCT3V-G0033 CE4-related: ARP simplification’ by Sharp. Performance evaluation results reported by the proponent were verified.

# Test results

Sharp’s proposed modifications to the software were implemented into the 3DV-HTM reference software HTM 9.0r1 and evaluated according to the common test conditions. The results of the crosscheck verified the test results reported by the proponent. The test results can be found in Table 1. BD-Rates comparing the proposed method and the anchor encodings are also attached to this document.

Table 1.  Performance comparisons between HTM9.0r1 CTC anchor and JCT3V-G0033

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.41% | 0.43% | 0.13% | 0.12% | 0.05% | 100.4% | 87.7% |
| Kendo | 0.00% | 0.17% | 0.12% | 0.03% | 0.02% | 0.02% | 98.1% | 103.1% |
| Newspaper_CC | 0.00% | 0.17% | 0.13% | 0.05% | 0.03% | 0.04% | 97.8% | 98.8% |
| GT_Fly | 0.00% | -0.03% | 0.06% | 0.00% | 0.00% | -0.03% | 98.5% | 90.7% |
| Poznan_Hall2 | 0.00% | -0.09% | 0.14% | 0.02% | 0.01% | 0.07% | 98.8% | 97.2% |
| Poznan_Street | 0.00% | -0.03% | -0.02% | -0.01% | -0.01% | -0.01% | 100.0% | 101.4% |
| Undo_Dancer | 0.00% | -0.06% | -0.06% | -0.01% | -0.01% | -0.03% | 101.5% | 95.8% |
| Shark | 0.00% | 0.11% | 0.09% | 0.02% | 0.00% | 0.02% | 98.5% | 99.6% |
| 1024x768 | 0.00% | 0.25% | 0.23% | 0.07% | 0.06% | 0.04% | 98.7% | 96.5% |
| 1920x1088 | 0.00% | -0.02% | 0.04% | 0.00% | 0.00% | 0.01% | 99.4% | 97.0% |
| average | 0.00% | 0.08% | 0.11% | 0.03% | 0.02% | 0.02% | 99.2% | 96.8% |