| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, USA, 9 – 17 Jan. 2014 | Document: JCTVC-P0056 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, USA, 11 – 17 Jan. 2014 | Document: JCT3V-G0042 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Layer-tree POC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is asserted that the contribution presents a multi-layer POC derivation design that enables non-aligned IRAP pictures across layers, is error-resilient, does not change the POC values of the base layer and keeps the POC values static during their persistence. The contribution is a follow-up of contribution JCTVC-O0275v7/JCT3V-F0092v7.

When compared to JCTVC-O0275v7/JCT3V-F0092v7, this contribution includes the following changes:

- Section 4 of the contribution responds to comments that were included in JCTVC-P0041v5/JCT3V-G0031v5.

- The responses to the comments resulted into a few minor changes of the specification text, which are also described in Section 4 and included in the accompanying specification text document with user identity "(v8)".

- The comments concerning JCTVC-P0041v5/JCT3V-G0031v5 were updated (in Section 1).

The proposal includes the following ingredients:

- A layer tree is a set of layers that have inter-layer dependencies with each other.

- Slice header extensions optionally contain signalling for a layer-tree POC value, which is intended to be used in the IRAP pictures of the independent layers and may be used, for additional error resilience, in other pictures too.

- A layer-tree POC value, LayerTreePicOrderCntVal is derived for each picture based on the mentioned slice header signalling, if present, and based on PicOrderCntVal differences, otherwise.

- Both the POC value and the layer-tree POC value are derived relative to the POC value and the layer-tree POC, respectively, of the previous TID0 picture on the current layer or any direct reference layer. The layer-tree POC values of all pictures in the same layer tree in the same access unit are equal. This selection of the previous TID0 picture enables down- and up-switching of the number of layers, because the referenced TID0 picture is not chosen from the same layer, when the current picture is an IRAP picture in a dependent layer (i.e. layer up-switch point).

- The function DiffPicOrderCnt( ) is specified to use LayerTreePicOrderCntVal values.

- The POC MSB cycle is conditionally signalled in the slice segment header extension enabling its use also in base-layer slices. It is asserted that this change enables pruning of the base layer so that it only contains IRAP pictures, for which the POC difference can be greater than MaxPicOrderCntLsb / 2. Such pruning can be used for example when only IRAP pictures of the base layer may be used as references for inter-layer prediction (as indicated by max_tid_il_ref_pics_plus1[ i ][ j ]).

- The layer-tree POC MSB cycle is conditionally signalled in the slice segment header extension for additional error resilience and for enabling independent layer-tree POC derivation for each of multiple independent layers in the same layer tree.

Possibilities to combine certain aspects of the POC reset approach (JCTVC-P0041v3/JCT3V-G0031v3) to this contribution and vice versa are discussed. The following proposals are made:

- Instead of deriving POC values from poc_msb_val in enhancement layers with reference layers, it is proposed to derive them from the TID0 pictures of their reference layers.This is asserted to improve loss resilience, be robust against all kinds of layer switching, and reduce bitrate overhead.

- POC MSB signalling similar to that of JCTVC-P0041v3/JCT3V-G0031v3 has been incorporated in this contribution.

# Introduction

It is asserted that the poc_reset_flag design (JCTVC-P0041v4/JCT3V-G0031v4) has the following implications:

Constraints on picture coding patterns:

- RASL pictures are additionally constrained as follows: In HEVC v1 there is a requirement that any RASL picture associated with a CRA picture shall follow, in output order, any IRAP picture that precedes the CRA picture in decoding order. In the fourth AHG10 teleconference it was pointed out that outputting all the pictures in the DPB that are marked as "needed for output" at the beginning of the first access unit of a POC resetting period would cause an additional constraint on RASL pictures. That is, any RASL picture associated with a CRA picture shall follow, in output order, any picture that precedes the CRA picture in decoding order.

- The output order of pictures following, in decoding order, an IRAP picture at any layer is constrained.

JCTVC-P0041v5/JCT3V-G0031v5 constraints the output order so that no picture following, in decoding order, a POC resetting picture is allowed to precede, in output order, the POC resetting picture. In the following figure (copied from JCTVC-P0041v5/JCT3V-G0031v5) illustrates a non-conforming bitstream where the picture with POC 2 is forbidden when the BL picture with POC 4 is an IRAP picture.

This contribution has a similar but a bit looser constraint which applies to base-layer IDR and BLA pictures only.

- The POC resetting periods of independent layers cannot be overlapped.

This is illustrated with the following example, which could for example be used to code a field sequence of one parity as the base layer and a field sequence of the opposite parity as the enhancement layer. In this case the CRA picture in the enhancement layer cannot start a new POC resetting period, because that would also cause a all earlier pictures, in decoding order, in all layers to be output in the output-order-conforming HRD. Hence, CRA 0 of the base layer would be output prior to the BL RASL pictures that should precede the BL CRA picture in output order.

It is not clear how the JCTVC-P0041v4/JCT3V-G0031v4 should be used in such a use case where POC resetting periods of independent layers would naturally overlap.

Impacts of dynamically changing POC values:

- If POC values are provided with the decoded and cropped pictures when the pictures are output, it is not clear if the POC values are those that are currently in effect in the DPB or those that were in effect when the pictures were decoded.

This should be clarified to have an unambiguous interface between the decoder and any post-processing units.

- The POC values are no longer static. This would complicate the use of POC values to label pictures in post-processing operations, which is required for example to conclude the persistence of some SEI messages.

  - Background: the persistence of the following SEI messages is specified similarly to the following: [the message pertains until] “the PicOrderCntVal of the next slice to be decoded is greater than currPicOrderCntVal + pic_order_cnt_delta, where currPicOrderCntVal is the value of PicOrderCntVal of the picture in the access unit containing the SEI message”

    - Recovery point SEI message

    - Progressive refinement start SEI message

    - Region refresh information SEI message

  - It has been proposed in JCTVC-P0041v4/JCT3V-G0031v4 that the persistence of the above three SEI messages is specified so that the picture order count of the picture where the SEI message appears is updated (whenever a new POC reset period appears) according to the POC derivation of MV-HEVC/SHVC. This appers to require re-deriving the PicOrderCntVal of the picture containing the SEI message after each POC resetting picture of the same layer. When SEI messages are handled by post-processing units, such as a displaying process, this design requires the post-processing unit to keep updating POC values identically to how the decoder operates. When SEI messages are handled by decoders, this design requires decoders to maintain and update POC values for the pictures that originally contained the SEI messages. Moreover, it is noted that the picture where the SEI message appeared may no longer be in the DPB, so its present POC value might not be available by accessing the DPB.

  - The structure of pictures information SEI message also has similar persistence than the SEI messages named above and additionally uses POC within the persistence scope to identify pictures.

  - It has been proposed in JCTVC-P0041v4/JCT3V-G0031v4 that the use of the structure of pictures information SEI message SEI message would be constrained in scalable bitstreams so that the persistence would not cross a POC resetting period.

  - Additionally, pan-scan rectangle, tone mapping information, frame packing arrangement, and display orientation SEI messages cease to persist when a picture with a greater POC value than that of the current picture are output. JCTVC-P0041v4/JCT3V-G0031v4 clarifies that the semantics of these SEI messages like follows:

Let picA be the current picture. pan_scan_rect_persistence_flag equal to 1 specifies that the pan-scan rectangle information persists in output order until any of the following conditions are true:

–	A new CVS begins.

–	The bitstream ends.

–	A picture picB in an access unit containing a pan-scan rectangle SEI message with the same value of pan_scan_rect_id is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

In order to conclude the correct persistence scope, the SEI message has to be decoded using the POC derivation of MV-HEVC/SHVC, which is a non-backward-compatible change.

An approach that POC values can be derived from their reference layers has been presented in this contribution and, earlier, as part of JCTVC-O0275v4/JCT3V-F0092v4 and its attachment "PrevPicOrderCnt change to JCTVC-O0213v4_JCT3V-F0073v4_attachment.doc" relative to the POC reset approach. It is asserted that POC MSB derivation from reference layers would significantly reduce the number of pictures where the poc_msb_val has to be included and hence reduce the bitrate overhead without sacrificing loss robustness. poc_msb_val would have to be present only for independent-layer IRAP pictures rather than IRAP pictures in any layers or pictures aligned with base-layer IRAP pictures.

Due to the above-mentioned reasons, if JCTVC-P0041v4/JCT3V-G0031v4 were adopted as basis for multi-layer POC derivation, it is proposed to modify it as follows:

Instead of deriving POC values from poc_msb_val in enhancement layers with reference layers, it is proposed to derive them from the TID0 pictures of their reference layers.

A design that would not change the POC values of the base layer and would keep the POC values static is targeted in this contribution.

It is remarked that PicOrderCntVal is used in the decoding process only as input to the DiffPicOrderCnt( ) function. This opens the possibility to re-specify the DiffPicOrderCnt( ) function to use other inputs than PicOrderCntVal.

# Overview of the Proposal

In the example of Figure 1, there is access unit with an IDR picture in the base layer, which resets the POC value to 0. The layer-tree POC values are anyhow continued by including layer_tree_poc_lsb with the base-layer IDR picture. In this example layer_tree_poc_lsb is equal to 10, and since the derived layer-tree POC for the POC reference picture is 1032 and consequently the layer-tree POC LSB is equal to 8 (when MaxPicOrderCntLsb is equal to 16), the layer-tree POC value is incremented by 2 for the base-layer IDR picture.

Figure 1. An example of layer-tree POC signalling and derivation.

Table 1 attempts to summarize whether this contribution proposed to use POC or layer-wise POC for different POC related functionalities.

Table 1. A summary of the use of POC and layer-tree POC for different POC related functionalities.

| POC-related functionality | POC or layer-tree POC used? How? |
| --- | --- |
| Decoding process (e.g. motion vector scaling in TMVP) | No changes in the base-layer decoding. For decoding of enhancement layers, the difference of layer-tree POCs is used. For decoding of the enhancement layers, the function DiffPicOrderCnt( picA, picB ) is specified to use layer-tree POCs. |
| Access unit boundary determination | It is not clear if POC related information is needed for AU boundary detection, see JCTVC-P0043/JCT3V-G0032. If POC related information is needed for AU boundary detection, it is sufficient to use pic_order_cnt_lsb, while PicOrderCntVal can likewise be used. Consecutive pictures having the same pic_order_cnt_lsb are contained in the same access unit. An AU boundary can be concluded when i) there is a picture with first_slice_segment_in_pic_flag equal to 1 and nuh_layer_id equal to 0, or ii) slice_pic_order_cnt_lsb differs in adjacent pictures. |
| Reference picture set signalling | POC for the base layer. Layer-tree POC for enhancement layers. |
| Picture output order determination in the bumping process of the HRD | POC. CVSs are output in their decoding order. Within a CVS, the output order of access units is determined through the POC value. (See Section 3.4 for more details.) |
| SEI message semantics Use of SEI messages by post-processing units | POC for the base layer. Layer-tree POC for enhancement layers. (See Section 3.4 for more details.) |
| Feedback based error recovery | POC for the base layer. Layer-tree POC for enhancement layers. (See Section 3.5 for more details.) |

The detailed proposal is indicated with change marks in the accompanying specification text document.

# Evaluation of the Proposal

## Evaluation Criteria

The agreed evaluation criteria for multi-layer POC derivation proposals as reported in the BoG report JCTVC-O0349/JCT3V-F0248 are copied below:

- Solution does not cause any problems in the no loss case, including sub-bitstream extraction and layer switching

- Understanding of what picture coding patterns were disallowed

- Error resiliency aspects: prioritize good POC behavior for typical use cases, and prioritizing when the decoded pictures themselves were likely to be useful

  - POC corruption caused by picture loss should be detectable

  - POC problems shouldn’t persist beyond sample accuracy problems, after a loss

- Specification consistency

- POC values can be determined for feedback based error recovery (classified as an additional evaluation criteria)

The proposal is asserted to meet the evaluation criteria favorably as described in the following sub-sections.

## Constraints in Picture Coding Patterns

It is asserted that there are no known constraints on picture coding patterns. Considerations on the following scenarios have been given:

- Unaligned IRAP pictures across layers, IDR picture in the base layer:

- Unaligned IRAP pictures across layers, IDR picture in the enhancement layer:

- Unaligned IRAP pictures across layers, layer down-switching followed by layer up-switching at an IDR picture in the enhancement layer:

- Only base-layer IRAP pictures used as references for inter-layer prediction, removal of non-IRAP pictures from the base layer.

Encoders have two options, either to signal POC MSB for the base-layer IRAP pictures or to make slice_pic_order_cnt_lsb codeword longer such that the POC interval between adjacent IRAP pictures is less than or equal to MaxPicOrderCntLsb / 2. These options are illustrated below.

- EL picture not present in an access unit with BL IDR picture

- Two independent layers and a dependent enhancement layer in the same layer tree

- Two independent layers, which are never present in the same access unit

## Error Resiliency

It is asserted that the derivation of LayerTreePicOrderCntVal is similarly loss-tolerant as the derivation of PicOrderCntVal for the base-layer pictures.

## Specification Consistency

- Use with SEI messages the semantics of which depend on picture order count values:

  - According to the contribution PicOrderCntVal and LayerTreePicOrderCntVal are static (i.e. are required to be derived only when the picture is decoded and need not be updated when later pictures are decoded).

  - SEI messages applying to base-layer pictures or entire access units are proposed to use PicOrderCntVal in their semantics (i.e. no change to version 1 is needed).

  - It is proposed that SEI messages with nuh_layer_id greater than 0 or included in a scalable nesting SEI message use LayerTreePicOrderCntVal in their semantics when the SEI message applies to enhancement-layer pictures.

  - Consequently, it is asserted that the semantics of SEI messages are unambigious and post-processing units handling the SEI messages need not update the POC values of past pictures (according to POC derivation of later pictures) but can rely on the POC values of the pictures at the time they are decoded.

- Determination of output order:

  - The determination of output order basically follows the base-layer output order determination. In other words, access units of a CVS are contiguous in output order.

  - The output order determination is first based on the CVS order, i.e. all access units of an earlier CVS are output before any access unit of a later CVS. Within a CVS, the access unit output order is determined by PicOrderCntVal. All the pictures of an access unit (with PicOutputFlag equal to 1) are output contiguously.

  - The proposed text for the output order determination for the HRD is included in the accompanying document "JCTVC-O0275 DPB operation (relative to JCTVC-O0217_JCT3V-F0075_Changes to Spec E1004-v6).doc".

## Use with Feedback Based Error Recovery

It is proposed that PicOrderCntVal is used for identifying base-layer pictures and LayerTreePicOrderCntVal is used for identifying enhancement-layer pictures in the feedback messages.

As the PicOrderCntVal and LayerTreePicOrderCntVal are static, their use in labeling of pictures in the feedback messages is unambiguous. Furthermore, as PicOrderCntVal and LayerTreePicOrderCntVal are derived in a loss-tolerant manner, the correct interpretation of the feedback messages (by the encoder that receives the feedback messages) is asserted to be guaranteed.

For example, the Reference Picture Selection Indication (RPSI) payload of RTP/AVPF (IETF RFC 4585) could be specified for MV-HEVC/SHVC streams as follows:

The field "Native RPSI bit string defined per codec" is a base16 [RFC4648] representation of the 8 bits consisting of 2 most significant bits equal to 0 and 6 bits of nuh_layer_id, as defined in [HEVC], followed by the 32 bits representing the value of the PicOrderCntVal (in network byte order) when nuh_layer_id is equal to 0 or the 32 bits representing the value of the LayerTreePicOrderCntVal (in network byte order), as defined in [HEVC], for the picture that is requested to be used for reference when encoding the next picture.

# Response to the comments in JCTVC-P0041v5/JCT3V-G0031v5

The comments are repeated in most parts below and responses are given:

- Comment: After an IRAP picture in layerA with NoRaslOutputFlag equal to 1, if there is immediately one or more AUs where there is no picture in layerA but there are pictures in those AUs of another layer, then the picture output process would not work, unless there is a restriction on output order to disallow those pictures to have output order earlier than any picture in an AU that precedes the AU containing the IRAP picture. This is similar to the output order restriction added in P0041v4.

Response: Agreed. A similar but a bit looser constraint has been added: A picture with nuh_layer_id greater than 0 following, in decoding order, a IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0 shall not precede, in output order, any picture preceding, in decoding order, the IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0.

- Comment: There is a hidden requirement that slice_pic_order_cnt_lsb must be cross-layer aligned. This should be clearly stated.

Response: Agreed and added to the text.

- Comment: On AU boundary detection, Table 1 of O0275v6 says to use (first_slice_segment_in_pic_flag equal to 1 and nuh_layer_id equal to 0) or (slice_pic_order_cnt_lsb differs in adjacent pictures). However, this is not sufficient, because the base layer picture may be absent or lost in some AUs, and the slice_pic_order_cnt_lsb values can be the same in adjacent pictures – and the requirement of cross-layer alignment of slice_pic_order_cnt_lsb increases the possibility of the same value of slice_pic_order_cnt_lsb in adjacent pictures.

Response: The AU boundary detection in the standard needs to be correct only for loss-free bitstreams. It is questioned how common the envisioned loss case would be, because:

  - IDR pictures next to each other are not commonly used.

  - Typical IDR pictures are much larger than typical MTU size and hence would be split into multiple packets. It is asserted that losing all these packets is not very common.

  - Typical systems provide other means to detect losses than relying solely on the RPS. For example sequence numbers can be used to detect losses.

- Comment: On POC-related SEI message semantics, Table 1 of O0275v6 says to use POC for the base layer and layer-tree POC for enhancement layers. But what about nested SEI messages that apply to a set of layers containing the base layer and one or more enhancement layers? Also, since LayerTreePicOrderCntVal does not need to be cross-layer aligned, what about nested SEI messages that apply to a set of enhancement layers?

Response: The SEI messages with POC related semantics should be enclosed in the scalable nesting SEI message so that the semantics can be applied to all layers indicated in the scalable nesting SEI message. For example, the scalable nesting SEI message containing SEI messages with POC related semantics should not cross layer-trees.

- Comment: The output process involving BaseLayerCVSCnt has three problems:

- The value range of BaseLayerCVSCnt of is undefined. So how many bits a decoder needs for that variable?

- When the value of BaseLayerCVSCnt wraps-up, the process would not work.

- The process would not work with CRA pictures with NoRaslOutputFlag equal to 1.

Response: The value range for the variable was not considered important, as the HRD documents a hypothetical decoder and a real decoder may implement its output differently. Anyhow, the value range for BaseLayerCVSCnt could be constrained based on the maximum number of picture storage buffers in one sub-DPB (presumably 16). In other words, in the worst case the base-layer sub-DPB is filled in by IDR pictures. It is sufficient to have value range that is 1 larger than the maximum number of picture storage buffers in one sub-DPB and to maintain a "read pointer" (the next base layer CVS index in modulo MaxBLCvsIdx arithmetic that is to be output) and a "write pointer" (the next index to be allocated or the last index that was used, similarly using modulo MaxBLCvsIdx arithmetic).

Comment 5c is not understood. Is it suggested that it is allowed to have base-layer CRA pictures with NoRaslOutputFlag equal to 1 but with NoClrasOutputFlag equal to 1? What is the use case for such use of the externally set flags?

- Comment: Why a sign flag is needed for POC MSB cycle and layer-tree POC MSB cycle? They are needed for a current picture in an AUs with IRAPs, which follows, in output order, any previous picture in decoding order, and the MSB cycle is the difference of the MSB values of the current picture and a previous picture in an AU with IRAPs.

Response: They are needed for the case when the first picture following, in decoding order, an independent-layer IRAP picture precedes, in output order, the independent-layer IRAP picture.

- Comment: The fact that both POC MSB cycle and layer-tree POC MSB cycle may be signalled, and that may be signalled also for non-IRAP pictures – and that can generate a relatively heavy overhead.

Response: The selection of TID0 pictures among the current layer and the direct reference layers was designed to reduce the overhead.

- Comment: How does the encoder sets the values of the POC MSB cycle and layer-tree POC MSB cycle?

Response: The encoder starts updating the POC MSB cycle from the first picture of the bitstream in the base layer or from the previous IDR or BLA picture of the base layer, whichever is later in decoding order. It starts updating the layer-tree POC MSB cycle when NoClRasOutputFlag is equal to 1 or when a new VPS is activated (see comment 11).

- Comment: The conditions on when the POC MSB cycle, layer-tree POC MSB cycle, and layer_tree_poc_lsb must be signalled are pretty complicated. Please clarify.

Response: Comments are interleaved with the spec text as [Ed.] below:

layer_tree_poc_lsb and layer_tree_poc_msb_present_flag shall be present, when MaxLayersMinus1 is greater than 0, cross_layer_irap_aligned_flag is equal to 0, and at least one of the following conditions is true for the current picture picA:

–	picA is an IDR picture with nuh_layer_id equal to 0.

–	picA is in an indepent layer that is not an independent root layer and is the first picture that follows, in decoding order, an IRAP picture in the independent root layer of the same layer tree. [Ed. This is to always enable layer-tree POC derivation of an independent layer without referring to the independent root layer of the same layer tree.]

–	picA has nuh_layer_id layerIdA greater than 0, picA follows, in decoding order, an IDR picture with nuh_layer_id equal to 0 and prevTid0Pic derived for picA as specified in subclause Error: Reference source not found precedes, in decoding order, the IDR picture with nuh_layer_id equal to 0. [Ed. When the POC reference/anchor picture precedes, in decoding order, a base-layer IDR picture, the layer-tree POC must be indicated explicitly.]

poc_msb_sign and poc_msb_cycle shall be present, when VpsMaxLayersMinus1 is greater than 0 and at least one of the following conditions is true for the current picture picA:

–	picA is a CRA or BLA picture in the base layer and the next IRAP picture picB, in decoding order, in the base layer is a CRA picture and has PicOrderCntVal such that PicOrderCnt( picB ) – PicOrderCnt( picA ) is greater than MaxPicOrderCntLsb / 2. [Ed. This is to enable pruning the base layer so that it only contains IRAP pictures which are further apart (in POC values) than MaxPicOrderCntLsb / 2.]

–	picA is an IRAP picture picture in an independent layer with nuh_layer_id greater than 0. [Ed. This is to enable transmission or decoding of independent layers to start at any point relative to the base layer, not merely within MaxPicOrderCntLsb / 2 apart (in POC values) from the first base layer picture.]

–	picA has nuh_layer_id greater than 0, picA follows, in decoding order, an IRAP picture with nuh_layer_id equal to 0 and prevTid0Pic derived for the current picture as specified in subclause Error: Reference source not found precedes, in decoding order, the IRAP picture with nuh_layer_id equal to 0. [Ed. This condition is to enable POC derivation an independent layer starting with any picture (not necessarily IRAP), when decoding is started from a BL IRAP picture or when the bitstream is spliced at a non-aligned IRAP position. Here "an IDR picture" (of JCTVC-O0275v7/JCT3V-F0092v7) was changed to "an IRAP picture".]

- Comment: The POC anchor picture, prevTid0Pic, can have discardable_flag equal to 1. What if the picture is discarded?

Response: Our assumption was that pictures with discardable_flag equal to 1 are also sub-layer non-reference pictures, which are not selected as the POC anchor picture. We notice, however, that there is no constraint requiring discardable_flag to be equal to 0 for IRAP pictures. We would think such a constraint makes more sense than further constraining the selection of prevTid0Pic to have discardable_flag equal to 0 (which of course could be easily specified too).

- Comment: The definitions of layer tree and layer tree pic order count value are heavily dependent on the reference structure of layers, and moreover there seems to be an assumption that they only lose persistence when NoClrasOutputFlag equal to 1. However, the VPS activation could also occur at an AU that has all IDR pictures, and is complete (all layers present and all layer pictures are IDR). What happens if a new VPS is activated with a different reference structure at an all-IDR access unit – what happens to layer tree definition then?

Response: Good comment. In our opinion, a new VPS (differing in content from the old one) should be handled like a "clean" access position, i.e. no references to pictures preceding the activation point, in decoding order, and no output interleaved with pictures preceding the activation point, in decoding order. The easiest way to achieve this behavior seemed to be to infer NoClrasOutputFlag equal to 1 when the active VPS changes in content – which was added in the text. It is noted that this change might be needed regardless of which multi-layer POC approach is adopted.

- Comment: When is layer_tree_poc_info_present_flag equal to 1?

Response: layer_tree_poc_info_present_flag is equal to 1 when layer_tree_poc_lsb and layer_tree_poc_msb_present_flag shall be present or when the encoder decides to encode layer-tree POC values for error resilience. (A related typo of missing words "equal to 1" was fixed in the specification text.)

- Comment: When is layer_tree_poc_msb_present_flag equal to 1? Is it correct to say that it is used to calculate the MSB of the POC of the first IRAP picture of an independent layer, when that layer is not the root layer of its layer tree? If so what happens if you have two independent layers in a layer tree, and the first access unit contains BLA in independent root layer, and non-IRAP in the independent non-root layer?

Response: See the response to comment 9. It is required to have layer_tree_poc_msb_present_flag equal to 1 in the example case for the non-IRAP in the independent non-root layer. The respective condition is: "picA is in an indepent layer that is not an independent root layer and is the first picture that follows, in decoding order, an IRAP picture in the independent root layer of the same layer tree."

- Comment: NoClrasOutputFlag is only associated with base layer. What does NoClrasOutputFlag for the IRAP picture in an independent root layer mean? Is the assumption that the at the splice point, all the pictures in each independent root layer is present?

Response: The assumption was that splicing could be performed in a non-aligned manner in different layer trees and hence NoClrasOutputFlag could be set separately for each layer tree. If the use of NoClrasOutputFlag were limited to the base layer only, the required changes for the layer-tree POC approach would be simpler.

# Appendix: Version History of JCTVC-O0275/JCT3V-F0092

Version 2 of the contribution included an illustrative figure and updated the specification text to handle properly an access containing a base-layer IDR picture while some enhancement layer pictures are absent as well as enhancement-layer IRAP picture POC derivation.

Version 3 of the contribution (dated 13 Nov. 2013) updates the design and the proposed specification text to address comments received during the JCT-VC/JCT-3V review as follows:

- Reference picture sets for enhancement layers are proposed to use layer-wise picture order count to avoid the first ue(v)-coded delta POC value in the reference picture set syntax to be a relatively long codeword and to reduce the likelihood of having to use the MSB cycle signalling to differentiate between long-term pictures.

- To avoid the vulnerability of the loss of the enhancement-layerTID0 picture preceding a base-layer IDR picture in decoding order in the derivation of the layer-wise POC values correctly, the layer_delta_pic_order_cnt_minus1 syntax element in the slice header was replaced by layer_pic_order_cnt_lsb, which provides the LSBs of layer-wise POC value. This LSB value is used for layer-wise POC derivation similarly to how slice_pic_order_cnt_lsb is used for POC derivation.

It is asserted that consequently the proposed scheme is tolerant of losses of enhancement-layer TID0 pictures as long as there is a received TID0 picture for which ( prevLayerPicOrderCntLsb − layer_pic_order_cnt_lsb ) results into the same half of the total range of 0 to MaxPicOrderCntLsb – 1, inclusive, regardless of whether prevLayerPicOrderCntLsb is assigned to be the LayerPicOrderCntLsb of the received TID0 picture or the TID0 picture that the encoder used for the layer-wise POC derivation.

- Version 3 fixes a bug in the previous versions where PicOrderCntMsb was not correctly updated for the cases in which:

  - A first access unit contains a base-layer IRAP picture with NoRaslOutputFlag equal to 1 but no pictures in some enhancement layer(s) and later access units, in decoding order, precede the first access unit in output order.

  - A first access unit contains a base-layer IRAP picture with NoRaslOutputFlag equal to 1 but no pictures in an enhancement layer and the first picture of the enhancement layer is contained in an  access unit for which the PicOrderCntMsb value is greater than 0. This case could happen, for example, in the adaptive resolution change (ARC) use case.

The bug was fixed by assigning prevTid0Pic for the first picture within an enhancement layer to be the previous picture in decoding order that has TemporalId equal to 0 and nuh_layer_id equal to 0 or any nuh_layer_id value among the direct and indirect reference layers of the current picture and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture.

Version 3 of the contribution also provides a summary table how POC or layer-wise POC is used for different POC-related functionalities and describes how the contribution performs the agreed evaluation criteria of multi-layer POC derivation.

Version 4 of the proposal included the following ingredients:

- A layer tree is defined to consist of one independent layer (without any reference layers) and all layers that directly or indirectly use the independent layer as reference.

- Slice header extensions optionally contain signalling for a layer-tree POC value, which is intended to be used in the IRAP pictures of the independent layers and may be used, for additional error resilience, in other pictures too.

- A layer-tree POC value, LayerTreePicOrderCntVal is derived for each picture based on the mentioned slice header signalling, if present, and based on PicOrderCntVal differences, otherwise.

- Both the POC value and the layer-tree POC value are derived relative to the POC value and the layer-tree POC, respectively, of the previous TID0 picture on the current layer or any lower layer in the same layer tree on which the decoding of the current picture may depend on. The layer-tree POC values of all pictures in the same layer tree in the same access unit are equal. This selection of the previous TID0 picture enables down- and up-switching of the number of layers, because the referenced TID0 picture is not chosen from the same layer, when the current picture is an IRAP picture in a dependent layer (i.e. layer up-switch point).

- The function DiffPicOrderCnt( ) is specified to use LayerTreePicOrderCntVal values.

Possibilities to combine certain aspects of the POC reset approach (JCTVC-O0213/JCT3V-F0073) to this contribution and vice versa were discussed in version 4. The following proposals were made:

- PrevPicOrderCnt derivation in the POC reset approach is proposed to be modified to happen among the current layer as well as direct and indirect reference layers. This is asserted to improve loss resilience.

- If it is preferred to include base-layer IDR/BLA picture loss detection means in the slice segment header syntax, enhancement-layer slice headers in this contribution can contain POC resetting information. This signalling shall be used for a picture that is in a different layer tree than the layer tree containing the base layer and which is the first picture, in decoding order, in a layer tree after a base-layer IDR or BLA picture. The signalling may also be used in other pictures for additional error resilience.

Version 5 of the contribution updated the layer tree definition so that multiple independent layers in the same layer tree are enabled. A layer tree index is derived for each layer and identifies which layer tree the layer belongs to.

Version 6 of the contribution includes the following changes relative to version 5:

- The POC MSB cycle is conditionally signalled in the slice segment header extension enabling its use also in base-layer slices (whereas in version 5 the POC MSB cycle was signalled within the slice segment header for enhancement layer slices only). It is asserted that this change enables pruning of the base layer so that it only contains IRAP pictures, for which the POC difference can be greater than MaxPicOrderCntLsb / 2. Such pruning can be used for example when only IRAP pictures of the base layer may be used as references for inter-layer prediction (as indicated by max_tid_il_ref_pics_plus1[ i ][ j ]).

- The layer-tree POC MSB cycle is conditionally signalled in the slice segment header extension for additional error resilience and for enabling independent layer-tree POC derivation for each of multiple independent layers in the same layer tree.

- Inter-layer dependencies in POC and layer-tree POC derivation are constrained further as follows:

  - The POC reference picture prevTid0Pic is selected from direct reference layers (rather than from direct or indirect reference layers as in version 5).

  - A non-IRAP picture from a direct reference layer is not prevTid0Pic, when max_tid_il_ref_pics_plus1[ i ][ j ] indicates that only IRAP pictures of the reference layer may be used as references for inter-layer prediction.

  - In version 5, when a layer tree contained more than one independent layer, TID0 pictures of the independent root layer (with the smallest value of nuh_layer_id among all independent layers of the layer tree) could serve as prevTid0Pic. In version 6, pictures of an independent root layer are no longer candidates for prevTid0Pic (unless the independent root layer is the current layer or a direct reference layer for the current layer).

As a consequence, it is asserted that POC and layer-tree POC derivation have the same or a subset of the reference pictures than what inter-layer prediction has.

Version 7 of the contribution updates the introduction, i.e. Section 1 (analysis of JCTVC-P0041v4/JCT3V-G0031v4). Change marks of Section 1 were first accepted.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).