| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0052 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0044 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: VPS extension clean-up |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjin Cho Byeongdoo Choi Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Tel: Email: | {yongjin9.cho, b.d.choi, m.w.park, jinyoung79, hc.wey, dionism}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes modification of video parameter set (VPS) extension syntax structure for its intended usage.

# Problem statement

The current design of the VPS extension does not guarantee its intended usage. Hence, this contribution proposes some modification of the current design toward its purpose.

# Proposal

- profile_ref_minus1

Current syntax and semantics:

| vps_extension( ) { | Descriptor |
| --- | --- |
| …… |  |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| …… |  |
| } |  |

profile_ref_minus1[ i ] specifies that the profile and tier information for the i-th profile_tier_level( ) syntax structure is inferred to be equal to the profile and tier information for the ( profile_ref_minus1[ i ] + 1 )-th layer set. The value of profile_ref_minus1[ i ] + 1 shall be less than or equal to i.

Problem: Broken syntax and semantics

When i is equal to 1 and the value of vps_profile_present_flag[1] is equal to 0 (i.e., when the 1st profile information is not present), profile_ref_minus1[1] must take a value corresponding to the 0th profile information. But, because the minimum value of the syntax element is 0 (i.e., the value that points at 1st profile information that is not present), reference to 0th profile information is impossible with the current syntax and semantics.

In the current semantics, the value of profile_ref_minus1[i]+1 is allowed to be equal to i, which means that the profile reference of the i-th layer is i-th profile information. But, profile_ref_minus1[i] is present when i-th profile information is not available, which is self contradictory.

Proposed syntax and semantics:

| vps_extension( ) { | Descriptor |
| --- | --- |
| …… |  |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] profile_ref [ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| …… |  |
| } |  |

profile_ref_minus1[ i ] specifies that the profile and tier information for the i-th profile_tier_level( ) syntax structure is inferred to be equal to the profile and tier information for the ( profile_ref_minus1[ i ] + 1 )-th layer set. The value of profile_ref_minus1[ i ] + 1 shall be less than or equal to i.

- vps_max_layer_id and layer_id_included_flag in video_parameter_set( )

Current syntax and semantics:

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_reserved_three_2bits | u(2) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_extension_flag | u(16) |
| profile_tier_level(1, vps_max_sub_layers_minus1) |  |
| vps_sub_layer_ordering_info_present_flag | u(1) |
| for ( i=(vps_sub_layer_ordering_info_present_flag ? 0: vps_max_sub_layers_minus1); i <= vps_max_sub_layers_minus1; i++) { |  |
| vps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| vps_max_layer_id | u(6) |
| vps_num_layer_sets_minus1 | ue(v) |
| for ( i = 1 ; i <= vps_num_layer_sets_minus1 ; i++ ) |  |
| for ( j = 0 ; j <= vps_max_layer_id ; j ++ ) |  |
| layer_id_included_flag[ i ][ j ] | u(1) |
| …… |  |
| } |  |

| vps_extension( ) { | Descriptor |
| --- | --- |
| …… |  |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| …… |  |
| for ( i = 1 ; i < NumOutputLayerSets; i ++ ) { |  |
| if ( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for ( j = 0 ; j < NumLayersInIdList[ lsIdx ] -1 ; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| …… |  |
| } |  |

Problem: Parsing dependency

From the current syntax, NumLayersInIdList variable in vps_extension( ) is calculated from a loop containing layer_id_included_flag in video_parameter_set_rbsp( ). If the parsing of the loop in video_parameter_set_rbsp( ) is skipped (e.g., in the middle box), the value of NumLayersInIdList is not known. Also, the variable NumSubDpbs that signals the number of sub DPBs is set equal to the value of NumLayersInIdList. Hence, without the value of NumLayersInIdList vps_extension( ) cannot be parsed correctly.

Proposal 1: Mirror layer_id_included_flag and vps_max_layer_id in vps_extension( ) similarly to vps_number_layer_sets_minus1 (vps_layer_id_included_flag and vps_maximum_layer_id)

Syntax and semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| …… |  |
| vps_number_layer_sets_minus1 | u(10) |
| vps_maximum_layer_id | u(6) |
| for ( i = 1 ; i <= vps_number_layer_sets_minus1 ; i ++) |  |
| for ( j = 0 ; j <= vps_maximum_layer_id ; j ++ ) |  |
| vps_layer_id_included_flag[ i ][ j ] | u(1) |
| vps_num_profile_tier_level_minus1 | u(6) |
| …… |  |
| } |  |

- vps_maximum_layer_id specifies the maximum allowed value of nuh_layer_id of all NAL units in the CVS. The value of vps_maximum_layer_id shall be equal to vps_max_layer_id.

vps_layer_id_included_flag[ i ][ j ] equal to 1 specifies that the value of nuh_layer_id equal to j is included in the layer identifier list LayerSetLayerIdList[ i ]. vps_layer_id_included_flag[ i ][ j ] equal to 0 specifies that the value of nuh_layer_id equal to j is not included in the layer identifier list LayerSetLayerIdList [ i ]. The value of vps_layer_id_included_flag[ i ] [ j ] shall be equal to layer_included_flag[ i ][ j ] for the values i from 1 to vps_number_layer_sets_minus1, inclusive and for the values of j from 0 to vps_maximum_layer_id, inclusive.

The value of NumLayersInIdList[ 0 ] is set equal to 1 and the value of LayerSetLayerIdList [ 0 ][ 0 ] is set equal to 0.

For each value of i in the range of 1 to vps_number_layer_sets_minus1, inclusive, the variable NumLayersInIdList[ i ] and the layer identifier list LayerSetLayerIdList [ i ] are derived as follows:

n = 0
for( m = 0; m  <=  vps_maximum_layer_id; m++ )
	if( vps_layer_id_included_flag[ i ][ m ] )	
		LayerSetLayerIdList[ i ][ n++ ] = m
NumLayersInIdList[ i ] = n

For each value of i in the range of 1 to vps_number_layer_sets_minus1, inclusive, NumLayersInIdList[ i ] shall be in the range of 1 to vps_maximum_layers_minus1 + 1, inclusive.

When NumLayersInIdList[ iA ] is equal to NumLayersInIdList [ iB ] for any iA and iB in the range of 0 to vps_number_layer_sets_minus1, inclusive, with iA not equal to iB, the value of LayerSetLayerIdList [ iA ][ n ] shall not be equal to LayerSetLayerIdList [ iB ][ n ] for at least one value of n in the range of 0 to NumLayersInIdList [ iA ], inclusive.

A layer set is identified by the associated layer identifier list. The i-th layer set specified by the VPS extension is associated with the layer identifier list LayerSetLayerIdList [ i ], for i in the range of 0 to vps_num_layer_sets_minus1, inclusive.

A layer set consists of all operation points that are associated with the same layer identifier list.

- Each operation point is identified by the associated layer identifier list, denoted as OpLayerIdList, which consists of the list of nuh_layer_id values of all NAL units included in the operation point, in increasing order of nuh_layer_id values, and a variable OpTid, which is equal to the highest TemporalId of all NAL units included in the operation point. The bitstream subset associated with the operation point identified by OpLayerIdList and OpTid is the output of the sub-bitstream extraction process as specified in clause Error: Reference source not found with the bitstream, the target highest TemporalId equal to OpTid, and the target layer identifier list equal to OpLayerIdList as inputs. The OpLayerIdList and OpTid that identify an operation point are also referred to as the OpLayerIdList and OpTid associated with the operation point, respectively.

Proposal 2: Signal the number of layers in ID list (vps_num_layers_in_id_list)

Syntax and semantics:

| vps_extension( ) { | Descriptor |
| --- | --- |
| …… |  |
| vps_number_layer_sets_minus1 | u(10) |
| for ( i = 1 ; i <= vps_number_layer_sets_minus1 ; i ++) | u(6) |
| vps_num_layers_in_id_list[ i ] |  |
| …… |  |
| for ( i = 1 ; i < NumOutputLayerSets; i ++ ) { |  |
| if ( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for ( j = 0 ; j < NumLayersInIdList vps_num_layers_in_id_list [ lsIdx ] -1 ; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| …… |  |
| } |  |

vps_num_layers_in_id_list[ i ] specifies the number of layers in the i-th layer set.

output_layer_set_idx_minus1[ i ] plus 1 specifies the index of the layer set for the i-th output layer set. The value of output_layer_set_idx_minus1[ i ] shall be in the range of 0 to vps_num_layer_sets_minus1 − 1, inclusive. The length of the output_layer_set_idx_minus1[ i ] syntax element is Ceil( Log2( vps_num_layer_sets_minus1 ) ) bits.

The layer set for the i-th output layer set with i in the range of 0 to vps_num_layer_sets_minus1, inclusive, is inferred to be the i-th layer set.

The variable NumSubDpbs[ i ],specifying the number of sub-DPBs for the i-th output layer set, is set equal to NumLayerInIdList vps_num_layers_in_id_list[ i ].

- Semantics of sub_layer_dpb_info_present_flag in dpb_size( ) syntax structure

Current semantics:

sub_layer_dpb_info_present_flag[ i ][ j ] equal to 1 specifies that max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] is present for k in the range of 0 to NumSubDpbs[ i ] – 1, inclusive, for the j-th sub-layer, and max_vps_num_reorder_pics[ i ][ j ] and max_vps_latency_increase_plus1[ i ][ j ] are present for the j-th sub-layer. sub_layer_dpb_info_present_flag[ i ][ j ] equal to 0 specifies that the values of max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] are equal to max_vps_dec_pic_buffering_minus1[ i ][ k ][ j − 1 ] for k in the range of 0 to NumSubDpbs[ i ] – 1, inclusive, and that the values max_vps_num_reorder_pics[ i ][ j ] and max_vps_latency_increase_plus1[ i ][ j ] are set equal to max_vps_num_reorder_pics[ i ][ j - 1 ] and max_vps_latency_increase_plus1[ i ][ j - 1 ], respectively. The value of sub_layer_dpb_info_present_flag[ i ][ 0 ] for any possible value of i is inferred to be equal to 1.

Problem: No inference rule when the syntax element is not present

Proposed semantics:

sub_layer_dpb_info_present_flag[ i ][ j ] equal to 1 specifies that max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] is present for k in the range of 0 to NumSubDpbs[ i ] – 1, inclusive, for the j-th sub-layer, and max_vps_num_reorder_pics[ i ][ j ] and max_vps_latency_increase_plus1[ i ][ j ] are present for the j-th sub-layer. sub_layer_dpb_info_present_flag[ i ][ j ] equal to 0 specifies that the values of max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] are equal to max_vps_dec_pic_buffering_minus1[ i ][ k ][ j − 1 ] for k in the range of 0 to NumSubDpbs[ i ] – 1, inclusive, and that the values max_vps_num_reorder_pics[ i ][ j ] and max_vps_latency_increase_plus1[ i ][ j ] are set equal to max_vps_num_reorder_pics[ i ][ j - 1 ] and max_vps_latency_increase_plus1[ i ][ j - 1 ], respectively. The value of sub_layer_dpb_info_present_flag[ i ][ 0 ] for any possible value of i is inferred to be equal to 1. When sub_layer_dpb_info_present_flag[i][j] for j greater than 0 is not present, the value of sub_layer_dpb_info_present_flag[i][j] is inferred to be 0.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).