| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0047 |
| --- | --- |

| Title: | 3D-CE2 related: Simplification on interview motion prediction for depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Yi-Wen Chen, Jian-Liang Lin, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yulin.chang, yiwen.chen, jl.lin, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the interview motion vector prediction for depth coding uses a disparity vector derived from neighboring reconstructed depth pixels to retrieve the corresponding motion vector in the reference view. The decoding pipeline needs to wait for the reconstruction of the neighboring depth pixels to proceed the derivation of the disparity vectors. This procedure may produce bubble cycles in the decoder pipeline. It is proposed to use depth oriented neighboring block disparity vector (DoNBDV) with an initial disparity vector converted from a constant depth value 128 in the interview motion vector prediction for depth coding in this contribution. The experimental results reportedly show 0.1% coding gain for synthesized views.

# Introduction

In the current 3D-HEVC, the interview motion vector prediction is achieved by using an estimated disparity vector to retrieve the corresponding motion vectors in the reference view. The estimated disparity vector comes from neighboring block disparity vector (NBDV) or depth-oriented neighboring block disparity vector (DoNBDV) in texture coding, and it uses the disparity vectors converted from the neighboring depth pixels in depth coding, respectively. However, for depth coding, the decoder needs to wait for the reconstruction of neighboring blocks to proceed the derivation of the disparity vectors which are converted from the neighboring depth pixels. In this contribution, it is proposed to use DoNBDV with an initial disparity vector converted from a constant depth value 128 instead of the disparity vector derived from the neighboring depth pixels for the interview motion vector prediction in depth coding.

# Proposed schemes

In this contribution, it is proposed to change the estimated disparity vector from the disparity vectors converted by neighboring depth pixels to the DoNBDV, which is already used in texture coding. A further simplification is done by replacing the NBDV part for depth coding with the disparity vector converted from a constant depth value 128. The changes are as following:

## Use the DoNBDV as the estimated disparity vector for interview motion vector prediction

The estimated disparity vector used for interview motion vector prediction is changed to the DoNBDV instead of the disparity vector converted by neighboring depth pixels. The DoNBDV procedure uses the reconstructed depth pixels from base view, which is already decoded, reconstructed, and stored in the memory. The decoder does not need to wait for the reconstruction of the neighboring depth pixels to proceed the derivation of estimated disparity vectors.

## Replace NBDV part by a disparity vector converted from a constant depth value 128 for depth coding

Further simplification of the proposed scheme is done by replacing the NBDV part with a disparity vector converted from a constant depth value 128 for depth coding. The constant depth value 128 is the middle value of the depth range [0, 255]. The depth map can present the 3D structure of a scene more precisely, so using a constant disparity vector for initial warping is useful to locate a rough location of the scene structure. The simplified DoNBDV does not need to access the motion vectors of neighboring blocks. It only accesses the four corners of the reconstructed depth pixels within one coding unit in the depth reference picture, which is already reconstructed and can be referenced.

# Experiment Results

The proposed methods are integrated into HTM 9.0r1. Two tests are conducted. The first test changes the estimated disparity vector of interview motion vector prediction to a disparity vector converted from depth value 128 in depth coding. The second test changes the estimated disparity vector of interview motion vector prediction to DoNBDV with an initial disparity vector converted from depth value 128 in depth coding. Table 1 and 2 shows the coding results of test 1 and 2, respectively. Both of the tests report no coding loss for coded view. The results of test 2 even shows 0.1% of coding gain for synthesized view.

Table 1	Experiment results of test 1

Table 2 Experiment results of test 2

# Working Draft Changes

Changes

- VPS Extension 2

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if ( layerId ! = 0 ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| depth_refinement_flag[ layerId ] | u(1) |
| log2_sub_pb_size_minus2[ layerId ] | ue(v) |
| if ( !VpsDepthFlag[ layerId ] ) { |  |
| iv_res_pred_flag[ layerId ] | u(1) |
| depth_refinement_flag[ layerId ] | u(1) |
| view_synthesis_pred_flag[ layerId ] | u(1) |
| } else { |  |
| mpi_flag[ layerId ] | u(1) |
| vps_depth_modes_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| vps_inter_sdc_flag[ layerId ] | u(1) |
| } |  |
| } |  |
| } |  |
| cp_precision | ue(v) |
| for( i = 0; i < NumViews; i++ ) { |  |
| cp_present_flag[ i ] | u(1) |
| if( cp_present_flag[ i ] ) { |  |
| cp_in_slice_segment_header_flag[ i ] | u(1) |
| if( !cp_in_slice_segment_header_flag[ i ] ) |  |
| for( j = 0; j < i; j++ ) { |  |
| vps_cp_scale[ i ][ j ] | se(v) |
| vps_cp_off[ i ][ j ] | se(v) |
| vps_cp_inv_scale_plus_scale[ i ][ j ] | se(v) |
| vps_cp_inv_off_plus_off[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

- H.8.5 General decoding process for coding units coded in inter prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause 8.6.1 is invoked with the luma location ( xCb, yCb ) as input.

The variable nCbSL is set equal to 1  <<  log2CbSize and the variable nCbSC is set equal to 1  <<  ( log2CbSize − 1 ).

The decoding process for coding units coded in inter prediction mode consists of following ordered steps:

  - When iv_mv_pred_flag[ nuh_layer_id ] is equal to 1, or iv_res_pred_flag[ nuh_layer_id ] is equal to 1 or view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1, following applies:

    - If DepthFlag is equal to 0 the The derivation process for disparity vectors as specified in subclause  is invoked with the luma locations ( xCb, yCb ), the coding block size nCbSL as the inputs.

    - Otherwise (DepthFlag is equal to 1), the derivation process for disparity vectors from neighouring depth samples as specified in subclause  is invoked with the luma locations ( xCb, yCb ), the coding block size nCbSL as the inputs.

  - The inter prediction process as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) and the luma coding block size log2CbSize as inputs, and the outputs are three arrays predSamplesL, predSamplesCb, and predSamplesCr.

- H.8.5.3.2.15 Derivation process for the shifted disparity merging candidate

This process is not invoked when DepthFlag is equal to 1.

- …

- The variable availableFlagIvDCShift is set equal to 0 and for i in the range of 0 to numMergeCand - 1, inclusive, the following applies:

  - The variable N is set equal to mergeCandList[ i ].

  - When availableFlagIvDCShift is equal to 0 and availableFlagN is equal to 1, the candidate N is not equal to IvMC or IvDC, and mergeCandIsVspFlag[ i ] is not equal to 0, predFlagL0N is equal to 1 and ViewIdx( RefPicList0[ refIdxL0N ] ) is not equal to ViewIdx, the following applies:

    - availableFlagIvDCShift is set equal to 1

    - predFlagLXIvDCShift is set equal to predFlagLXN, ( with X being replaced by 0 and 1 )

    - refIdxLXIvDCShift is set equal to refIdxLXN, ( with X being replaced by 0 and 1 )

    - mvL0IvDCShift[ 0 ] is set equal to mvL0N[ 0 ] + ( DepthFlag ? 1 : 4 )

    - mvL0IvDCShift[ 1 ] is set equal to ( view_synthesis_pred_flag[ nuh_layer_id ] ? 0 : mvL0N[ 1 ] )

    - mvL1IvDCShift = mvL1N

When availableFlagIvDCShift is equal to 0 and availableFlagIvDC is equal to 1, availableFlagIvDCShift is set to 1 and the following applies for X being 0 to 1, inclusive:

- predFlagLXIvDCShift is set equal to predFlagLXIvDC,

- refIdxLXIvDCShift is set equal to refIdxLXIvDC,

- mvLXIvDCShift[ 0 ] is set equal to mvL0IvDC[ 0 ] + ( DepthFlag ? 1 : 4 )

- mvLXIvDCShift[ 1 ] is set equal to mvL0IvDC[ 1 ]

- H.8.5.5 Derivation process for disparity vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

The flag availableDV is set equal to 0, and both components of the disparity vector mvDisp are set equal to 0.

            - …

When availableDV is equal to 0 for each N being A1 and B1, the following applies.

  - When availableDV is equal to 0 and availableFlagIvpMvN is equal to 1, the following applies:

      - mvDisp = ivpMvDispN		(Error: Reference source not found-1)

      - refViewIdx = refViewIdxN		(Error: Reference source not found-2)

      - availableDV = 1		(Error: Reference source not found-3)

When availableDV is equal to 0, refViewIdx is set equal to DefaultViewIdx, and mvDisp is set equal to ( 0, 0 ). The variable mvRefinedDisp is set equal to mvDisp.

If DepthFlag is equal to 1, the mvDisp is set equal to ( DepthToDisparityB[ 0 ][ 128 ], 0 ), and the refViewIdx is set equal to DefaultViewIdx.

When depth_refinement_flag[ nuh_layer_id ] is equal to 1, the following ordered steps apply:

  - The derivation process for a disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations xCb, yCb, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPSW equal to nCbS, the variable nPSH equal to nCbS,  and the variable splitFlag equal to 0 as the inputs, and the output is the array disparitySamples of size (nCbS)x(nCbS).

  - The horizontal component of the disparity vector mvRefinedDisp[ 0 ] is set equal to disparitySamples[ 0 ][ 0 ].

…

- H.8.5.6 Derivation process for disparity vectors from neighbouring depth samples

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block.

Let p be the array of constructed samples prior to the deblocking filter process.

The disparity vector mvDisp is set equal to ( 0, 0 ). and modified as specified in the following:

[Ed. (GT): The derivation process below is e.g. not compatible with tiles, since a reasonable availability check (as e.g. for intra sample prediction is not applied. ]

  - The variable avgDep is derived as follows:

    - If xCb is greater than 0 and yCb is greater than 0, the following applies:

        - avgDep =  ( 5 * p[ xCb − 1 ][ yCb − 1 ] + 5 * p[ xCb − 1 ][ yCb + nCbS − 1 ] 
	+ 6 * p[ xCb + nCbS − 1 ][ yCb −1 ] + 8 )  >>  4 )	(Error: Reference source not found-4)

    - Otherwise, if xCb is greater than 0 and yCb is equal to 0, the following applies:

        - avgDep = p[ xCb − 1 ][ yCb + nCbS − 1 ]		(Error: Reference source not found-5)

    - Otherwise, if xCb is equal to 0 and yCb is greater than 0, the following applies:

        - avgDep = p[ xCb + nCbS − 1 ][ yCb − 1 ]		(Error: Reference source not found-6)

    - Otherwise, (xCb is equal to 0 and yCb is equal to 0), the following applies:

        - avgDep = −1		(Error: Reference source not found-7)

  - When avgDep is not equal to −1, mvDisp[ 0 ] is modified as specified in the following:

mvDisp[ 0 ] = DepthToDisparityB[ 0 ][ avgDep ]

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xC..( xCb + nCbS − 1 ), y = yCb..( yCb + nCbS− 1 ):

  - MvRefinedDisp[ x ][ y ] = mvDisp		(Error: Reference source not found-8)

  - RefViewIdx[ x ][ y ] = 0		(Error: Reference source not found-9)

# Conclusion

In order to prevent the decoder pipeline from being stopped by deriving disparity vectors from neighboring reconstructed depth pixels, it is proposed to use DoNBDV with an initial disparity vector converted from depth value 128 for the interview motion vector prediction in depth coding in this contribution. The disparity vector derivation method which utilizes neighboring reconstructed depth pixels is replaced by the simplified DoNBDV. The experimental results reportedly show 0.1% coding gain for synthesized views.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- G. Tech, K. Wegner, Y. Chen, S. Yea, “Test Model 6 of 3D-HEVC and MV-HEVC,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1005, Oct., 2013.

- V. Thirumalai, et al, “Inter-view motion vector prediction for depth coding, “ Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E0133, Aug., 2013.