| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0048 |
| --- | --- |

| Title: | 3D-HEVC HLS: On inter-component reference pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Yi-Wen Chen, Jian-Liang Lin, Yu-Pao Tsai, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yulin.chang, yiwen.chen, jl.lin, yupao.tsai, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the depth oriented coding tools – depth oriented neighboring block disparity vector (DoNBDV), view synthesis prediction (VSP) and motion parameter inheritance (MPI) – all need to access a picture in other component (e.g. texture picture refers to depth picture) to derive a sample array or predictors. It is assumed that the reference pictures of the other component can be found and accessed by default. However, the reference picture of the other component is implicitly used in the 3D-HTM but not clearly defined in the working draft text. The availability and location of the reference picture of the other component are not described for 3D-HEVC. It is proposed to derive an inter-component reference picture set and use the inter-component reference picture set to indicate the availability and the location of the inter-component reference picture in this contribution.

# Introduction

The depth-oriented neighboring block disparity vector (DoNBDV) and view synthesis prediction (VSP) in texture coding [1] utilizes a depth reference picture to derive corresponding disparity vectors. The depth reference picture and the current texture picture are in different layers but the same access unit. The current texture picture implicitly uses the depth reference picture as a reference picture for disparity vector derivation, but the depth reference picture is not in the reference picture list of the current texture picture because they are different view components and in different views. There is no clear definition where the depth reference picture could be found and accessed.

The following yellow highlighted working draft text describes the restriction of the depth reference picture refDepPic while deriving the disparity sample array, which is used by DoNBDV and VSP to retrieve depth samples. However, it doesn’t define where to find the depth reference pictures. The same issue could also be sound in motion parameter inheritance (MPI) for depth coding.

- H.8.5.5.2 Derivation process for a disparity sample array

- Inputs to this process are:

- a luma location ( xP, yP ) relative to the top-left luma sample of the current picture,

- a disparity vector mvDisp,

- a view order index refViewIdx specifying a reference view,

- a view order index depthViewIdx specifying the view the depth should be derived from

- variables nPSW and nPSH specifying a width and a height, respectively

- a variable splitFlag.

Outputs of this process are:

- a (nPSW)x(nPSH) array disparitySamples of disparities values.

Let refDepPic the picture in the current access unit with ViewIdx( refDepPic ) equal to ViewIdx and DepthFlag( refDepPic ) equal to 1.

- …

# Proposed schemes

In this contribution, it is proposed to define an inter-component reference picture set RefPicSetIntercomX which is very similar to the inter-layer reference picture set RefPicSetInterLayerX defined in MV-HEVC[2]. The inter-component reference picture set is derived before decoding the first slice of each picture. It contains all the active reference pictures in other layers with the same access unit and different VpsDepthFlag, so the depth reference picture could be found in this inter-component reference picture set for texture coding, and a texture reference picture could be found in this inter-component reference picture set for depth coding.

The inter-component reference picture set 0 and inter-component reference picture set 1 are searched. If one of the reference picture in these two inter-component reference picture sets is with view order index equal to the given depthViewIdx, the reference picture is assigned as the depth reference picture to derive the disparity sample array. If the reference picture could not be found, a zero array is returned.

# Working Draft Changes

Changes

Changes on H.7.3.2.1.2 Video parameter set extension 2 syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if ( layerId ! = 0 ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| log2_sub_pb_size_minus2[ layerId ] | ue(v) |
| for ( j = 0; j < i; j++ ) |  |
| intercom_dependency_flag[ i ][ j ] | u(1) |
| if ( !VpsDepthFlag[ layerId ] ) { |  |
| iv_res_pred_flag[ layerId ] | u(1) |
| depth_refinement_flag[ layerId ] | u(1) |
| … |  |

intercom_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not an inter-component reference layer for the layer with index i. intercom_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with index j may be an inter-component reference layer for the layer with index i. When intercom_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to MaxLayersMinus1, it is inferred to be equal to 0. The intercom_dependency_flag[ i ][ j ] is inferred as 0 when VpsDepthFlag[ layer_id_in_nuh[ i ] ] is equal to VpsDepthFlag[ layer_id_in_nuh[ j ] ].

The variables NumIntercomRefLayers[ i ] and IntercomRefLayerId[ i ][ j ] are derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	iNuhLId = layer_id_in_nuh[ i ]
	NumIntercomRefLayers[ iNuhLId ] = 0
	for( j = 0; j < i; j++ )
		if( intercom_dependency_flag[ i ][ j ] )
			IntercomRefLayerId[ iNuhLId ][ NumIntercomRefLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ]
}

Changes on H.7.3.6.1	General slice segment header syntax

| … |  |
| --- | --- |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if( nuh_layer_id > 0 && all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( nuh_layer_id > 0 && NumIntercomRefLayers[ nuh_layer_id ] > 0 ) { |  |
| intercom_pred_enabled_flag | u(1) |
| if( intercom_pred_enabled_flag && NumIntercomRefLayers[ nuh_layer_id ] > 1) { |  |
| for ( i = 0; i < NumIntercomRefLayers[ nuh_layer_id ]; i++ ) |  |
| intercom_pred_layer_idc[ i ] | u(v) |
| } |  |
| … |  |

- intercom_pred_enabled_flag shall be the same in all slice segment headers of a coded picture.

- intercom_pred_layer_idc[ i ] shall be the same in all slice segment headers of a coded picture for each possible value of  i.

- intercom_pred_enabled_flag equal to 1 specifies that inter-component prediction may be used in decoding of the current picture. intercom_pred_enabled_flag equal to 0 specifies that inter-component prediction is not used in decoding of the current picture.

intercom_pred_layer_idc[ i ] specifies the variable, IntercomRefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. The length of the syntax element intercom_pred_layer_idc[ i ] is Ceil( Log2( NumIntercomRefLayers[ nuh_layer_id ] ) ) bits. The value of intercom_pred_layer_idc[ i ] shall be in the range of 0 to NumIntercomRefLayers[ nuh_layer_id ] − 1, inclusive. When not present, the value of intercom_pred_layer_idc[ i ] is inferred to be equal to 0.

When i is greater than 0, intercom_pred_layer_idc[ i ] shall be greater than intercom_pred_layer_idc[ i − 1 ].

The variables RefPicLayerId[ i ] for all values of i in the range of 0 to NumIntercomRefLayers[ nuh_layer_id ] − 1, inclusive, are derived as follows:

for( i = 0, j = 0; i < NumIntercomRefLayers[ nuh_layer_id ]; i++)
	IntercomRefPicLayerId[ i ] = IntercomRefLayerId[ nuh_layer_id ][ intercom_pred_layer_idc[ i ] ]

- H.8.1.2 Decoding process for inter-component reference picture set

Outputs of this process are updated lists of inter-component pictures RefPicSetIntercom0 and RefPicSetIntercom1 and the variables NumActiveIntercomRefLayerPics0 and NumActiveIntercomRefLayerPics1.

The lists RefPicSetIntercom0 and RefPicSetIntercom1 are first emptied, NumActiveIntercomRefLayerPics0 and NumActiveIntercomRefLayerPics1 are set equal to 0 and the following applies:

for( i = 0; i < NumIntercomRefLayers[ nuh_layer_id ]; i++ ) {
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
										nuh_layer_id equal to IntercomRefPicLayerId[ i ] ) {
		if( ( ViewId[ nuh_layer_id ]  <=  ViewId[ 0 ]   &&
									ViewId[ nuh_layer_id ]  <=  ViewId[ IntercomRefPicLayerId[ i ] ] )  | |
			( ViewId[ nuh_layer_id ]  >=  ViewId[ 0 ]  &&
									ViewId[ nuh_layer_id ]  >=  ViewId[ IntercomRefPicLayerId[ i ] ] ) ) {
			RefPicSetIntercom0[ NumActiveIntercomRefLayerPics0 ] = picX
			RefPicSetIntercom0[ NumActiveIntercomRefLayerPics0++ ] is marked as "used for long-term reference"
		} else {
			RefPicSetIntercom1[ NumActiveIntercomRefLayerPics1 ] = picX
			RefPicSetIntercom1[ NumActiveIntercomRefLayerPics1++ ] is marked as "used for long-term reference"
		}
	} else
		RefPicSetIntercom0[ NumActiveIntercomRefLayerPics0++ ] = "no reference picture"
}

There shall be no entry equal to "no reference picture" in RefPicSetIntercom0 or RefPicSetIntercom1.

If the current picture is a RADL picture, there shall be no entry in RefPicSetIntercom0 or RefPicSetIntecom1 that is a RASL picture.

NOTE – An access unit may contain both RASL and RADL pictures.

- H.8.5.5.2 Derivation process for a disparity sample array

- Inputs to this process are:

- a luma location ( xP, yP ) relative to the top-left luma sample of the current picture,

- a disparity vector mvDisp,

- a view order index refViewIdx specifying a reference view,

- a view order index depthViewIdx specifying the view the depth should be derived from

- variables nPSW and nPSH specifying a width and a height, respectively

- a variable splitFlag.

Outputs of this process are:

- a (nPSW)x(nPSH) array disparitySamples of disparities values.

Let refDepPic the picture in the current access unit with ViewIdx( refDepPic ) equal to ViewIdx and DepthFlag( refDepPic ) equal to 1.

- For each X from 0 to 1, the following applies:

- for( i = 0; i < NumActiveIntercomRefLayerPicsX;i++ )

- {
		tempRefPic = RefPicSetIntercomX[ i ]

- if ( ViewIdx(tempRefPic) == depthViewIdx )

- {

- refDepPic = tempRefPic;

- break;

- }

- }

- If refDepPic cannot be found, then a (nPSW)x(nPSH) zero array for disparitySamples is returned.

Let refDepPels be an array of reconstructed depth samples refDepPic. The luma location (xTL, yTL) of top-left luma sample of a block in refDepPels is derived by

  - xTL = xP + ( ( mvDisp[ 0 ] + 2 )  >>  2 )		(Error: Reference source not found-1)

  - yTL = yP + ( ( mvDisp[ 1 ] + 2 )  >>  2 )		(Error: Reference source not found-2)

- The variables nSubBlkW and nSubBlkH are set equal to nPSW and nPSH, respectively.

- When splitFlag is equal to 1, nSubBlkW, nSubBlkH are modified as specified in the following:

- H.8.5.3.2.14 Derivation process for the texture merging candidate

This process is not invoked when mpi_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,

Outputs of this process are:

- a flag availableFlagT specifying whether the texture merging candidate is available,

- the prediction utilization flags predFlagL0T and predFlagL1T,

- the reference indices refIdxL0T and refIdxL1T (when availableFlagT is equal to 1),

- the motion vectors mvL0T and mvL1T (when  availableFlagT is equal to 1).

The variable availableFlagT is set equal to 0. The variables predFlagL0T and predFlagL1T are set equal to 0. The variables refIdxL0T and refIdxL1T are set equal to −1. Both components of the motion vectors mvL0T and mvL1T are set equal to 0.

The texture luma location ( xRef, yRef ) is derived by:

  - xRefFull = xPb + ( ( nPbW – 1 )  >>  1 )		(Error: Reference source not found-3)

  - yRefFull = yPb + ( ( nPbH – 1 )  >>  1 ) 		(Error: Reference source not found-4)

  - xRef = Clip3( 0, PicWidthInSamplesL – 1, ( xRefFull  >>  3 )  <<  3 )	(Error: Reference source not found-5)

  - yRef = Clip3( 0, PicHeightInSamplesL – 1,( yRefFull  >>  3 )  <<  3 )	(Error: Reference source not found-6)

- For each X from 0 to 1, the following applies:

- for( i = 0; i < NumActiveIntercomRefLayerPicsX;i++ )

  - {
	    tempRefPic = RefPicSetIntercomX[ i ]

- if ( ViewIdx( tempRefPic ) == ViewIdx( CurrPic ) )

- {

- textPic = tempRefPic;

- break;

- }

- }

- If textPic cannot be found, then availableFlagT equal to 0 is returned.

Let textPic be the picture with PicOrderCntVal and ViewIdx equal to PicOrderCnt and ViewIdx of the current picture and DepthFlag being equal to 0 and lLet textPU be the PU at covering the position ( xRef, yRef ) in textPic.

For X in the range of 0 to 1, inclusive, the following applies:

….

# Conclusion

In order to provide the depth oriented coding tools – DoNBDV, VSP and MPI with correct inter-component reference picture to derive the sample array or predictors, it is proposed to derive the inter-component reference picture sets and search in the inter-component reference picture sets for the available inter-component reference picture with given view order index. With correct inter-component reference picture, the decoder is able to locate the inter-component reference picture and retrieve correct sample array or predictors more clearly.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- G. Tech, K. Wegner, Y. Chen, S. Yea, “Test Model 6 of 3D-HEVC and MV-HEVC,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1005, Oct., 2013.

- G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 6,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1004, Oct., 2013.