| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0048 |
| --- | --- |

| Title: | 3D-HEVC HLS: On inter-component reference pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Yi-Wen Chen, Jian-Liang Lin, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yulin.chang, yiwen.chen, jl.lin, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the depth oriented coding tools – depth oriented neighboring block disparity vector (DoNBDV) and view synthesis prediction (VSP) for texture coding – all need to access a depth reference picture in other layers to derive a disparity vector. It is assumed that the depth reference pictures can be found and accessed by default. However, the depth reference picture is implicitly used in the 3D-HTM but not clearly defined in the working draft text. The availability and location of the depth reference picture are not described for texture coding. It is proposed to use the inter-layer reference picture set to indicate the availability and the location of the depth reference picture in this contribution.

# Introduction

The depth-oriented neighboring block disparity vector (DoNBDV) and view synthesis prediction (VSP) in texture coding [1] utilizes a depth reference picture to derive corresponding disparity vectors. The depth reference picture and the current texture picture are in different layers but the same access unit. The current texture picture implicitly uses the depth reference picture as a reference picture for disparity vector derivation, but the depth reference picture is not in the reference picture list of the current texture picture because they are different view components and in different views. There is no clear definition where the depth reference picture could be found and accessed.

The following yellow highlighted working draft text describes the restriction of the depth reference picture refDepPic while deriving the disparity sample array, which is used by DoNBDV and VSP to retrieve depth samples. However, it doesn’t define where to find the depth reference pictures.

- H.8.5.5.2 Derivation process for a disparity sample array

- Inputs to this process are:

- a luma location ( xP, yP ) relative to the top-left luma sample of the current picture,

- a disparity vector mvDisp,

- a view order index refViewIdx specifying a reference view,

- a view order index depthViewIdx specifying the view the depth should be derived from

- variables nPSW and nPSH specifying a width and a height, respectively

- a variable splitFlag.

Outputs of this process are:

- a (nPSW)x(nPSH) array disparitySamples of disparities values.

Let refDepPic the picture in the current access unit with ViewIdx( refDepPic ) equal to ViewIdx and DepthFlag( refDepPic ) equal to 1.

- …

# Proposed schemes

In this contribution, it is proposed to find the depth reference picture from the inter-layer reference picture set RefPicSetInterLayerX defined in MV-HEVC[2]. The inter-layer reference picture set is derived before decoding the first slice of each picture. It contains all the active reference pictures in other layers with the same access unit, so the depth reference picture could be found in this inter-layer reference picture set.

The inter-layer reference picture set 0 and inter-layer reference picture set 1 are searched. If one of the reference picture in these two inter-layer reference picture sets is a depth picture and with view order index equal to the given depthViewIdx, the reference picture is assigned as the depth reference picture to derive the disparity sample array. If the reference picture could not be found, a zero array is returned.

# Working Draft Changes

Changes

- H.8.5.5.2 Derivation process for a disparity sample array

- Inputs to this process are:

- a luma location ( xP, yP ) relative to the top-left luma sample of the current picture,

- a disparity vector mvDisp,

- a view order index refViewIdx specifying a reference view,

- a view order index depthViewIdx specifying the view the depth should be derived from

- variables nPSW and nPSH specifying a width and a height, respectively

- a variable splitFlag.

Outputs of this process are:

- a (nPSW)x(nPSH) array disparitySamples of disparities values.

Let refDepPic the picture in the current access unit with ViewIdx( refDepPic ) equal to ViewIdx and DepthFlag( refDepPic ) equal to 1.

- For each X from 0 to 1, the following applies:

- for( i = 0; i < NumActiveRefLayerPicsX;i++ )

- {
		tempRefPic = RefPicSetInterLayerX[ i ]

- if ( DepthFlag(tempRefPic) == 1 && ViewIdx(tempRefPic) == depthViewIdx )

- {

- refDepPic = tempRefPic;

- break;

- }

- }

- If refDepPic cannot be found, then a (nPSW)x(nPSH) zero array for disparitySamples is returned.

Let refDepPels be an array of reconstructed depth samples refDepPic. The luma location (xTL, yTL) of top-left luma sample of a block in refDepPels is derived by

  - xTL = xP + ( ( mvDisp[ 0 ] + 2 )  >>  2 )		(Error: Reference source not found-1)

  - yTL = yP + ( ( mvDisp[ 1 ] + 2 )  >>  2 )		(Error: Reference source not found-2)

- The variables nSubBlkW and nSubBlkH are set equal to nPSW and nPSH, respectively.

- When splitFlag is equal to 1, nSubBlkW, nSubBlkH are modified as specified in the following:

# Conclusion

In order to provide the depth oriented coding tools – DoNBDV and VSP with correct depth reference picture to derive the disparity sample array, it is proposed to search in the inter-layer reference picture sets for the available depth reference picture with given view order index. With correct depth reference picture, the decoder is able to locate the depth reference picture and retrieve correct disparity sample array more clearly.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- G. Tech, K. Wegner, Y. Chen, S. Yea, “Test Model 6 of 3D-HEVC and MV-HEVC,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1005, Oct., 2013.

- G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 6,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1004, Oct., 2013.