| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0051 |
| --- | --- |

| Title: | 3D-CE5 related: Removal of boundary filters for depth intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin1, Yi-Wen Chen1, Xianguo Zhang2, Yu-Wen Huang1, Shawmin Lei1 1MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 2MediaTek Beijing, Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, xianguo.zhang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In order to reduce the complexity of depth intra prediction, it is proposed to disable the boundary filter of DC mode and the gradient based boundary filter of vertical and horizontal intra prediction modes in depth picture coding. Experimental results reportedly show that removing the boundary filters for depth intra prediction causes no BD-rate increase (slight coding gain is achieved), while the complexity of depth intra coding could be reduced.

# Introduction

In 3D-HEVC [1], a boundary filter is applied on the DC intra prediction mode after the prediction samples are generated as the mean value of the neighboring reconstructed samples. The boundary prediction samples of DC mode will be smoothed with an [1,3] or [1,2,1] filter as shown in Fig. 1.

Fig. 1.  The boundary filter for DC mode

For vertical and horizontal intra prediction directions, a gradient based boundary filter is applied on the boundary prediction samples. Fig. 2 shows an example of the gradient based boundary filter for vertical intra prediction direction. For each prediction sample on the boundary, it will be updated as Pi = A0 + (Li-AL).

Fig. 2.  The gradient based boundary filter for vertical mode

# Proposed scheme

In order to simplify the intra prediction in depth coding, this contribution proposes to disable the boundary filter for DC mode and the gradient based boundary filter for vertical and horizontal modes in depth picture coding.

# Experimental results

The proposed simplifications are integrated into HTM-9.0r1 [2]. The tests are conducted under the common test conditions [3]. The results of disabling boundary filter of DC mode, gradient based boundary filter, and the combination are shown in Table 1, Table 2, and Table 3, respectively. The results of disabling boundary filter of DC mode, gradient based boundary filter, and the combination under the all-intra test conditions [4] are also shown in Table 4, Table 5, and Table 6, respectively. Compared to the anchor, HTM-9.0r1, removing the boundary filter for DC mode and the gradient based boundary filter for vertical and horizontal modes in depth intra prediction doesn’t cause any BD-rate increase while the complexity of depth intra coding could be reduced.

Table 1.  The results of disabling boundary filter of DC mode (CTC)

Table 2.  The results of disabling gradient based boundary filter (CTC)

Table 3.  The results of disabling both boundary filter of DC mode and gradient based boundary filter (CTC)

Table 4.  The results of disabling boundary filter of DC mode (All-Intra)

Table 5.  The results of disabling gradient based boundary filter (All-Intra)

Table 6.  The results of disabling both boundary filter of DC mode and gradient based boundary filter (All Intra)

# Conclusion

In this contribution, it is proposed to disable the boundary filter for DC mode and the gradient based boundary filter for vertical and horizontal modes in depth coding in order to reduce the complexity of depth intra prediction. The experimental results show no coding performance loss.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 2,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1001, Nov., 2013.

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Nov., 2013.

- H. Liu, “Description of Core Experiment 5 (CE5) on Depth Intra Modes,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1105, Nov., 2013.