| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0052 |
| --- | --- |

| Title: | 3D-CE5 related: Removal of neighbouring sample filtering for depth intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In 3D-HEVC, for the luma component, the neighbouring samples used for intra prediction sample generations need to be filtered before the generation process. This contribution proposes to disable the neighbouring samples filtering procedure for depth intra prediction in order to reduce the complexity of depth coding. The experimental results show that disabling the neighbouring samples filter for depth coding causes no coding efficiency loss.

# Introduction

In 3D-HEVC [1], for the luma component, the neighbouring samples used for intra prediction sample generations are filtered before the generation process. The filtering is controlled by the given intra prediction mode and transform block size as shown in Figure 1. If the intra prediction mode is DC or the transform block size is equal to 4x4, neighbouring samples are not filtered. If the distance between the given intra prediction mode and vertical mode (or horizontal mode) is larger than predefined threshold, the filtering process is enabled. The predefined threshold is specified in Table 1, where nT represents the transform block size.

Figure 1.  Neighbouring sample filtering

Table 1.  Specification of predefined threshold for various transform block sizes

|  | nT = 8 | nT = 16 | nT = 32 |
| --- | --- | --- | --- |
| Threshold | 7 | 1 | 0 |

A strong intra smoothing filter (bi-linear interpolation) as shown in Figure 2 is applied, when the transform block size is 32x32 and the following conditions are true.

- | (AL+BL) – 2 x ref[32] | < Threshold_left

- | (AL+AR) – 2 x ref[96] | < Threshold_above

Figure 2.  Bi-linear interpolation of reference samples using the corner samples

# Proposed scheme

In order to simplify the intra prediction in depth coding, this contribution proposes to disable the neighbouring samples filters including the [1,2,1] smoothing filter and the bi-linear interpolation filter in depth picture coding.

# Experimental results

The proposed simplifications are integrated into HTM-9.0r1 [2]. The tests are conducted under the common test conditions [3] and all-intra test conditions [4] and the results are shown in Table 2 and Table 3, respectively. Compared to the anchor, HTM-9.0r1, disabling the [1,2,1] smoothing filter and the bi-linear interpolation filter in depth intra prediction does not cause any BD-rate increase while the complexity of depth intra coding could be reduced.

Table 2.  The results of disabling neighbouring samples filters (CTC – retesting coding time)

Table 3.  The results of disabling neighbouring samples filters (All-Intra)

# Conclusion

In this contribution, it is proposed to disable the neighbouring samples filters including the [1,2,1] smoothing filter and the bi-linear interpolation filter in depth coding in order to reduce the complexity of depth intra prediction. The experimental results show no coding performance loss.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 2,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1001, Nov., 2013.

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Nov., 2013.

- H. Liu, “Description of Core Experiment 5 (CE5) on Depth Intra Modes,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1105, Nov., 2013.