| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0053 |
| --- | --- |

| Title: | 3D-HEVC HLS: On ARP reference picture signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Lin Chang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jl.lin, yiwen.chen, yulin.chang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In advanced residual prediction (ARP), the first temporal reference picture in the reference picture list is selected as the reference picture of the corresponding block in the reference view. A change of reference list reordering between slides in a same picture could cause the reference picture of the reference view to change. Frequently changing the reference picture for ARP will break the hardware pipeline design and causes a burden for decoders due to the DRAM access limitations. This contribution simply proposes to prohibit changes to the reference picture for ARP within a picture. An additional benefit of more flexibility in the reference picture selection for ARP is also provided.

# Introduction

In 3D-HEVC [1], the advanced residual prediction (ARP) is adopted to use the previously encoded residual information of adjacent views. The residual signal for current block (PU) can be predicted by the residual signal of a corresponding block in a reference picture as shown in Figure 1.

Figure 1.  Illustration of ARP in 3D-HEVC

In ARP, the first temporal reference picture in the reference picture list is selected as the reference picture of the corresponding block in the reference view. The motion information of the current PU are then scaled towards the selected reference picture in the reference view to generate the predicting residual signal by performing motion compensation as show in Figure 1.

Because the first temporal reference picture in the reference picture list is defined as the reference picture of the reference view, even a change of reference list reordering between slides in a same picture could cause the reference picture of the reference view to change. Frequently changing the reference picture of the reference view causes a burden for decoders due to the DRAM access limitations.

Optimizing the DRAM memory access is a critical issue in hardware design. The reference pictures of the reference views need to be stored in DRAM because the data of the reference pictures is too large to be stored in on-chip memory. In order to optimize the bandwidth efficiency of DRAM access, the decoders need to load the data of multiple LCUs in a single burst and fetch that data in advance in order to compensate the longer latencies caused by accessing data from DRAM. Allowing the reference picture of the reference view to be changed per slice will break the hardware pipeline design and could become the ultimate bottleneck of a decoder.

# Proposed solutions

This contribution simply proposes to prohibit changes to the reference picture of the reference view within a picture. Two solutions are proposed to avoid the problem mentioned above.

In solution 1, when ARP is enabled, it is proposed to add one new syntax element to the slice header to signal the reference picture for residual prediction. The new syntax element simply gives the reference picture index for ARP and is semantically restricted to always point to a same reference picture for all slices within a picture. For the comment test conditions in which the first temporal reference picture in the reference picture list is selected, only 1 bit needs to be coded, and therefore having negligible impact on the coding efficiency. Moreover, this syntax could also provide the flexible of reference picture selection for ARP, although that is not the primary motivation. The Modified syntax for the solution is illustrated in Table 1.

In solution 2, the reference picture of the reference view for ARP is derived at the encoder and decoder through an identical derivation process. Instead of selecting the first temporal reference picture in the reference picture list for each slice as the reference picture for ARP, solution 2 proposes to select the reference picture closest to current coding picture (has smallest POC distance) as the reference picture. With this simply modification, all slices within a picture will have a same reference picture for ARP and will not be affected by the reference picture reordering.

Table 1.  The modified syntax for the proposed solution 1.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| …. |  |
| if(iv_res_pred_flag[ nuh_layer_id ]){ |  |
| if (slice_type != I){ |  |
| ARP_ref_ l0_curr_view | ae(v) |
| if( slice_type = = B ) |  |
| ARP_ref_ l1_curr_view | ae(v) |
| } |  |
| } |  |
| …. |  |

ARP_ref_ lX_curr_view specifies the reference index of the reference picture used for residual prediction in list X. The value of ARP_ref_ lX_curr_view shall be in the range of 0 to num_ref_idx_lX_active_minus1, inclusive.

It is a requirement of bitstream conformance that the picture referred to by ARP_ref_ lX_curr_view shall be the same for all slices of a coded picture.

…

H.8.3.9 Derivation process for the target reference index for residual prediction

This process is invoked when the current slice is a P or B slice.

The variables RpRefIdxL0 and RpRefIdxL1 are set equal to –1, the variables RpRefPicAvailFlagL0 and RpRefPicAvailFlagL1 are set equal to 0.

The following applies for X in the range of 0 to 1, inclusive:

- When X is equal to 0 or the current slice is a B slice the following applies:

  - Whe n ARP_ref_ lX_curr_view is larger than or equal to zero, the following applies:

        - RpRefIdxLX = ARP_ref_ lX_curr_view		(Error: Reference source not found-1)

        - RpRefPicAvailFlagLX = 1		(Error: Reference source not found-2)

- The variable RpRefPicAvailFlag  is set equal to ( RpRefPicAvailFlagL0  | |  RpRefPicAvailFlagL1 ).

- When RpRefPicAvailFlag is equal to 1, the following applies for X in the range of 0 to 1, inclusive:

    - …

Table2.  The modified syntax for the proposed solution 2.

H.8.3.9 Derivation process for the target reference index for residual prediction

This process is invoked when the current slice is a P or B slice.

The variables RpRefIdxL0 and RpRefIdxL1 are set equal to –1, the variables RpRefPicAvailFlagL0 and RpRefPicAvailFlagL1 are set equal to 0.

The following applies for X in the range of 0 to 1, inclusive:

- When X is equal to 0 or the current slice is a B slice the following applies:

  - Set diffPOC equalt to 215 – 1

  - Set Idx = -1

  - For i in the range of 0 to num_ref_idx_lX_active_minus1, inclusive, the following applies:

    - If PicOrderCnt( RefPicListX[ i ] ) is not equal to PicOrderCntVal  and abs(PicOrderCnt( RefPicListX[ i ] ) – PicOrderCntVal) is less than diffPOC,  the following applies:

        - Idx = i	                                                         (Error: Reference source not found-3)

        - diffPOC = abs(PicOrderCnt( RefPicListX[ i ] ) – PicOrderCntVal)         (Error: Reference source not found-4)

    - Else If PicOrderCnt( RefPicListX[ i ] ) is not equal to PicOrderCntVal  and abs(PicOrderCnt( RefPicListX[ i ] ) – PicOrderCntVal) is equal to diffPOC,  the following applies:

      - When (X is equal to 0 and  PicOrderCnt( RefPicListX[ i ] ) is less than PicOrderCntVal) or (X is equal to 1 and  PicOrderCnt( RefPicListX[ i ] ) is larger than PicOrderCntVal),  the following applies:

        - Idx = i									(Error: Reference source not found-5)

        - diffPOC = abs(PicOrderCnt( RefPicListX[ i ] ) – PicOrderCntVal)		(Error: Reference source not found-6)

        - When Idx is larger than or equal to zero,  the following applies:

        - RpRefIdxLX = Idx		(Error: Reference source not found-7)

- RpRefPicAvailFlagLX = 1		(Error: Reference source not found-8)

- The variable RpRefPicAvailFlag  is set equal to ( RpRefPicAvailFlagL0  | |  RpRefPicAvailFlagL1 ).

- When RpRefPicAvailFlag is equal to 1, the following applies for X in the range of 0 to 1, inclusive:

- When X is equal to 0 or the current slice is a B slice the following applies:

    - …

# Experimental results

The proposed solution 2 is integrated into HTM-9.0r1 [2] and conducted under the common test conditions [3] to verify that it will not affect the results under the common test conditions. As shown in Table3, the coding results are identical to the anchor.

Table 3.  The simulation results of the proposed solution 2

# Conclusion

In this contribution, it is proposed to prohibit changes to the reference picture for ARP within a picture. Two solutions are proposed to avoid the frequent changes of the reference picture in the reference view.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 2,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1001, Nov., 2013.

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Nov., 2013.