| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San Jose, US, 11 – 17 Jan. 2014 | Document: JCT3V-G0054 |
| --- | --- |

| Title: | 3D-CE5 related: A simplified angular intra prediction for depth coding in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Jian-liang Lin2, Shawmin Lei2 1 MediaTek Beijing, Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, jl.lin, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a simplified angular intra prediction method for depth coding in 3D-HEVC. In the current 3D-HEVC, a linear filtering (intra interpolation) process is utilized for all directions in angular intra prediction. It is proposed to disable the intra interpolation in depth coding for all intra prediction directions in order to reduce the encoding and decoding complexity. The experimental results reportedly show that the proposed method causes no performance loss on CTC and 0.06% synthesis view BD-Rate gains on All Intra.

# Introduction

In angular intra prediction of HEVC and the current 3D-HEVC [1], there are multiple directional intra prediction modes. The relationship between variable intraPredAngle and the corresponding direction index is shown in Fig. 1 and Table 1. As shown in Fig. 2, each intra direction predicts the sample at position (x, y) of one Prediction Unit (PU) by

((32−iFact)×ref[x+iIdx+1]+iFact×ref[x+iIdx+2 ]+16) >> 5,

when the direction is relatively vertical (the direction index corresponding to the direction intraPredAngle is greater than or equal to 18). In this expression, the array ref is the decoded line buffer above the current PU, idx is the floor position of the pixel in ref which (x, y) points to along the intra direction (i.e., idx=((y+1) × intraPredAngle)>>5 ), and iFact is the distance between the floor poistion and pointed position (i.e., iFact =((y+1) × intraPredAngle)&31). Similar process works when the intra prediction direction is relatively horizontal (the direction index corresponding to variable intraPredAngle is smaller than 18). In that case, idx=((x+1) × intraPredAngle)>>5 and iFact =((x+1) × intraPredAngle)&31.

However, for the depth videos in 3D-HEVC, such an intra interpolation process may not be necessary, because the depth boundaries or gradients are usually important for the depth-dependent processes such as view synthesis prediction and rendering. Therefore, we propose to simplify the angular intra prediction for depth coding.

Fig. 1.  The commonly used intra prediction directions in 3D-HEVC and HEVC.

Fig. 2.  The sample prediction for position (x, y) under one relatively vertical direction, where intra interpolation is not disabled

Table 1.  Specification of intraPredAngle

| direction in Fig. 1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | −2 | −5 | −9 | −13 | −17 | −21 | −26 |
| direction in Fig. 1 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraPredAngle | −32 | −26 | −21 | −17 | −13 | −9 | −5 | −2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

# Proposed method

To address the problems above, it is proposed to simplify the intra interpolation by a nearest-position prediction for all intra prediction directions. In this nearest-position prediction, for each sample in the current intra prediction unit (PU), the predicted sample is set to the nearest sample of the pointed position along the intra prediction direction as shown in Fig. 3.

In order to analyze the complexity, Table 2 gives the contrast between our proposed method and the current intra prediction based on adds(ADD), Multiply(MUL), shift(SHI) and pixel read(PR) operations for one decoded intra prediction direction in the worst case. As we can see, the worst case for our method has lower computational complexity.

Table 2.  Number of operations for intra prediction direction in two methods

| Operations | ADD | MUL | SHI | PR |
| --- | --- | --- | --- | --- |
| Intra Interpolation in the Current 3D-HEVC | 3 | 2 | 1 | 2 |
| Proposed Method | 0 | 0 | 1 | 1 |

Fig. 3.  Proposed method, the nearest sample is selected as the predicted value

# Experimental results

The proposed method is implemented on the reference software HTM-9.0r1 [2] and the experiments are conducted under the common test conditions [3] and All Intra. Table 3 and 4 show the simulation results of the proposed method. The results on CTC show no BD-rate loss on the synthesis views and a slight BD-rate saving on video PSNR and video over total bitrate. The results on All Intra show there are 0.1% average BD-rate gains on the synthesis views and a slight BD-rate saving on video PSNR and video over total bitrate.

Table 3.  The results of the proposed method under CTC

Table 4.  The results of the proposed method for All Intra

# Conclusion

In this contribution, it is proposed to replace the intra interpolation by a nearest-position prediction in angular intra prediction process of depth coding for the encoding and decoding complexity reduction. Experimental results show the method has no BD-Rate loss on CTC and 0.1% gains on All Intra on the synthesis views.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S.Yea, “3D-HEVC Draft Text 2,” Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-F1100, Nov. 2013

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1

- D. Rusanovksy, A. Vetro, and K. Müller, "Common Test Conditions for 3D Video Extensions Development," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-F1100, Nov. 2013.