| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San Jose, US, 11 – 17 Jan. 2014 | Document: JCT3V-G0055 |
| --- | --- |

| Title: | 3D-CE2 related: A texture-partition-dependent depth partition for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Jian-liang Lin2, Shawmin Lei2 1MediaTek Beijing, Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2MediaTek Hsinchu, No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, jl.lin, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the depth quad-tree predictive coding restricts the possible PU partitions of a depth CU according to the PU partition of its collocated CU. In this contribution, the restrictions are modified as follows. Vertical/horizontal partitions are allowed for a depth CU only when the collocated texture CU is coded with vertical/horizontal or NxN partition. The experimental results reportedly show 0.2% BD-rate reduction on synthesis views without encoding and decoding time increase.

# Introduction

In 3D-HEVC [1], the quad-tree limitation (QTL) and depth quad-tree predictive coding (PC)Error: Reference source not found proposed in JCT3V-A0044 [2] and JCT3V-B0068 [3], respectively, were adopted to significantly reduce the encoding time and improve the coding performance for depth coding. With the quad-tree limitation, a coding depth CU cannot be split deeper than its collocated CU in the texture. For the depth quad-tree predictive coding, it reduces the PU partition candidates of current depth CU according to the PU partition of its collocated CU as shown in Figure 1. When the collocated CU is coded by 2Nx2N, 2NxN, or Nx2N partition, only the 2Nx2N partition is allowed for current depth CU. Please note that the AMP partitions were not included since the AMP had not been adopted into 3D-HEVC at that time when these two methods were adopted.

Later on, the AMP were enabled and directly classified into the 2Nx2N category for the quad-tree predictive coding as shown in Figure 2. With this strong restriction, 2Nx2N partition is the only allowed partition for a depth CU unless the collocated CU is coded in NxN partition.

Figure 1.  The depth quad-tree predictive coding proposed in JCT3V-B0086.

Figure 2.  The depth quad-tree predictive coding with AMP

# Proposed method

It is proposed to allow depth partitions based on the texture partition direction. For a depth CU, the vertical/horizontal partitions are allowed only when the collocated texture CU at the same quad-tree level is coded with the vertical/horizontal partitions or NxN partition as shown in Figure 3.

Figure 3.  Proposed method, where vertical/horizontal partitions for depth are allowed only if the collocated texture CU is coded with vertical/horizontal or NxN partition

# Experimental results

The proposed method is conducted on HTM-9.0r1 [4] under the common test conditions [5]. The experimental results are shown in Table 1. The proposed method achieves an average of 0.2% BD-rate reduction on the synthesis views without coding time difference.

Table 1.  The results of the proposed method under CTC

# Conclusion

In this contribution, vertical/horizontal partitions are allowed for a depth CU only when the collocated texture CU is coded with vertical/horizontal or NxN partition. The experimental results show the proposed achieves 0.2% BD-rate reduction on synthesis views.

# Patent rights declarations

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S.Yea, “3D-HEVC Draft Text 2,” Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-E1100, Nov., 2013

- J. Jung, K. Viswanathan, “Depth Quadtree Prediction for HTM,” Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-A0044, July, 2012

- J. Jung and E. Mora, “3D-CE3.h: Depth Quadtree Prediction for 3DHTM 4.1,” Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-B0068, Oct., 2012

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1

- D. Rusanovksyy, A. Vetro, and K. Müller, "Common Test Conditions for 3D Video Extensions Development," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-E1100, July, 2013.