| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0056 |
| --- | --- |

| Title: | 3D-CE5: Results on bin reduction for inter SDC residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, jl.lin, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of CE5 experiments related to the bin reduction for inter SDC residual coding. In the current 3D-HEVC, the residual coding of inter SDC modes may account for large coding bins. This is due to the fact that the binary codeword for the SDC residual magnitude could be very long. Moreover, the first 13 bins of the syntax for the SDC residual magnitude are context coded. To improve the throughput of the CABAC, the number of context coded bins should be reduced. This contribution proposes to constrain the residual magnitude of inter SDC within a predefined range so that the context coded bin number for coding the syntax element associated with the inter SDC residual could be significantly reduced. Experimental results reportedly show that constraining the inter SDC residual magnitude within a small range (e.g. “3” or “5”) causes no coding efficiency loss.

# Introduction

The inter Segment-wise DC coding (SDC) approach is an alternative residual coding mode in 3D-HEVC for inter prediction modes. The basic idea of inter SDC is to encode only one residual for each PU of non-skipped inter-coded depth CUs. For each PU, one residual value is signalled to the decoder. At the encoder side, the difference between average value of original signal and average value of prediction signal is usually used as the residual and is signalled to the decoder.

To code the residual for each inter SDC coded PU, a sign flag is transmitted along with a syntax element to indicate the absolute value of the residual. The decoder could reconstruct the residual value after receiving the sign flag and the absolute value of the residual. The binary codeword of the syntax element for the absolute value could be divided into prefix codeword and suffix codeword. The binarization for the prefix codeword is simply the truncated unary (TU) codeword assignment and the value 13 is viewed as an escape code. Any value larger than 13 will be appended with a suffix codeword generated by the ununsigned 0-th order Exp-Golomb (UEG0) binarization process. An example of binary codeword for the absolute residual value is demonstrated in Table 1. In current 3D-HEVC (HTM-9.0) [1], each bin of the prefix codeword of the absolute residual values is CABAC coded and the each bin of the suffix codeword is bypass coded.

Table 1.  The binary codeword for the residual magnitude for inter SDC mode

As shown in Table 1, the number of the bins for coding residual magnitude could be very long (larger than 100) and the prefix codeword bins are all context coded. In context-based adaptive binary arithmetic coding (CABAC), it is difficult to parallelize context coded bins. To improve the throughput of the CABAC, the number of context coded bins should be reduced.

# Proposed method

In this proposal, to reduce the number of the context coded bins with simple modifications, we propose to constrain the residual magnitude for inter SDC mode within a predefined range N (N is a non-negative integer). Since the absolute residual values are constrained within a range, the truncated unary (TU) binarization is employed for the syntax elements associated with the residual magnitude. Table 2 shows example codewords for the residual magnitude when N is set equal to 5. Note that, in our simplified scheme, only the first bin is context coded and the remaining bins are all bypass coded.

Table 2.  The TU binarization for the residual magnitude of inter SDC mode with N=5

# Experimental results

The proposed schemes are integrated into HTM-9.0r1 [2], and the tests are conducted under the common test conditions (CTC) [3]. The experimental results under CTC of N equals to “5” and “3” are shown in Table 3 and Table 4, respectively. As shown in the Tables, constraining the inter SDC residual magnitude within “5” and “3” under CTC introduces no coding efficiency loss under CTC. Compared to the anchor, the tested cases could significantly reduce the bin number for coding SDC residual magnitude as well as the number of context coded bins without performance drops.

Table 3.  The results of constraining inter SDC residual magnitude within N=5

Table 4.  The results of constraining inter SDC residual magnitude within N=3

# Conclusion

In this contribution, it is proposed to constrain the inter SDC residual magnitude within a small range to significantly reduce the number of the context coded bins for coding SDC residual. The results show that the proposed modifications bring almost no change to the coding performance by constraining the inter SDC residual within “5” or “3”.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 2,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1001, Oct., 2013.

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Oct., 2013.