| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0057 |
| --- | --- |

| Title: | 3D-AHG5: On complexity reduction of bi-prediction for advanced residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, jl.lin, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HTM-9.0, when the two pieces of motion information used for bi-prediction refer to a same reference block, the encoder and the decoder can non-normatively change the inter prediction direction from bi-prediction to uni-prediction to avoid the unnecessary computation caused by the redundant motion compensation process. However, changing the inter prediction PU with identical motion from bi-prediction to uni-prediction in current reference software, HTM-9.0, generates a bitstream which does not conform to the 3D-HEVC text due to the unequal operations between uni-prediction and bi-prediction in advanced residual prediction (ARP). To resolve this issue caused by ARP, it is proposed to align the operations between uni-prediction and bi-prediction for ARP. Experimental results reportedly show that the proposed method causes no coding efficiency loss while the unnecessary computation of bi-prediction can be removed. Additional experiments also show that the decoder could save 5.0% decoding time when the decoder could change the inter prediction PUs with identical list0/list1 motion from bi-prediction to uni-prediction to avoid the unnecessary computation.

# Introduction

Advanced residual prediction (ARP) is a technique to predict the residual signal for an inter prediction unit (PU). As shown in Fig. 1, in ARP, to ensure high correlation between residuals of two views, motion information of the current PU in the current picture of the current view is applied to the corresponding block in the reference picture of the reference view to generate the predicting residual signal by performing motion compensation based on the list0 and list1 motion information of the current PU.

In HEVC, when motion information of list0 is the same as that of list1, i.e., two motion vectors point to a same reference block, an HEVC encoder or decoder can non-normatively change the inter prediction direction from bi-prediction to uni-prediction in order to avoid the unnecessary computation caused by the redundant motion compensation process. In this proposal, we would like to call the operations as non-normative fast bi-prediction. However, the non-normative fast bi-prediction cannot be supported in the current 3D-HEVC when ARP is enabled due to the two reasons described in the next two paragraphs.

Fig. 1.  Illustration of ARP with bi-prediction and different motion vectors for the two lists

When the current PU is uni-prediction, clipping is applied after the predicting residual signal is added to the motion compensated prediction signal. However, no clipping is applied after the predicting residual signal is added to the motion compensated prediction signal when it is a bi-prediction PU.

Moreover, in ARP, the first temporal reference picture of each reference list in the reference view (i.e., V0) is selected as the reference picture of the corresponding block as shown in Fig. 1. The motion vectors of the current PU are then scaled towards the selected reference picture of the corresponding block in the reference view to generate the predicting residual signal by performing motion compensation. As shown in Fig. 2, when the current PU is bi-prediction with identical motion for list0 and list1, different predicting residual signals may be generated for list0 and for list1 due to different reference pictures for list0 and list1. In this example, the list1 MV is scaled to the reference picture in the opposite direction. It may result in degradation of the prediction performance by scaling a MV to the reference picture in the opposite direction when a reference picture in the same direction is available.

Fig. 2.  Illustration of ARP with bi-prediction and a same motion vector for the two lists in current 3D-HEVC

# Proposed method

We first propose to align the clipping operations between the uni-prediction and bi-prediction when ARP is enabled. Specifically, for both uni-prediction and bi-prediction, a clipping operation is applied on the motion compensated signal after adding the predicting residual signal.

To solve the issue caused by the reference picture selection for ARP, we propose a modified reference picture selection algorithm. In the proposed scheme, among the two reference pictures of the corresponding block for list0 and list1, the one closer to the reference picture of the current PU is selected as the reference picture of the corresponding block. As an example shown in Fig. 3, in the proposed scheme, when list0 motion and list1 motion are identical, they will both select the picture in view V0 at time T0 as the reference picture. With this modification of reference picture selection, we could solve the issue of supporting non-normative fast bi-prediction because the list0 and list1 MVs will be scaled to the same reference picture when they are identical. Moreover, the modified scheme could also improve the prediction efficiency since the MVs will be scaled to the reference picture in the same direction rather than scaled to the one in the opposite direction.

Fig. 3.  Illustration of ARP with bi-prediction and a same motion vector for the two lists in the proposed scheme

# Experimental results

The proposed scheme was already tested in previous meeting [1] and is conducted again on HTM-9.0r1 [2]. The tests are conducted under the common test conditions [3], and the anchor is HTM-9.0r1. As shown in Table 2, the proposed modification brings 0.05% bit-rate reduction while the non-normative fast bi-prediction can be supported for ARP coded CUs.

We also compare the decoding time of the decoder with the non-normative fast bi-prediction to the decoder without the non-normative fast bi-prediction. As shown in Table 3, the comparison results show that the decoder with the non-normative fast bi-prediction could achieve 5% decoding time reduction.

Table 2.  The results of the proposed modifications

Table 3.  The results of the comparison between the decoder with identical motion flexibility and the decoder without identical motion flexibility

# Conclusion

In this contribution, a scheme is proposed for advanced residual prediction to avoid unnecessary computation of bi-prediction when the two lists share the same motion information. The experimental results show no coding efficiency loss is introduced by the proposed scheme while the unnecessary bi-prediction could be avoided when ARP is enabled.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei, “3D-CE4 related: On complexity reduction of bi-prediction for advanced residual prediction,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F0112, Oct., 2013.

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Oct., 2013.