| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0058 |
| --- | --- |

| Title: | 3D-CE5 related: Conditional zero residual for segment-wise depth inter mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, Shawmin Lei (MediaTek) Jin Young Lee, Min Woo Park, and Chanyul Kim (Samsung) | Email: | {yiwen.chen, jl.lin,yuwen.huang, shawmin.lei}@mediatek.com jinyoung79.lee@samsung.com |
| Source: | MediaTek Inc. and Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Segment-wise depth inter mode coding was adopted at Vienna meeting and integrated into current 3D-HEVC. It encodes only one residual for each PU instead of encoding the residual using the conventional quadtree structure. As a follow-up of F0149, it is proposed to apply the segment-wise depth inter mode coding to only 2Nx2N in JCT3V-G0101. Also, in order to find more accurate residual value, the encoder is modified to check more candidates through RDO process. The results demonstrate that it achieves the coding gain of -0.46% in the synthesized view when five residual candidates are used. On the top of JCT3V-G0101, this contribution proposes a conditional zero residual signaling to signal the residual value more efficiently. The experimental results reportedly show that with the proposed conditional residual signaling, the performance could be improved to achieve the coding gain of -0.53% in the synthesized view while the encoding time could also be slightly reduced.

# Introduction

The inter Segment-wise DC coding (SDC) is an alternative residual coding mode in 3D-HEVC for inter prediction modes. The basic idea of inter SDC is to encode only one residual for each PU of non-skipped inter-coded depth CUs. For each PU, one residual value is signalled to the decoder. As shown in Fig. 1, the current method is used in all the PUs such as 2Nx2N, 2NxN, Nx2N, NxN, 2NxnU, 2NxnD, nLx2N, and nRx2N.

# Fig. 1. PUs for the segment-wise depth inter mode coding [1]

In last meeting, it was proposed to restrict the segment-wise coding into only 2Nx2N and modify the encoder to check three different candidates, which consist of DC, DC-N, and DC+N, and decides the best one through RDO in JCT3V- F0149 [1]. It was decided to be further studied in CE5.

# Proposed method

In the scheme proposed in [2](JCT3V-G0101), in which the inter SDC is only enabled for 2Nx2N CUs, the residual value is signaled in inter SDC mode starting from zero. However, it is found when inter SDC is enabled under Merge mode, the zero residual introduces exactly the same results as skip mode. But the signaling of Skip mode is more efficient than signaling the inter SDC under Merge mode with zero residual. In this contribution, therefore, we propose a conditional zero residual signaling for inter SDC. When inter SDC is enabled under Merge mode, the residual magnitude is signaled starting from 1 instead of starting from 0.

# Experimental results

The proposed modifications are integrated based on the software released by Samsung for CE5 tests. The anchor is HTM9.0r1 [3]. Table 1 show the results of CE5 experiment when five residual candidates are used [2](JCT3V-G0101) under CTC [4] which obtains about -0.46% gain in terms of the synthesized PSNR over the total bit rate. Table 2 shows the modification based on Samsung’s work. With the proposed simple modification in this contribution, the performance is improved to achieve about -0.53% gain in terms of the synthesized PSNR over the total bit rate. Table 3 and Table 4 show the results of the proposed modification based on Samsung’s work when three residual candidates and single candidate are used, respectively. With the proposed simple modification in this contribution, the performance is improved to achieve about -0.34% gain and about 0.01% loss, respectively. Note that, since the proposed modification does not need to signal zero residual for inter SDC under merge mode, we could skip some redundant computation for the encoder, which is reflected on the reduced encoding time.

Table 1.  The results of the CE5 tests when five residual candidates are used [2](JCT3V-G0101)

Table 3.  The results of the proposed modification based on [2] when five residual candidates are used.

Table 4.  The results of the proposed modification based on [2] when three residual candidates are used.

Table 5.  The results of the proposed modification based on [2] when one residual candidate is used.

# Conclusion

By integrating the proposed modification on the scheme proposed CE5 for inter SDC [2], the results show that restricting the residual magnitude for inter SDC Merge mode to be non-zero value could improve the coding efficiency while also reduces the encoder complexity.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Y. Lee, M. W. Park, C. Kim, “3D-CE5 related: Simplified depth inter mode coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F0149, Oct., 2013.

- J. Y. Lee, M. W. Park, C. Kim, “3D-CE5: Segment-wise depth inter mode coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G0101, Jan., 2014.

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Oct., 2013.