| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0059 |
| --- | --- |

| Title: | 3D-CE1 related: VSP combining inter-view ARP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, jl.lin, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Before the adoption of inter-view advanced residual prediction (ARP) in the last meeting (JCT3V-F0108/JCT3V-F0123 in the 6th Geneva meeting), view synthesis prediction (VSP) and ARP are mutually exclusive, allowing that combination is redundant. Because the adoption of inter-view residual prediction enables the possibility of combining VSP and ARP, this contribution presents the results of allowing VSP and ARP both enabled in one PU. Experimental results reportedly show that enabling inter-view ARP on VSP could bring 0.1% BD-rate reduction for the overall synthesis results. For the “UndoDancer” sequence, this combination could also achieve 1.3%, 1.1% and 0.3% BD-rate reduction on the video1, video2 and overall synthesis results.

# Introduction

When the advanced residual prediction was first proposed [1], it only supports the residual prediction for temporal inter prediction. Therefore, VSP merging candidates will not be inserted into the merge candidate list for a PU with ARP enabled since the combination of VSP and ARP is redundant. However, in the last meeting (the 6th Geneva meeting), a scheme of residual prediction for inter-view prediction was proposed by [2][3], therefore the combination of VSP and ARP becomes possible.

# Proposed method

In this proposal, we simply enable the combination of VSP and ARP. In the proposed scheme, VSP merging candidates could still be inserted into the merge candidate list for an ARP enabled PU. When a VSP candidate is selected and ARP is enabled for one PU, the VSP performs inter-view residual prediction on a sub-block basis.

# Experimental results

The proposed schemes are integrated into HTM-9.0r1 [4], and the tests are conducted under the common test conditions (CTC) [5]. Experimental results reportedly show that enabling inter-view ARP on VSP could bring 1.3%, 1.1% and 0.3% BD-rate reduction to sequence “UndoDancer”. For the overall synthesis results, the combination of VSP and ARP could also bring 0.1% BD-rate reduction.

Table 1.  The results of the proposed scheme of enabling VSP combining inter-view ARP

# Conclusion

In this contribution, it is proposed to enable the inter-view ARP on VSP. The results show that the proposed modification brings 0.1% overall BD-rate reduction. For sequence “UndoDancer”, the BD-rate reduction for video1, video2 and overall synthesis results could achieve 1.3%, 1.1% and 0.3%.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- L. Zhang, Y. Chen, X. Li, M. Karczewicz, “CE4: Advanced residual prediction for multiview coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D0177, April, 2013.

- K. Zhang, J. An, J.-L. Lin, Y.-L. Chang, S. Lei, “3D-CE4: Results on improved advanced residual prediction,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F0108, Oct, 2013.

- L. Zhang, Y. Chen, M. Karczewicz, “CE4: Further improvements on advanced residual prediction,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F123, Oct., 2013.

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Oct., 2013.