| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0060 |
| --- | --- |

| Title: | 3D-AHG8: Simplification on CABAC contexts for the syntax related to inter SDC, intra SDC and DMM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin, Tzu-Der Chuang, Yu-Lin Chang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, jl.lin, peter.chuang, yulin.chang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of simplifications on CABAC for the syntax related to inter SDC, intra SDC and DMM. The proposed simplifications first simplify the parsing process of the flags including depth_dc_flag, inter_sdc_flag and inter_sdc_resi_sign_flag by modifying them from context coding into bypass coding to reduce the number of context coded bins as well as the context models. As for the syntax of absolute residual or DC value (depth_dc_abs and inter_sdc_resi_abs_minus1), in current 3D-HEVC, the prefix codeword bins (13 bins for the worst cases) are context coded by sharing one context. In this contribution, we also propose to simplify the CABAC of depth_dc_abs and inter_sdc_resi_abs_minus1 by only context coding the first bin with single context and bypass coding the remaining bins. Experimental results reportedly show that the simplification on each syntax causes no coding efficiency loss.

# Introduction

The Segment-wise Depth Coding (SDC) is an alternative residual coding mode in 3D-HEVC. In current 3D-HEVC, SDC is employed in both inter-prediction and intra-prediction for depth coding.

For inter SDC, the basic idea is to encode only one residual for each PU of non-skipped inter-coded depth CU. For each PU, the difference between average value of original signal and average value of prediction signal is used as the residual for all pixels in the PU, and is signalled to the decoder.

As for an intra SDC coded depth CU, the depth block could be intra predicted by a Planar prediction mode (one segment) or depth modeling mode (2 segments) and only one residual value is coded for each segment within the CU.

Depth modeling mode (DMM) is an alternative prediction mode for depth coding in 3D-HEVC. Each PU is partitioned into two segments and a delta DV value is signaled for each segment to compensate the prediction error. In current 3D-HEVC, DMM and intra SDC share the same syntax to signal the delta DC and residual values.

Table 1 summarizes the parsing process and the number of context coded bins for the syntax related to inter SDC, intra SDC and DMM. To further improve the throughput of CABAC processing in 3D-HEVC by reducing the number of context coded bins, in this contribution, we propose to the reduce the context coded bin as well as reduce the number of context models for the coding of the syntax related to inter SDC, intra SDC and DMM.

Table 1.  The number of contexts and contexts coded bins for the syntax related to inter SDC, intra SDC and DMM in 3D-HEVC.

As shown in Table 1, the number of the bins for coding residual magnitude could be very long (larger than 100) and thus the residual coding of inter SDC will account for a high percentage of the overall context coded bins in this case. In context-based adaptive binary arithmetic coding (CABAC), it is difficult to parallelize context coded bins. To improve the throughput of the CABAC, the number of context coded bins should be reduced. Moreover, hardware is designed for the worst case, it is also important to reduce the worst case of the number of the context coded bins.

# Proposed Method

In this proposal, to reduce the number of the context coded bins, we mainly propose two simplifications.

Firstly, all the flags syntax including depth_dc_flag, inter_sdc_flag and inter_sdc_resi_sign_flag are bypass coded.

Secondly, only the first bin of the codeword for the syntax of depth_dc_abs and inter_sdc_resi_abs_minus1 is context coded and the remaining bins are bypass coded.

Table 2 summarizes the number of contexts and contexts coded bins for the syntax with these two proposed simplifications. As can be seen, the context coded bins for the worst case has been reduced from 29 to 2 and the context models is also reduced from 5 to 2.

Table 2. The number of contexts and contexts coded bins for the syntax related to inter SDC, intra SDC and DMM in the simplified scheme.

# Experimental results

The proposed schemes are integrated into HTM-9.0r1 [2], and the tests are conducted under the common test conditions (CTC) [3]. The anchor is HTM-9.0r1. The results for the simplifications on the flags depth_dc_flag, inter_sdc_flag and inter_sdc_resi_sign_flag are shown in Table 3, Table 4 and Table 5, respectively. The results for combining all simplifications on the three flags are demonstrated in Table 6. The results for the simplifications on the reisudal/DC syntax depth_dc_abs and inter_sdc_resi_abs_minus1 are shown in Table 7 and Table 8, respectively. Experimental results show that all the proposed simplifications introduce no coding efficiency loss under CTC. Compared to the anchor, the simplifications could significantly reduce the context coded bin for coding the inter SDC, intra SDC and DMM without performance drops.

Table 3.  The results of bypass coding of “depth_dc_flag”

Table 4.  The results of bypass coding of “inter_sdc_flag”

Table 5.  The results of bypass coding of “inter_sdc_resi_sign_flag”

Table 6.  The results of bypass coding of “depth_dc_flag”, “inter_sdc_flag” and “inter_sdc_resi_sign_flag”

Table 7.  The results of bypass coding of “depth_dc_abs”

Table 8.  The results of bypass coding of “inter_sdc_resi_abs_minus1”

# Conclusion

In this contribution, it is proposed to simplify the CABAC coding of the syntax related to inter SDC, intra SDC and DMM modes. The experimental results show that no coding loss is caused by any of the proposed simplification, while the number of contexts and contexts coded bins could be significantly reduced.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 2,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1001, Oct., 2013.

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Oct., 2013.