| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San Jose, US, 11th – 17th, Jan. 2014 | Document: JCT3V-G0061 |
| --- | --- |

| Title: | 3D-AHG8: Simplification on CABAC contexts for IC and ARP Flags |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen1, Kai Zhang2, Jicheng An2, Jian-Liang Lin1, Xianguo Zhang2, Shawmin Lei1 1 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 2 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, kai.zhang, jicheng.an, jl.lin, xianguo.zhang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, two neighboring blocks are accessed to select one of the three CABAC context models for ic_flag and iv_res_pred_weight_idx, and corresponding line buffers are needed to store the coded information of upper CTU row. In this contribution, it is proposed to simplify the CABAC for the two syntax elements. Experimental results reportedly show that the proposed simplifications do not cause any BD-rate increase in the common test condition.

# Introduction

In the current 3D-HEVC [1], two neighboring blocks are accessed to select one of the three CABAC context models for ic_flag and iv_res_pred_weight_idx. Take ic_flag for example. The CABAC context model index is derived as

ContexIdx(ic_flag) = ic_flag(A)+ic_flag(B),

where A and B are two neighboring blocks as depicted in Fig. 1. It is obvious that the ic_flag data in the upper CTU row must be stored in a line buffer to calculate the context model index.

Fig. 1.  Neighboring blocks used in the CABAC context model derivation.

# Proposed methods

## Simplified CABAC for ic_flag

It is proposed that only one context is used for the CABAC of ic_flag without referring to any information from neighboring blocks. In this way, the line buffer could be removed, and the number of context models used to code ic_flag could also be reduced from three to one.

## Simplified CABAC for iv_res_pred_weight_idx

It is proposed that only the left neighboring block A are accessed to derive the CABAC context models for iv_res_pred_weight_idx. In this way, the line buffer could be removed and the number of context models used to code iv_res_pred_weight_idx could also be reduced from three to two.

# Experimental results

The proposed simplifications are integrated on HTM-9.0r1 [2]. Table 1 and Table 2 show the results of the simplifications for ic_flag and iv_res_pred_weight_idx under CTC [3], respectively. It can be seen that the both the simplifications do not affect BD-rate performance noticeably.

Table 1.  Performance of the proposed solution 2 for ic_flag

Table 2.  Performance of the proposed solution 2 for iv_res_pred_weight_idx

# Conclusion

In this contribution, we propose to simplify the context model derivation for the CABAC of ic_flag and iv_res_pred_weight_idx. The proposed methods can remove three context models as well as remove the line buffers without causing coding efficiency loss for the synthesized views.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1001, Nov. 2013.

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Nov. 2013.