| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San Jose, US, 11th – 17th, Jan. 2014 | Document: JCT3V-G0063 |
| --- | --- |

| Title: | 3D-CE2: Results on additional merging candidates for depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Jian-Liang Lin2, Yu-Lin Chang2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, jl.lin, yulin.chang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of CE2 experiments related to the additional merging candidates for depth coding proposed in JCT3V-F0107. A disparity derived depth (DDD) candidate was proposed to derive a depth prediction value converted from the disparity vector of a collocated texture block. The derived depth value is used to represent all prediction samples of the current block when the proposed candidate is selected. Moreover, an additional texture merging candidate was also proposed to inherit the motion parameters from the below-right position of the collocated texture PU. The experimental results reportedly show that a combination of the two proposed methods can achieve 0.3% BD-rate saving for the synthesized views.

# Introduction

In the current 3D-HEVC [1], several additional merging candidates specially designed for dependent-view coding or depth coding are adopted, such as the backward view synthesis prediction (BVSP) candidate, the inter-view merging candidates, the disparity vector candidates, and the texture candidate.

BVSP utilizes samples in the depth component to derive disparity vectors for texture component coding. A disparity vector (DVx, 0) can be derived from its corresponding depth value (dep) by a linear relationship as

,

where w and b are two camera parameters. In 3D-HEVC, Equ. (1) is implemented in an integer form, so w and b can be transmitted from the encoder to the decoder as integers.

In addition, two inter-view merging candidates are adopted into the merging candidate list at the first position and the position before the temporal motion vector prediction (TMVP) candidate in texture coding. These two candidates inherit motion parameters from the center position and the below-right position of the collocated PU in the inter-view reference picture respectively as depicted in Fig. 1.

Similarly, a texture merging candidate is adopted into the merging candidate list at position 0 in depth coding, which inherits motion parameters from the center position of the collocated PU in the reference texture picture.

Fig. 1.  The center (green) and the below-right (red) position in the collocated PU.

# Proposed methods

## Disparity derived depth coding

A disparity derived depth (DDD) coding method is proposed to improve the coding efficiency for the depth component. It can be seen from Equ. (1) that the conversion between a depth value dep and DVx is reversible. Thus, a depth value can also be derived from its corresponding disparity vector (DVx, DVy) as follows.

The dep can be approximately calculated with operations of multiplication and shift to avoid division. When the collocated texture block of the current depth block is predicted by disparity-compensated prediction (DCP), a DDD candidate will be inserted into the merging candidate list right after the texture candidate. In a DDD candidate, all prediction samples in the current block are set to the approximated dep value. Fig. 2 shows the procedure to derive a depth value from its corresponding disparity vector. The DDD candidate is disabled for the base view since DCP is applied only on dependent views.

Fig. 2.  A depth value derived from its corresponding disparity vector

## An additional texture merging candidate

Similar to inter-view merging candidate, an additional texture merging candidate is proposed in depth coding. The additional candidate inherits motion parameters from the below-right position of the collocated PU in the reference texture picture as depicted in Fig. 1. It is inserted into the merging candidate list before TMVP after pruning with the first merging candidate.

# Experimental results

The proposed two methods are integrated on HTM-9.0r1 [2]. All the experiments are conducted under the common test condition [3]. Table 1 shows the results of the DDD candidate, which contributes 0.2% BD-rate saving for the synthesized views. Table 2 shows the results of the additional texture merging candidate, with 0.1% BD-rate saving for the synthesized views. Table 3 shows the results of a combination of these two candidates, with 0.3% BD-rate saving for the synthesized views.

Table 1.  Performance of the DDD candidate

Table2.  Performance of the additional texture candidate

Table 3.  Performance of the combination of two candidates

# Conclusion

In this contribution, a disparity derived depth (DDD) candidate and an additional texture merging candidate are proposed for depth coding. The DDD candidate utilizes the disparity vector in the collocated texture block to derive a depth prediction value, and the additional texture merging candidate inherits motion parameters from the below-right position of the collocated texture PU. Experimental results show that the combination of the two candidates achieves an average BD-rate saving of 0.3% for the synthesized views.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1001, Oct., 2013.

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Oct., 2013.