| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San Jose, US, 11th – 17th, Jan. 2014 | Document: JCT3V-G0064 |
| --- | --- |

| Title: | 3D-CE4: Results on improved advanced residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Jian-Liang Lin2, Yu-Lin Chang2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, jl.lin, yulin.chang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports results of JCT3V-F0108. An adaptive disparity vector (DV) derivation method for advanced residual prediction (ARP) is proposed. Three DV/MV candidates can be selected to find more accurate pseudo residues. Experimental results show that the proposed method can achieve 0.22% BD-rate saving for the synthesized views in the common test condition.

# Introduction

In the current 3D-HEVC [1], advanced residual prediction (ARP) [2] is adopted to replace the simple inter-view residual prediction for its better coding efficiency. In ARP, the base view is considered as a mirror of the current view. A mirror block is located with an estimated disparity vector (DV) and a mirror reference block is found for the mirror block with the same motion vector (MV) as the current block. Pseudo residues are generated by subtracting the reconstructed samples in the mirror reference block from those in the mirror block. Then pseudo residues are used to predict the real residues of the current block. Fig. 1 depicts a sketch of ARP. In ARP, a coding unit (CU) can choose to use pseudo residual prediction or not. If pseudo residual prediction is used, pseudo residues can be weighted by ½ or not. The encoder can find the best ARP choice for each CU according to the rate-distortion optimization (RDO) criterion, and signal the choice in the bitstream.

At the Geneva meeting, an advanced temporal residual prediction (ATRP) method [3][4] is adopted for the disparity compensated prediction. In ATRP, the disparity parameters of the current PU are applied to a mirror block located by an estimated MV in a temporal reference picture in the same view to generate the pseudo residual in temporal direction. The concept of proposed ATRP is illustrated in Fig. 2.

It plays a critical role to locate the mirror block since pseudo residues are similar to the real residues only if they are generated by similar samples. In ARP, the estimated DV is derived by a neighbouring block disparity vector (NBDV) procedure. The first available DV from a temporal or spatial neighbouring block is utilized by ARP as the estimated DV for the current block. In ATRP, the estimated MV is derived as the MV of the block which the current DV is pointing to in the reference view as illustrated in Fig. 3. Since such an estimated DV/MV may be not so accurate, pseudo residual prediction may be inefficient in some cases.

Fig. 1.  A sketch of ARP, where PR represents pseudo residue

Fig. 2.  The concept of the proposed ATRP

Fig. 3.  Derivation of estimated MV in ATRP

Fig. 4.  Aligned temporal DV and temporal DV

# Proposed method

An adaptive ARP method is proposed to improve ARP coding efficiency. Three DV/MV candidates are derived from temporal/spatial neighbouring blocks. A new candidate is appended into the list only if it is not equal to the first candidate in the list. If the candidate list is not fulfilled after exploiting neighbouring blocks, default ones will be added. An encoder can find the best DV/MV candidate used in ARP according to RDO criterion, and signal the index to the decoder. It should be noted that the proposed method only accesses temporal/spatial neighbouring blocks which are also accessed by the NBDV derivation process or the AMVP process.

For further improvement, aligned temporal DV (ATDV) is proposed as a new DV candidate. ATDV is obtained from the aligned block, which is located by a scaled MV to the collocated picture, as depicted in Fig. 4. Two collocated pictures are utilized, which are also used in the NBDV derivation. ATDV is inserted as the first DV candidate in the adaptive ARP candidate list when it is used.

# Experimental results

The proposed methods are integrated on HTM-9.0r1 [5]. Three experiments are conducted under the common test condition [6]. From Table 1 it can be observed that adaptive ARP can achieve 0.11% BD-rate saving for the synthesized views. The number goes up to 0.14% when ½ weighting is removed as depicted in Table 2. These results are reasonable since ½ weighting is not so useful when the estimated DV/MV is accurate enough. Furthermore, we can get 0.22% BD-rate saving for the synthesized views if ATDV is applied when ½ weighting is removed as depicted in Table 3.

Table 1.  Performance of adaptive ARP with ½ weighting

Table 2.  Performance of adaptive ARP without ½ weighting

Table 3.  Performance of adaptive ARP + ATDV without ½ weighting

# Conclusion

In this proposal, adaptive ARP is proposed to improve ARP. Experimental results show that 0.22% BD-rate saving for the synthesized views can be achieved.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1001, Nov. 2013.

- L. Zhang, Y. Chen, X. Li, M. Karczewicz, “CE4: Advanced residual prediction for multiview coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-D0177, April, 2013.

- K. Zhang, J. An, J. Lin, Y. Chang and S. Lei, “3D-CE4: Results on improved advanced residual prediction,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F0108, Oct. 2013.

- L. Zhang, Y. Chen, M. Karczewicz, “CE4: Further improvements on advanced residual prediction,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F0123, Oct. 2013.

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Nov. 2013.