| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0065 |
| --- | --- |

| Title: | CE3 : AMVP candidate list construction for DCP blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Taesup Kim Junghak Nam Sehoon Yea 19, Yangjae-daero 11gil Seocho-gu, Seoul South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Working draft

##### H.8.5.3.2.5 	Derivation process for texture luma motion vector prediction

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

- a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

- the reference index of the current prediction unit partition refIdxLX, with X being 0 or 1,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Output of this process is the prediction mvpLX of the motion vector mvLX, with X being 0 or 1.

The motion vector predictor mvpLX is derived in the following ordered steps:

- The derivation process for motion vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, refIdxLX, with X being 0 or 1, and the partition index partIdx as inputs, and the availability flags availableFlagLXN and the motion vectors mvLXN, with N being replaced by A or B, as output.

- If both availableFlagLXA and availableFlagLXB are equal to 1 and mvLXA is not equal to mvLXB, availableFlagLXCol is set equal to 0. Otherwise, the derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and refIdxLX, with X being 0 or 1, as inputs, and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

- The motion vector predictor candidate list, mvpListLX, is constructed as follows:

i = 0
if( availableFlagLXA )
	mvpListLX[ i++ ] = mvLXA
if( availableFlagLXB )
	mvpListLX[ i++ ] = mvLXB		(8-140)
if( availableFlagLXCol )
	mvpListLX[ i++ ] = mvLXCol

- The motion vector predictor list is modified as follows:

    - When mvLXA and mvLXB have the same value, mvLXB is removed from the list and the variable numMvpCandLX is set equal to the number of elements within the mvpListLX.

    - When numMvpCandLX is less than 2, the following applies:

      - When DiffViewId(CurrPic, RefPicListX[refIdxLX]) is not equal to 0 and availableDV is equal to 1, the following applies:

mvpListLX[ numMvpCandLX ] = mvRefinedDisp[xCb][yCb]	(8-141)

numMvpCandLX = numMvpCandLX + 1	(8-142)

      - The following applies repeatedly until numMvpCandLX is equal to 2:

mvpListLX[ numMvpCandLX ][ 0 ] = 0	(8-143-)

mvpListLX[ numMvpCandLX ][ 1 ] = 0	(8-144-)

numMvpCandLX = numMvpCandLX + 1	(8-145-)

    - When numMvpCandLX is greater than 2, all motion vector predictor candidates mvpListLX[ idx ] with idx greater than 1 are removed from the list.

- The motion vector of mvpListLX[ mvp_lX_flag[ xPb ][ yPb ] ] is assigned to mvpLX.

##### H. 8.5.3.2.6	Derivation process for depth luma motion vector prediction

The specifications in subclause 8.5.3.2.5 apply.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).