| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San Jose, US, 11th – 17th, Jan. 2014 | Document: JCT3V-G0066 |
| --- | --- |

| Title: | Syntax redundancy removal for illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Jian-Liang Lin2, Yu-Lin Chang2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, Xianguo.zhang, jl.lin, yulin.chang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, there is syntax redundancy for illumination compensation (IC) in merge blocks. It is proposed to remove such redundancy without imposing parsing dependency. Experimental results reportedly show that the proposed method achieve 0.1% BD-rate saving for the synthesized views in the common test condition.

# Introduction

In the current 3D-HEVC [1], illumination compensation (IC) only takes effects to disparity compensation prediction (DCP). However, an ic_flag is always signaled when the merge index is nonzero. Although the parsing dependency in merge blocks is removed and the parsing throughput can be improved, a redundant ic_flag is sent when the selected merging candidate does not employ DCP.

# Proposed method

It is proposed to remove the syntax redundancy by modifying the merging candidate construction process as follows. In the merging candidate construction process, when an ic_flag equal to 1 is received, a candidate that does not employ DCP is considered as unavailable. When an ic_flag equal to 0 is received, the original merging candidate construction applies. With this proposed method, there is no parsing dependency problem or syntax redundancy.

# Experimental results

The proposed methods are integrated on HTM-9.0r1 [3].	Table 1 shows that the proposed method achieved 0.1% BD-rate saving for the synthesized view.

Table 1.  Performance of the proposed method (CTC)

# Conclusion

In this proposal, it is proposed to remove the syntax redundancy by modifying the merging candidate construction process. The proposed method achieves 0.1% BD-rate saving for the synthesized view without increasing parsing dependency.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1001, Nov. 2013.

- HTM-9.0r1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Nov. 2013.