| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San Jose, US, 11 – 17 Nov. 2013 | Document: JCT3V-G0067 |
| --- | --- |

| Title: | Availability checking of temporal inter-view MV candidate and VSP candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An1, Yi-Wen Chen2, Kai Zhang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, yiwen.chen kai.zhang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution fixes a problem of using an unavailable reference view. It is proposed to check the availability of temporal inter-view MV candidate and VSP candidate according to the existence of inter-view reference picture. When there is no inter-view picture in the current reference lists, the temporal inter-view MV candidate and VSP candidate should not be inserted into the merge candidate list.

# Introduction

When there is no inter-view picture in current reference lists, the reference view of derived DV is unavailable, and in this situation the temporal inter-view MV candidate (including sub-PU temporal inter-view MV candidate and shifted temporal inter-view MV candidate) and VSP candidate should not be used anymore.

However, in the current 3D-HEVC draft text [1] and software HTM9.0r1, the temporal inter-view MV candidate and VSP candidate are still used even if there is no inter-view reference picture in the current reference lists, which will result in an unpredictable behavior because an improper reference view index value of derived DV is used.

# Proposed method

It is proposed to disable the temporal inter-view MV candidate and VSP candidate, that is to say, the two kinds of candidates cannot be inserted into the merge candidate list, when there is no inter-view picture in the current reference lists.

When there is no inter-view picture in current reference lists, the reference view index of derived DV will be set to -1 in the current draft text therefore that can be used to identify the case.

The draft text can be modified as follows, where the added sentences are highlighted.

        - Derivation process for a temporal inter-view motion vector candidate

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,

- a prediction list indication X,

- a reference view index refViewIdx.

- a disparity vector mvDisp,

Outputs of this process are:

- a flag availableFlagLXInterView specifying whether the temporal inter-view motion vector candidate is available,

- a temporal inter-view motion vector candidate mvLXInterView,

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX,

The flag availableFlagLXInterView is set equal to 0, the variable refIdxLX is set equal to –1, and both components of mvLXInterView are set equal to 0.

When the refViewIdx is equal to -1, the whole decoding process specified in this subclause terminates.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this subclause terminates.

The reference layer luma location ( xRef, yRef ) is derived by

  - xRefFull = xPb + ( nPbW  >>  1 ) + ( ( mvDisp[ 0 ] + 2 )  >>  2 )	(Error: Reference source not found-1)

  - yRefFull = yPb + ( nPbH  >>  1 ) + ( ( mvDisp[ 1 ] + 2 )  >>  2 )	(Error: Reference source not found-2)

  - xRef = Clip3( 0, PicWidthInSamplesL – 1, (  xRefFull  >>  3 )  <<  3 )	(Error: Reference source not found-3)

  - yRef = Clip3( 0, PicHeightInSamplesL – 1, (  yRefFull  >>  3 )  <<  3 )	(Error: Reference source not found-4)

The variable ivRefPic is set equal to the picture with ViewIdx equal to refViewIdx in the current access unit.

The variable ivRefPb specifies the luma prediction block covering the location given by ( xRef, yRef ) inside the inter-view reference picture specified by ivRefPic.

The luma location ( xIvRefPb, yIvRefPb ) is set equal to the top-left sample of the inter-view reference luma prediction block specified by ivRefPb relative to the top-left luma sample of the inter-view reference picture specified by ivRefPic.

When ivRefPb is not coded in an intra prediction mode, the following applies, for Y in the range of X to (1 – X), inclusive:

- The variables refPicListLYIvRef, predFlagLYIvRef[ x ][ y ], mvLYIvRef[ x ][ y ], and refIdxLYIvRef[ x ][ y ] are set equal to the corresponding variables of the inter-view reference picture specified by ivRefPic, RefPicListLY,, PredFlagLY[ x ][ y ], MvLY[ x ][ y ], and RefIdxLY[ x ][ y ], respectively.

- When predFlagLYIvRef[ xIvRefPb ][ yIvRefPb ] is equal to 1, the following applies for each i from 0 to num_ref_idx_lX_active_minus1, inclusive:

  - When PicOrderCnt( refPicListLYIvRef[ refIdxLYIvRef[ xIvRefPb ][ yIvRefPb ] ]) is equal to PicOrderCnt( RefPicListLX[ i ] ) and availableFlagLXInterView is equal to 0, the following applies.

      - availableFlagLXInterView = 1 		(Error: Reference source not found-5)

      - mvLXInterView = mvLYIvRef[ xIvRefPb ][ yIvRefPb ]	(Error: Reference source not found-6)

      - refIdxLX = i		(Error: Reference source not found-7)

        - Derivation process for a view synthesis prediction merge candidate

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

Outputs of this process are

- the availability flag availableFlagVSP whether the VSP merge candidate is available,

- the reference indices refIdxL0VSP and refIdxL1VSP ,

- the prediction list utilization flags predFlagL0VSP and predFlagL1VSP,

- the motion vectors mvL0VSP and mvL1VSP.

- The variable availableFlagVSP is set equal to 1, the variables predFlagL0VSP and predFlagL1VSP are set equal to 0, the variables refIdxL0VSP and refIdxL1VSP are set equal to −1 and the variable refViewAvailableFlag is set equal to 0.

- When RefViewIdx[ xCb ][ yCb ] is equal to -1, the variable availableFlagVSP is set equal to 0 and the whole decoding process specified in this subclause terminates.

- For X in the range of 0 to 1, inclusive, the following applies:

  - For i in the range of 0 to NumRefPicsLX – 1, inclusive, the following applies:

    - When refViewAvailableFlag is equal to 0 and ViewIdx( RefPicListX[ i ] ) is equal to RefViewIdx[ xCb ][ yCb ], the following applies:

        - refViewAvailableFlag = 1		(Error: Reference source not found-8)

        - predFlagLXVSP = 1		(Error: Reference source not found-9)

        - mvLXVSP = MvDisp[ xCb ][ yCb ]		(Error: Reference source not found-10)

        - refIdxLXVSP = i		(Error: Reference source not found-11)

        - Y = 1 – X		(Error: Reference source not found-12)

When the current slice is a B slice and refViewAvailableFlag is equal to 1, refViewAvailableFlag is set equal to 0 and the following applies:

  - For i in the range of 0 to NumRefPicsLY – 1, inclusive, the following applies.

    - When refViewAvailableFlag is equal to 0 and ViewIdx( RefPicListY[ i ] ) is not equal to RefViewIdx[ xCb ][ yCb ] and ViewIdx( RefPicListY[ i ] ) is not equal to ViewIdx, the following applies:

      - The variables refViewAvailableFlag, predFlagLYVSP, mvLYVSP, and refIdxLYVSP are derived as specified in the following:

        - refViewAvailableFlag = 1		(Error: Reference source not found-13)

        - predFlagLYVSP = 1		(Error: Reference source not found-14)

        - mvLYVSP =  MvDisp[ xCb ][ yCb ]		(Error: Reference source not found-15)

        - refIdxLYVSP = i		(Error: Reference source not found-16)

      - When iv_mv_scaling_flag is equal to 1, mvLYVSP is modified as specified in the following:

        - td = Clip3( −128, 127, ViewId – view_id_val[ RefViewIdx[ xCb ][ yCb ] ) ]	(Error: Reference source not found-17)

        - tb = Clip3( −128, 127, ViewId – ViewId( RefPicListY[ i ] ) )	(Error: Reference source not found-18)

        - tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(Error: Reference source not found-19)

        - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(Error: Reference source not found-20)

- mvLYVSP[ 0 ] = Clip3( −32768, 32767, Sign2( distScaleFactor * mvLYVSP[ 0 ] ) 
	* ( (Abs( distScaleFactor * mvLYVSP[ 0 ] ) + 127 )  >>  8 ) )	(Error: Reference source not found-21)

        - mvLYVSP[ 1 ] = Clip3( −32768, 32767, Sign2( distScaleFactor * mvLYVSP[ 1 ] ) 
	* ( (Abs( distScaleFactor * mvLYVSP[ 1 ] ) + 127 )  >>  8 ) )	(Error: Reference source not found-22)

# Experimental results

The proposed fix has been implemented on top of HTM9.0r1, and the experiments are conducted following the common test conditions [2]. The experimental results shown in Table 1 are exactly the same with the anchor because the inter-view reference picture are always exists in the reference lists in current test condition.

Table 1.  Coding performance of the proposed fix

# Conclusion

This contribution proposed to check the availability of temporal inter-view MV candidate and VSP candidate according to the existence of inter-view reference picture. When there is no inter-view picture in the current reference lists, the temporal inter-view MV candidate and VSP candidate should not be inserted into the merge candidate list. The proposed method fixed the problem of using an unavailable reference view.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, and S.Yea, “3D-HEVC Draft Text 2,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1001, Nov., 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Nov., 2013.