| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San Jose, US, 11 – 17 Nov. 2013 | Document: JCT3V-G0068 |
| --- | --- |

| Title: | 3D-CE1 related: Deblocking for sub-PU edge |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An1, Kai Zhang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, kai.zhang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

A method of deblocking the sub-PU edge is proposed to reduce the artifacts resulting from the sub-PU based processing including the sub-PU inter-view motion prediction and VSP. It is proposed to set the sub-PU edge as PU edge instead of TU edge for deblocking. 0.6%-0.7% BD-rate saving for video1 and video2 and 0.2% BD-rate saving for synthesized views are achieved.

# Introduction

In the current 3D-HEVC, the sub-PU temporal inter-view motion vector candidate and VSP merge candidate are both processed at sub-PU level, i.e., the PU is divided into multiple sub-PUs and each sub-PU has its own motion parameters for motion compensation. However, the deblocking process is still the same as HEVC: only PU and TU edges can be deblocked, while the sub-PU edges are not considered.

In JCT3V-F0135 [1], the sub-PU edge is proposed to be set to TU edge for deblocking. In deblocking, it is needed to check the existence of any non-zero transform coefficient of the two blocks of a TU edge to reduce the artifacts caused by the block based transform. However, the sub-PU edge is not TU edge and does not have the transform artifacts, so setting the sub-PU edge as TU edge is not reasonable.

# Proposed method

In light of the previously described problem, it is proposed to set the sub-PU edge to PU edge for deblocking because the sub-PU edge and PU edge have the same feature for deblocking, i.e., the two blocks of one edge may have different motion vectors.

The current VSP mode stored the motion vectors at PU level. However, these motion vectors are different from those which are actually utilized on generating prediction signals at sub-PU level. JCT3V-F0176 [2] proposed to store the finally used motion vectors for each sub-PU. In this contribution, the motion vector storage for VSP is the same as that in JCT3V-F0176.

# Experimental results

The proposed sub-PU deblocking has been implemented on top of HTM9.0r1, and the experiments are conducted following the common test conditions [3]. The experimental results are shown in Table 1.

Table 1.  Coding performance of the proposed sub-PU deblocking

# Conclusion

A method of deblocking the sub-PU edge is proposed to reduce the artifacts arising from the sub-PU based processing including the sub-PU inter-view motion prediction and VSP. It is proposed to set the sub-PU edge to PU edge instead of TU edge for deblocking. 0.6%-0.7% BD-rate saving for video1 and video2 and 0.2% BD-rate saving for synthesized views are achieved.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- H. Liu, Y. Chen, G. V.D.Auwera, L. Zhang, “CE3 related: HEVC compatible de-blocking for sub-PUs,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F0135, Nov., 2013.

- S. Shimizu, S. Sugimoto, “CE1-related: Motion parameters stored at VSP-coded blocks,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F0176, Nov., 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Nov., 2013