| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San Jose, US, 11 – 17 Nov. 2013 | Document: JCT3V-G0069 |
| --- | --- |

| Title: | 3D-CE1 related: Restricted bi-prediction for sub-PU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An1, Jian-Liang Lin2, Kai Zhang1, Yi-Wen Chen2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, jl.lin, kai.zhang, yiwen.chen, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HEVC, in order to reduce the memory bandwidth in motion compensation, the bi-prediction in small PU sizes (8x4, 4x8) has been disabled by only performing the motion compensation for list 0. However, in 3D-HEVC, the sub-PUs generated by sub-PU inter-view motion prediction and view synthesized prediction could still have bi-prediction motion compensation. This contribution proposes to disable the bi-prediction for sub-PU whose block size is smaller than 8x8. The experimental results reportedly shows that the proposed restricted bi-prediction for sub-PU cause no coding performance loss (0.02% BD-rate saving for synthesized views).

# Introduction

In HEVC, in order to reduce the memory bandwidth in motion compensation, the bi-prediction in small PU sizes (8x4, 4x8) has been disabled by only performing the motion compensation for list 0. However, the sub-PU related methods such as sub-PU level inter-view motion prediction (SPIVMP) and view synthesized prediction (VSP) in the current 3D-HEVC miss the restriction. Therefore, the worst case memory bandwidth could be unacceptable.

# Proposed method

In light of the previously described problem, it is proposed to disable the bi-prediction for sub-PU whose block size is smaller than 8x8.

In current 3D-HEVC, the SPIVMP divides one 16x4/4x16 PU into two 8x4/4x8 sub-PUs under common test condition and the VSP mode always divides one PU into sub-PUs with size equal to 8x4 or 4x8. In order to reduce the memory bandwidth and align with HEVC, it is proposed to avoid bi-prediction for sub-PU with size smaller than 8x8 by disabling the prediction in list1.

# Experimental results

The proposed restriction has been implemented on top of HTM9.0r1, and the experiments are conducted under the common test conditions [1]. The experimental results are shown in Table 1.

Table 1.  Coding performance of the proposed restriction

# Conclusion

This contribution proposes to disable the bi-prediction for sub-PU whose block size is smaller than 8x8 by only performing the motion compensation for list 0. The experimental results show that the proposed restricted bi-prediction for sub-PU brings no coding performance loss (0.02% BD-rate gain for synthesized views).

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Nov., 2013.