| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0071 |
| --- | --- |

| Title: | The removal of quad-tree limitation for depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jin Heo Sehoon Yea 19 Yangjae-daero 11gil Seocho-gu, Seoul South Korea | Tel: Email: | +82-2-6912-6477 sunmi.yoo@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In this proposal, the removal of quad-tree limitation (QTL) for depth map is proposed. QTL prohibits further quad-tree split since the depth map is generally homogenous so that the depth of quad-tree is hardly bigger than the corresponding texture block[1]. However, the experimental result shows that QTL degrades the quality of synthesized sequences because the depth map quality can deteriorate by limitation of further block split. As a result, QTL causes 1.2% of loss in average for the synthesized sequences.

# Problem statement

In 3D-HEVC standardization, QTL for depth has been adopted and accelerated the encoding time by restriction of further block split. When QTL was adopted it is reported that it decreases the encoding complexity with 0.4% of loss from synthesized result. However, as the standardization progresses and variety of coding tools for depth quality enhancement were adopted, QTL does not show any coding and synthesized efficiency.

# Experimental results

The performance evaluation for the proposed method has been conducted under CTC, according to JCT3V-F1100[2]. The experimental result for the proposed method is shown in Error: Reference source not found. Even though the encoding complexity increases, no limitation of quad-tree depth for depth maps achieves 1.2% of coding gain from synthesized view.

Table 1 The proposed method (QTL off) vs. Anchor (HTM-9.0r1)

# Conclusion

In this proposal, QTL for depth maps is turned off and experimented. Since not to use QTL achieved 1.2% of coding gain from synthesized view, we recommend QTL to be excluded for the next version of 3D-HEVC.

# Reference

- J. Jung, E. Mora, “3D-CE3.h: Depth Quadtree Prediction for 3DHTM 4.1,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-B0068, Shanghai, CN, Sep. 2012.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F1100, Geneva, CH, Oct. 2013.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).