| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0072 |
| --- | --- |

| Title: | 3D-CE4: Results on IC and ARP Flags Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents the results of the signaling of the illumination compensation (IC) flag and the residual prediction (RP) weighting factor. In the current 3D-HEVC, IC and RP are rarely performed simultaneously. Therefore, the signaling of these syntax elements needs to be improved. In order to resolve this problem, it is proposed two methods; one is the method for disabling IC when RP is enabled and the other is the method for disabling RP when IC is enabled. In case of disabling IC when RP is enabled, it reportedly provides 0.06% and 0.04% bit-savings for coded and synthesized views, respectively, with 97.7% encoding time. On the other hand, in case of disabling RP when IC is enabled, it reportedly provides 0.04% and 0.03% bit-savings for coded and synthesized views, respectively, with 97.9% encoding time.

# Introduction

In the current 3D-HEVC [1], the illumination compensation (IC) and the residual prediction (RP) are rarely performed simultaneously. Both tools can be performed at the same time only when the current coding unit (CU) is coded as the bi-predictive prediction mode from both temporal and view reference pictures. Therefore, the signaling of these two syntax elements (ic_flag and iv_res_pred_weight_idx) needs to be optimized by removing a redundant signaling.

# Proposed Methods

In this contribution, in order to resolve this problem, it is proposed two methods; the method 1 is for disabling IC when RP is enabled and the method 2 is for disabling RP when IC is enabled. Since disabling the illumination compensation when the residual prediction is enabled has the same meaning to disabling the residual prediction when the illumination compensation, it is needed to choose the method which can have the best coding performance.

# Experimental Results

The simulation was carried out on the common test condition [2] and was based on HTM 9.0r1 reference software.

## Method 1: Disabling of IC when RP is enabled

As can be seen in Table 1, the method for disabling IC when RP is enabled provides –0.06% and –0.04% bit-savings for coded and synthesized views, respectively, with 97.7% encoding time.

Table 1. Comparison between CTC anchor and method for disabling of IC when RP is enabled

## Method 2: Disabling of RP when IC is enabled

As can be seen in Table 2, the method for disabling RP when IC is enabled provides –0.04% and –0.03% bit-savings for coded and synthesized views, respectively, with 97.9% encoding time.

Table 2. Comparison between CTC anchor and method for disabling of RP when IC is enabled

From the experimental results, the method 1 has slightly better performance than the method 2.

# Conclusion

In this contribution, it is proposed to improve the signaling of the illumination flag and the residual prediction weight factor. The method for disabling IC when RP is enabled provides –0.06% and –0.04% bit-savings for coded and synthesized views, respectively, with 97.7% encoding time. Therefore, it is recommended to adopt the proposed method 1 into next 3D-HEVC Working Draft.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 2,” JCT3V-F1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Oct. 2013, Geneva, CH.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-F1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Oct. 2013, Geneva, CH.