| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0073 |
| --- | --- |

| Title: | 3D-CE3 related: Simplification of DV Derivation and Default DV Improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Byeongdoo Choi Yongjin Cho Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com b.d.choi@samsung.com yongjin9.cho@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current NBDV derivation, if these is no disparity vector from spatial and temporal neighboring blocks searching, then DV-MCP process is additionally performed to find the disparity vector used in the spatial MCP neighboring blocks. However, DV-MCP process needs the additional memory storage and relatively large number of operations. Therefore, it is proposed to remove DV-MCP process from the NBDV derivation. Additionally, it is also proposed to change the default disparity vector to improve coding performance. When the camera parameters are available, the default disparity vector can be set to the disparity vector corresponding to the depth value 128, which means the middle value of the depth value range. And when the camera parameters are unavailable, the default disparity vector can be set to (–128, 0) or (128, 0) according to the view direction. With the proposed method, the comparison operation can be reportedly reduced by 58% and the additional memory storage can be completely removed from the NBDV derivation. The proposed method reportedly provides no coding loss (0.01% bit-saving for synthesized views in case of CTC, and 0.02% bit-saving for synthesized views in case of non-CTC).

# Introduction

In the current NBDV derivation [1], if there are no DCP (disparity compensation prediction) temporal or spatial neighboring blocks, then the DC-MCP process is additionally performed to find the disparity vector used in the spatial MCP (motion compensation prediction) neighboring blocks. However, DC-MCP process needs additional memory storage to keep disparity vectors and their availability flags of the spatial neighboring blocks, and relatively large number of operations, especially comparisons.

The current NBDV derivation can be illustrated as follows.

- A variable availableDV, which indicates whether DV is found or not, is set to 0.

- For 2 temporal and 2 spatial neighboring blocks (co-located block in co-located picture and RAP picture, and A1 and B1 blocks), if availableDV is equal to 0, find DV from DCP neighboring block, and once DV is found availableDV is set to 1.

- For 2 spatial neighboring blocks (A1 and B1 blocks), if availableDV is equal to 0, find disparity vector from MCP neighboring block, and once DV is found availableDV is set to 1.

- If availableDV is equal to 0, DV is set to (0, 0) by default.

# Proposed Method

In order to simplify the NBDV derivation process, it is proposed to remove the DV-MCP process from the derivation process. Additionally, it is also proposed to change the default disparity vector to improve coding performance. When the camera parameters are available, the default disparity vector can be set to the disparity vector corresponding to the depth value 128, which means the middle value of the depth value range. And when the camera parameters are unavailable, the default disparity vector can be set to (–128, 0) or (128, 0) according to the view direction. And also the default disparity vector can be used for the residual prediction.

The proposed NBDV derivation can be illustrated as follows.

- A variable availableDV, which indicates whether DV is found or not, is set to 0.

- For 2 temporal and 2 spatial neighboring blocks (co-located block in co-located picture and RAP picture, and A1 and B1 blocks), if availableDV is equal to 0, find DV from DCP neighboring block, and once DV is found availableDV is set to 1.

- If availableDV is equal to 0, DV is set to the disparity vector corresponding to the depth value of 128 when the camera parameters are available, DV is set to (–128, 0) or (128, 0) according to the view direction when the camera parameters are unavailable, and availableDV is set to 1.

# Experimental Results

The simulation was carried out on the common test condition [2] and was based on HTM 9.0r1 reference software.

In CTC case, the proposed method provides no coding loss as shown in Table 1.

Table 1. Comparison between CTC anchor and proposed method

In non-CTC case (i.e. BVSP and DoNBDV off case), the proposed method also provides no coding loss as shown in Table 2.

Table 2. Comparison between non-CTC anchor and proposed method

# Conclusion

In this contribution, in order to simplify the current NBDV derivation process, it is proposed to remove the DV-MCP process, to change the default disparity vector value and to use the default disparity vector for the residual prediction. The proposed method can completely remove the addition memory storage for the NBDV derivation, can reduce the comparison operation by 58%, and provides no coding loss. Therefore, it is recommended to adopt the proposed method into next 3D-HEVC Working Draft.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 2,” JCT3V-F1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Oct. 2013, Geneva, CH.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-F1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Oct. 2013, Geneva, CH.