| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0074 |
| --- | --- |

| Title: | 3D-CE2 related: Simplification of DV Derivation for Depth Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the disparity vector for depth coding is derived using neighboring reconstructed pixel values. However, the current method has a potential problem with pipeline processing because of dependency with the neighboring blocks. Therefore, it is proposed to derive the disparity vector for depth coding only using the depth value 128, which means the middle value of the depth value range. With the proposed method, the dependency of the neighboring blocks can be completely removed. The proposed method reportedly provides no coding loss. Additionally, this contribution presents the results of proposed method with the removal of DV candidates from the depth merge list. This also reportedly provides no coding loss. And when shifted inter-view candidate is additionally removed, it reportedly provides 0.06% loss for synthesized views.

# Introduction

In the current 3D-HEVC [1], the disparity vector for depth coding is derived using neighboring reconstructed pixel as depicted in Figure 1. However, the DV derivation has a potential problem with pipeline processing since the pixel reconstruction in neighboring blocks is needed.

Figure 1. Neighboring reconstructed pixel used for the DV derivation

# Proposed Method

In this contribution, in order to solve the potential problem, it is proposed to derive the disparity vector for depth coding only using the depth value 128, which means the middle value of the depth value range.

Figure 2. Comparison between the current and proposed DV derivation

The current and proposed DV derivation can be illustrated as shown in Figure 2. The proposed method completely removes the dependency of neighboring blocks since the proposed method always uses the constant depth value 128.

# Experimental Results

The simulation was carried out on the common test condition [2] and was based on HTM 9.0r1 reference software.

## Simplification of DV Derivation

The performance of the proposed simplification method is shown in Table 1. The proposed method provides no coding loss. The simulation results show that the disparity vector corresponding to the depth value 128 is sufficient for the depth coding.

Table 1. Comparison between CTC anchor and proposed method

## Simplification of DV Derivation + Removal of DV candidates

This contribution also presents the results of the proposed method with the removal of DV candidates (disparity inter-view and two shifted DV candidates) from the depth merge list. The results of this are shown in Table 2. This also provides no coding loss. The simulation results show that DV candidates of the depth merge list does not give any coding benefit.

Table 2. Comparison between CTC anchor and proposed method with removal of DV candidates

## Simplification of DV Derivation + Removal of DV and Shifted Inter-View candidates

Additionally, this contribution also presents the results of the proposed method with the remove of DV and shifted temporal inter-view motion candidates from the depth merge list. The results of this are shown in Table 3. This gives 0.06% bit increase for synthesized views. But the coding benefit of the shifted inter-view candidate in the depth merge list is also marginal.

Table 3. Comparison between CTC anchor and proposed method with removal of DV and Shifted Inter-view candidates

# Conclusion

In this contribution, it is proposed to simply DV derivation for the depth coding. The proposed method completely removes the dependency of neighboring blocks with no coding loss. And the coding benefit of the DV and shifted temporal inter-view motion candidates is very marginal. Therefore, it is recommended to adopt the proposed method and removal of these candidates from depth merge list into next 3D-HEVC Working Draft.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 2,” JCT3V-F1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Oct. 2013, Geneva, CH.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-F1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Oct. 2013, Geneva, CH.