| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0077_WD1 |
| --- | --- |

| Title: | Proposed text of 3D-HEVC Working Draft for JCT3V-G0077 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document provides the proposed text of 3D-HEVC Working Draft. The changes are highlighted in green.

H.7.3.2.1.2  Video parameter set extension 2 syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if ( layerId ! = 0 ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| log2_sub_pb_size_minus3[ layerId ] | ue(v) |
| … |  |

H.7.4.3.1.2  Video parameter set extension 2 semantics

…

log2_sub_pb_size_minus3[ layerId ] specifies the value of the variable SubPbSize[ layerId ] that is used in the decoding of prediction units using the inter-view merge candidate. The value of log2_sub_pb_size_minus3 shall be in the range of (MinCbLog2SizeY – 3) to(CtbLog2SizeY – 3), inclusive.

The variable SubPbSize[ layerId ] is derived as specified in the following:

  - SubPbSize[ layerId ] = VpsDepthFlag( layerId ) ? CtbSizeY : 1  <<  ( log2_sub_pb_size_minus3[ layerId ]+  3 )	(Error: Reference source not found-1)

…

H.8.5.3.2.16  Derivation process for a sub prediction block temporal inter-view motion vector candidate

…

The variables nSbW and nSbH are derived as:

  - nSbW = SubPbSize[ nuh_layer_id ]	(Error: Reference source not found-2)

  - nSbH = SubPbSize[ nuh_layer_id ]	(Error: Reference source not found-3)

The variable ivRefPic is set equal to the picture with ViewIdx equal to refViewIdx in the current access unit, the variable curSubBlockIdx is set equal to 0 and the variable lastAvailableFlag is set equal to 0.

The vertical component of mvDisp is set equal to 0.

For yBlk in the range of 0 to ( nPbH / nSbH  + (nPbH % nSbH ? 1 : 0 ) ), inclusive, the following applies:

- If ((yBlk + 1) * nSbH) is greater than nPbH, then the variable nRealSbH is set equal to nPbH – (yBlk * nSbH), otherwise nRealSbH is set equal to nSbH.

- For xBlk in the range of 0 to ( nPbW / nSbW + ( nPbW % nSbW ? 1 : 0 ) ), inclusive, the following applies:

  - If ((xBlk + 1) * nSbW) is greater than nPbW, then the variable nRealSbW is set equal to nPbW – (xBlk * nSbW), otherwise nRealSbW is set equal to nSbW.

  - For X in the range of 0 to 1, inclusive, the derivation process for a temporal inter-view motion vector candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb + xBlk*nSbW, yPb + yBlk * nSbH ), the variables nRealSbW and nRealSbH, the prediction list indication X, the view order index refViewIdx, and the disparity vector mvDisp as the inputs and the outputs are the flag spPredFlagLX[ xBlk ][ yBlk ], the motion vector spMvLX[ xBlk ][ yBlk ] and the reference index spRefIdxLX[ xBlk ][ yBlk ].

…

H.8.5.3.3.9  Decoding process for sub prediction block wise inter sample prediction

…

The variables nSbW and nSbH are derived as:

  - nSbW = SubPbSize[ nuh_layer_id ]	(Error: Reference source not found-4)

  - nSbH = SubPbSize[ nuh_layer_id ]	(Error: Reference source not found-5)

For x in the range of 0 to ( nPbW / nSbW + ( nPbW % nSbW ? 1 : 0 ) ), inclusive, the following applies:

- If ((x + 1) * nSbW) is greater than nPbW, then the variable nRealSbW is set equal to nPbW – (x * nSbW), otherwise nRealSbW is set equal to nSbW.

- For y in the range of 0 to ( nPbH / nSbH + ( nPbH % nSbH ? 1 : 0 ) ), inclusive, the following applies:

  - If ((y + 1) * nSbH) is greater than nPbH, then the variable nRealSbH is set equal to nPbH – (y * nSbH), otherwise nRealSbH is set equal to nSbH.

  - The luma location ( xSb, ySb ) specifying the top-left sample of the current luma sub prediction block relative to the top-left sample of the current luma coding block is derived as specified in the following:

      - xSb = xBl + x * nSbW		(Error: Reference source not found-6)

      - ySb = yBl + y * nSbH		(Error: Reference source not found-7)

  - For X in the range of 0 to 1, inclusive, the variables mvLX, mvCLX, refIdxLX, and predFlagLX are derived as specified in the following:

      - mvLX = SubPbMvLX[ xSb ][ ySb ]		(Error: Reference source not found-8)

      - mvCLX = SubPbMvCLX[ xSb ][ ySb ]		(Error: Reference source not found-9)

      - refIdxLX = SubPbRefIdxLX[ xSb ][ ySb ]		(Error: Reference source not found-10)

      - predFlagLX = SubPbPredFlagLX[ xSb ][ ySb ]		(Error: Reference source not found-11)

  - The decoding process for inter sample prediction as specified in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the luma prediction block location ( xBl, yBl ) equal to ( xSb, ySb ), the luma coding block size block nCbS, the luma prediction block width nPbW equal to nRealSbW, the luma prediction block height nPbH equal to nRealSbH, the luma motion vectors mvL0 and mvL1, the chroma motion vectors mvCL0 and mvCL1, the reference indices refIdxL0 and refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1 as inputs, and the inter prediction samples (predSamples) that are an (nCbSL)x(nCbSL) array predSamplesL of prediction luma samples and two (nCbSC)x(nCbSC) arrays predSamplesCr and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr, as outputs.