| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0077 |
| --- | --- |

| Title: | 3D-CE1 related: MCP Size and DV for Sub-PU Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the Sub-PU prediction allows the motion compensation prediction (MCP) block sizes which are not in the HEVC base specification, such as 12x8 and 8x12. These MCP block sizes need to be avoided to have a design consistency with HEVC version 1. In order to solve this problem, this contribution proposes two methods; the method 1 is to use only three MCP sizes 8x8, 8x4 and 4x8, and the method 2 is to allow the Sub-PU prediction only to 2Nx2N partition so that MCP size for the Sub-PU prediction is always 8x8. These proposed methods reportedly provide no coding loss (method 1: 0.01% bit-saving for synthesized views, method 2: 0.02% bit-saving for synthesized views). And in order to get more accurate motion information, it is also proposed to set the vertical component of the disparity vector used for searching motion information in the reference view to ‘0’. This proposed method with method 2 reportedly provides 0.1% bit-saving for synthesized views. Additionally, it is also proposed to improve the syntax element for the Sub-PU size signaling.

# Introduction

In the current 3D-HEVC [1], the Sub-PU prediction allows the motion compensation prediction (MCP) block sizes which are not in the HEVC base specification, such as 12x8 and 8x12.

Figure 1. Current Sub-PU partition for AMP mode (CU size: 16x16)

Figure 1 shows examples of not aligned MCP block sizes. These MCP block sizes need to be avoided to have a design consistency with HEVC version 1. Therefore, this contribution proposes two methods in order to solve this problem. Additionally, this contribution proposes to improve the syntax element for the Sub-PU size signaling.

# Proposed Method

## Method 1: Using three MCP sizes (8x8, 8x4 and 4x8)

It is proposed to use only three MCP size, which is aligned with HEVC version 1; 8x8, 8x4 and 4x8. The current and proposed method for partitioning PU can be compared as shown in Figure 2.

Figure 2. Comparison between the current and proposed method for partitioning PU

## Method 2: Allowing Sub-PU Prediction Only to 2Nx2N Partition

It is proposed to apply the Sub-PU prediction only to 2Nx2N partition size. If the Sub-PU prediction is allowed only to 2Nx2N partition, the MCP block size for the Sub-PU prediction is always 8x8.

## Method 3: Setting vertical comp. of DV used for searching MV to ‘0’

It is observed that the more accurate motion information can be obtained if the vertical component of the disparity vector used for searching motion information in the reference view is set to ‘0’. This method is examined with the method 1 and method 2.

## Clean-up of Sub-PU size Syntax Element

The range of the syntax element log2_sub_pu_size_minus2[ layerId ], which indicates the size of the Sub-PU, is always from 4 to 64. However, 4x4 MCP size needs to be prevented to align with the HEVC base specification and the minimum size of the Sub-PU should be aligned with the minimum CU (coding unit) size, and the maximum size of the Sub-PU should be aligned with CTU (coding tree unit) size. Thus the semantics of the syntax element can be changed as follows. And this syntax element can be moved from VPS to SPS to get the size information of the minimum CU and CTU.

log2_sub_pb_size_minus3[ layerId ] specifies the value of the variable SubPbSize[ layerId ] that is used in the decoding of prediction units using the inter-view merge candidate. The value of log2_sub_pb_size_minus3 shall be in the range of (MinCbLog2SizeY – 3) to(CtbLog2SizeY – 3), inclusive. The variable SubPbSize[ layerId ] is derived as specified in the following: - SubPbSize[ layerId ] = VpsDepthFlag( layerId ) ? CtbSizeY : 1 << ( log2_sub_pb_size_minus3[ layerId ]+ 3 ) (Error: Reference source not found-1)

# Experimental Results

The simulation was carried out on the common test condition [2] and was based on HTM 9.0r1 reference software.

## Method 1: Using three MCP size (8x8, 8x4 and 4x8)

The performance of the proposed method 1 is shown in Table 1. The proposed method 1 provides no coding loss (0.01% bit-saving for synthesized views).

Table 1. Comparison between CTC anchor and method 1

The performance of the proposed method 1 with the proposed method 3 is shown in Table 2. The proposed method provides no coding loss (0.04% bit-saving for synthesized views).

Table 2. Comparison between CTC anchor and method 1 with method 3

## Method 2: Allowing Sub-PU Prediction Only to 2Nx2N Partition

The performance of the proposed method 2 is shown in Table 3. The proposed method 2 provides no coding loss (0.02% bit-saving for synthesized views).

Table 3. Comparison between CTC anchor and method 2

The performance of the proposed method 1 with the proposed method 3 is shown in Table 4. The proposed method provides 0.1% bit-saving for synthesized views.

Table 4. Comparison between CTC anchor and method 2 with method 3

# Conclusion

This contribution proposes several methods. Two methods are proposed in order to align the MCP size with the HEVC base specification, one method is proposed to get more accurate motion information, and the clean-up of the syntax element which indicates the size of the Sub-PU is proposed. The proposed method 2 with the method 3 provides 0.1% bit-saving for synthesized views. Therefore, it is recommended to adopt method 2 with method 3 and the syntax clean-up into next 3D-HEVC Working Draft.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 2,” JCT3V-F1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Oct. 2013, Geneva, CH.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-F1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Oct. 2013, Geneva, CH.