| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0078 |
| --- | --- |

| Title: | 3D-CE3 related: Adaptive Disabling Temporal Inter-View Candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Woo Park Jin Young Lee Chanyul Kim 129, Samsung-ro Yeongtong-gu, Suwon-si Gyeonggi-do 443-742, Korea | Email: | m.w.park@samsung.com jinyoung79.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, the illumination compensation is only applied to the coding unit which performs the inter-view prediction. If the temporal inter-view motion candidates which always perform the temporal prediction are used, the illumination compensation is never performed for the current coding unit (i.e. ic_flag is always set to 0). Thus, if the illumination compensation is enabled (i.e. ic_flag is equal to 1), the temporal inter-view motion candidates can be excluded from the merge list. Although the temporal inter-view motion candidate will be disabled, motion information of the temporal inter-view motion candidate can be used for pruning process. The proposed method reportedly provides 0.1% bit-saving for coded and synthesized views. Additionally, when the illumination compensation is enabled, it is proposed to allow a non-zero vertical component for the disparity inter-view motion candidates to improve coding efficiency.

# Introduction

In the current 3D-HEVC [1], the illumination compensation is only applied to the coding unit which performs the inter-view prediction. If the temporal inter-view motion candidates which always perform the temporal prediction are used, the illumination compensation is never performed for the current coding unit (i.e. ic_flag is always set to 0). Therefore, when the illumination compensation is enabled, the temporal inter-view motion candidates need to be disabled. In addition, the vertical component of the disparity vector of the disparity inter-view motion candidates is always set to ‘0’, but this can have a negative effect on the prediction efficiency of the illumination compensation.

# Proposed Method

## Method 1: Adaptive Disabling Temporal Inter-view Motion Candidates

In this contribution, it is proposed to disable the temporal inter-view motion candidates from the merge list when the illumination compensation is enabled.

Figure 1. Comparison between the current and proposed merge list construction

The current and proposed merge list construction can be illustrated as shown in Figure 1. According to the value of the syntax element ic_flag, the temporal inter-view motion candidates (“Temporal Inter-view Candidate” and “Temporal Inter-view Candidate (BR)”) can be adaptively disabled. And although the temporal inter-view motion candidate will be disabled, the motion information of the temporal inter-view motion candidate can be used for pruning process of the spatial candidates A1 and B1. Here, the pruning process can be performed by only comparing the reference indices of the motion information, but not motion vectors.

## Method 2: Allowing non-zero vertical comp. for Disparity Candidates

Additionally, when the illumination compensation is enabled, it is also proposed to allow the non-zero vertical component of the disparity vector for the disparity inter-view temporal motion candidate. This can enable to have better prediction for the illumination compensation.

# Experimental Results

The simulation was carried out on the common test condition [2] and was based on HTM 9.0r1 reference software.

## Method 1

The performance of the proposed method 1 is shown in Table 1. The proposed method 1 provides 0.1% bit-saving for coded and synthesized views (0.09% and 0.05% bit-saving for coded and synthesized views, respectively).

Table 1. Comparison between CTC anchor and proposed method 1

## Method 1 + Method 2

The performance of the proposed method 1 with method 2 is shown in Table 2. The proposed method provides 0.1% bit-saving for coded and synthesized views (0.13% and 0.08% bit-saving for coded and synthesized views, respectively). The simulation results show that method 2 gives additional coding gain to the method 1.

Table 2. Comparison between CTC anchor and proposed method 1 with method 2

# Conclusion

In this contribution, when the illumination compensation is enabled it is proposed to disable the temporal inter-view motion candidates and to allow non-zero vertical component for the disparity inter-view motion candidates to improve coding performance. The proposed methods provide 0.1% bit-saving for coded and synthesized views with minor changes in text and software. Therefore, it is recommended to adopt the proposed methods into next 3D-HEVC Working Draft.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 2,” JCT3V-F1001, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Oct. 2013, Geneva, CH.

[2] D. Rusanovskyy, K. Mueller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-F1100, Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Oct. 2013, Geneva, CH.