This attachment is the modifications on the working draft by the proposed Method 1 in contribution “JCT3V-G0082 CE3 related: Results on Limitation of the Range of NBDV”. The modification is based on “JCT3V-F1001 3D-HEVC Draft Text 2” and in green highlight.

    - Derivation process for disparity vectors

……

When availableDV is equal to 0, refViewIdx is set equal to DefaultViewIdx. When depth_refinement_flag[ nuh_layer_id ] is equal to 1, mvDisp is set equal to ( (DepthToDisparityB[ refViewIdx ][ 0 ] + DepthToDisparityB[ refViewIdx ][ 1 << BitDepthY - 1 ])>>1, 0 ). Otherwise (when depth_refinement_flag[ nuh_layer_id ] is equal to 0), mvDisp is set equal to (0,0). The variable mvRefinedDisp is set equal to mvDisp.

When availableDV is equal to 1, mvDisp[0] is set equal to clip3(Min(DepthToDisparityB[ refViewIdx ][ 0 ], DepthToDisparityB[ refViewIdx ][ 1 << BitDepthY - 1 ]), Max(DepthToDisparityB[ refViewIdx ][ 0 ], DepthToDisparityB[ refViewIdx ][ 1 << BitDepthY - 1 ]), mvDisp[0] ), mvDisp[1] is set equal to clip3(-1*CtbSizeY, CtbSizeY, mvDisp[1]).

When depth_refinement_flag[ nuh_layer_id ] is equal to 1, the following ordered steps apply:

  - The derivation process for a disparity sample array as specified in subclause Error: Reference source not found is invoked with the luma locations xCb, yCb, the disparity vector mvDisp, the view identifier refViewIdx, the variable nPSW equal to nCbS, the variable nPSH equal to nCbS,  and the variable splitFlag equal to 0 as the inputs, and the output is the array disparitySamples of size (nCbS)x(nCbS).

  - The horizontal component of the disparity vector mvRefinedDisp[ 0 ] is set equal to disparitySamples[ 0 ][ 0 ].

……