| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0082_r1 |
| --- | --- |

| Title: | CE3 related: Results on Limitation of the Range of NBDV |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Lu and Lu Yu Zhejiang University Zheda Road 38 Hangzhou, Zhejiang, China | Tel: Email: | +86 15968882677 yiqiu8932@gmail.com |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This contribution proposes to limit the range of NBDV according to the possible disparity range between the current texture view and the reference view. Two different methods are proposed in this proposal and both of them can reduce the cache miss when fetching the reference views and bring small coding gain.

# Problem Statement

In current 3D-HEVC, some major coding tools, such as Inter-view Motion Prediction (IVMP), Advanced Residual Prediction and View Synthesis Prediction (VSP), need a disparity vector which specifies a corresponding location in an inter-view reference picture, as shown in Fig.1.

Fig. 1. Disparity vector specifying a corresponding location in an inter-view reference picture

Neighboring Block Disparity Vector (NBDV) [1][2][3] is proposed to estimate such a disparity vector. Currently, NBDV is derived or each CU. First temporal DCP neighboring blocks are evaluated, followed by a check of the spatial DCP neighbors. Finally, MCP coded neighbor blocks are searched. Once a disparity motion vector is found, the whole disparity vector derivation process terminates. When no disparity motion vector is found from the neighboring blocks, a zero disparity vector is used.

The current design has the following problems:

(1) Disparity vector represents the distance of the same object in different views. While the disparity vector of DCP neighboring blocks is derived by motion estimation, which takes both rate and distortion under consideration. Therefore, the disparity vector of DCP neighboring blocks may not be accurate.

(2) The range of NBDV is as the same as MV in current design, which means all the area of the coded texture picture and the coded depth map of the reference view are possibly accessed when coding the units of current texture picture. However, the range of all the possible accurate DVs among two views is limited and is often much smaller than the image size. Therefore, the current design may request more bandwidth or on-chip memory than it really needs, especially for the coded depth map, which will only be accessed by NBDV.

# Proposed Method

To solve the above problem, we propose to limit the range of NBDV under the following considerations:

(1) The range of all the possible accurate DVs among two views is limited. Assuming the depth map of the current texture view contains all possible depth values, e.g. from 0 to 255, a maximum disparity and a minimum disparity can be calculated using the camera parameters of the current view and reference view. For example, the disparity range between the left view and the base view of Poznan_Street is [-80, -2] pixel.

(2) The disparity vectors derived by motion estimation are not very accurate, even fall out of the possible disparity range sometimes. For example, motion estimation can generate a disparity vector valued 20, while the disparity range is [-80, -2] for Poznan_Street. Such a disparity vector is obviously not accurate.

We proposed two different methods to limit the range of NBDV when depth_refinement_flag equals to 1

## Method 1

Method 1 can be described as follows:

(1) Assuming the depth map of the current texture view contains all possible depth values. Find a maximum disparity (DispMax) and a minimum disparity (DispMin) between the current view and reference view. Note that DispMax and DispMin can be calculated once per slice (even once per sequence if the camera parameters do not change).

(2) Derive a NBDV using current method.

(3) If a NBDV can be derived, clip the horizontal component of NBDV to [DispMin, DispMax] and clip the vertical component of NBDV to [-CtbSizeY, CtbSizeY], where CtbSizeY is the size of LCU.

(4) If no NBDV is derived, use the defualt DV with horizontal component equal to (DispMin+DispMax)>>1 and vertical component equal to 0.

Method 1 does not change the current NBDV derivation process. Only a clip operation is added after the NBDV is derived. The calculation of DispMax and DispMin can be easily done during the establishment of the disparity look-up table for every slice.

By applying Method 1, the range of NBDV and the possible accessed area in reference view can be limited into a rectangle area, as the green rectangle area shown in Fig. 2.

Fig. 2. The possible accessed area after the limitation.

Such a limitation brings two benefits:

(1) If the derived NBDV is out of the possible disparity range, it can be improved by the above clipping process. Therefore, the accuracy of NBDV can be improved.

(2) The possible accessed area in reference view is much smaller. Table 2-1 shows the size of the possible accessed area before and after the limitation for a LCU with size 64*64. It can be observes that only 1.9% of the original possible accessed area may be accessed after the limitation. Therefore, the cache miss when fetching the reference views can be reduced.

Table 2-1 Size of the possible accessed area before and after the limitation

|  | Before Limitation |  |  | After Limitation |  |  | Ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Width | Height | Size | Width | Height | Size |  |
| Balloons | 1024 | 768 | 786432 | 112 | 192 | 21504 | 2.7% |
| Kendo | 1024 | 768 | 786432 | 112 | 192 | 21504 | 2.7% |
| Newspaper_CC | 1024 | 768 | 786432 | 134 | 192 | 25728 | 3.3% |
| GT_Fly | 1920 | 1088 | 2088960 | 110 | 192 | 21120 | 1.0% |
| Poznan_Hall2 | 1920 | 1088 | 2088960 | 166 | 192 | 31872 | 1.5% |
| Poznan_Street | 1920 | 1088 | 2088960 | 143 | 192 | 27456 | 1.3% |
| Undo_Dancer | 1920 | 1088 | 2088960 | 144 | 192 | 27648 | 1.3% |
| Shark | 1920 | 1088 | 2088960 | 97 | 192 | 18624 | 0.9% |
| Average | 1584 | 968 | 1600512 | 127.25 | 192 | 24432 | 1.9% |

## Method 2

To further remove those inaccurate NBDVs, method 2 is developed and can be described as follows:

(1) Assuming the depth map of the current texture view contains all possible depth values. Find a maximum disparity (DispMax) and a minimum disparity (DispMin) between the current view and reference view. This is as the same as Method 1.

(2) Derive the NBDV. During this process, if a candidate DV is out of the range [DispMin, DispMax], it will be ignored and the derivation process continues, otherwise, clip the vertical component of this candidate DV to [-CtbSizeY, CtbSizeY] and use this candidate DV as the final NBDV, where CtbSizeY is the size of LCU.

(3) If no NBDV is derived, use the defualt DV with horizontal component equal to (DispMin+DispMax)>>1 and vertical component equal to 0.

Method 2 is similar to Method 1, the range of NBDV and the possible accessed area in reference view can also be limited into a same rectangle area. Method 2 also eliminates those candidate DVs that fall out of the disparity range, thus a more accurate NBDV may be found.

# Experimental Results

Both the two proposed methods are implemented into HTM 9.0r1 software, and proposed method is compared with HTM 9.0r1. The test condition is CTC.

Experimental results show that such a limitation will not harm the coding performance. Actually, the coding performance has slight gain (-0.03% for method 1and -0.04% for method 2). For video 1 and video 2, there is around 0.1% coding gain, both for method 1and method 2.

The encoding and decoding time are similar compared to anchor.

Based on the experimental results, we recommend adopting the Method 1 because this method is simpler and without degradation of coding efficiency compared to Method 2.

## Method 1

Table 3-1: Experimental results for proposed method 1

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.15% | -0.12% | -0.04% | -0.05% | -0.08% | 97.2% | 88.4% |
| Kendo | 0.00% | -0.22% | -0.18% | -0.08% | -0.06% | -0.07% | 95.0% | 101.6% |
| Newspaper_CC | 0.00% | -0.23% | -0.11% | -0.05% | -0.06% | -0.03% | 99.4% | 102.8% |
| GT_Fly | 0.00% | -0.14% | -0.09% | -0.02% | -0.02% | -0.02% | 102.2% | 91.3% |
| Poznan_Hall2 | 0.00% | 0.17% | -0.02% | 0.03% | 0.00% | 0.06% | 97.0% | 99.6% |
| Poznan_Street | 0.00% | -0.03% | -0.08% | -0.01% | -0.01% | 0.00% | 108.1% | 101.1% |
| Undo_Dancer | 0.00% | -0.10% | -0.03% | -0.02% | -0.02% | -0.09% | 101.5% | 97.5% |
| Shark | 0.00% | -0.06% | -0.23% | -0.03% | -0.04% | -0.03% | 102.5% | 103.6% |
| 1024x768 | 0.00% | -0.20% | -0.14% | -0.05% | -0.06% | -0.06% | 97.2% | 97.6% |
| 1920x1088 | 0.00% | -0.03% | -0.09% | -0.01% | -0.02% | -0.02% | 102.3% | 98.6% |
| average | 0.00% | -0.09% | -0.11% | -0.03% | -0.03% | -0.03% | 100.4% | 98.2% |

## Method 2

Table 3-2: BD rate results for proposed method 2

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.06% | -0.03% | -0.02% | -0.02% | -0.08% | 101.2% | 90.9% |
| Kendo | 0.00% | -0.28% | -0.22% | -0.09% | -0.07% | -0.06% | 102.2% | 100.7% |
| Newspaper_CC | 0.00% | -0.34% | -0.02% | -0.05% | -0.05% | -0.04% | 98.4% | 99.7% |
| GT_Fly | 0.00% | -0.08% | 0.03% | -0.01% | 0.00% | -0.05% | 101.6% | 93.4% |
| Poznan_Hall2 | 0.00% | -0.02% | -0.19% | -0.04% | -0.04% | -0.04% | 100.0% | 103.5% |
| Poznan_Street | 0.00% | -0.15% | -0.01% | -0.02% | -0.01% | 0.00% | 103.9% | 97.1% |
| Undo_Dancer | 0.00% | -0.06% | -0.09% | -0.02% | -0.02% | -0.03% | 109.5% | 97.7% |
| Shark | 0.00% | -0.17% | -0.24% | -0.04% | -0.05% | -0.05% | 102.3% | 101.0% |
| 1024x768 | 0.00% | -0.23% | -0.09% | -0.05% | -0.05% | -0.06% | 100.6% | 97.1% |
| 1920x1088 | 0.00% | -0.09% | -0.10% | -0.02% | -0.02% | -0.03% | 103.5% | 98.5% |
| average | 0.00% | -0.14% | -0.10% | -0.03% | -0.03% | -0.04% | 102.4% | 98.0% |

# Conclusion

This proposal gives two methods to limit the range of NBDV. Both of the two methods can limit the range NBDV into a rectangle area, which makes the possible accessed area in reference view is much smaller than current design. Such a limitation can reduce the cache miss when fetching the reference views and save the bandwidth or on-chip memory for hardware design. This proposal can also improve the accuracy of NBDV and has some coding performance improvement. We recommend adopting the Method 1 because method 1 is simpler and without degradation of coding efficiency compared to Method 2.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- L. Zhang, Y. Chen and M. Karczewicz, “3D-CE5.h related: Disparity vector derivation for multiview video and 3DV,” MPEG document M24937, April 2012.

- J. Sung, M. Koo and S. Yea, “3D-CE5.h: Simplification of disparity vector derivation for HEVC-based 3D video coding,” JCT3V document A0126, July 2012.

- L. Zhang, Y. Chen and M. Karczewicz, “CE5.h: Disparity vector generation results,” JCT3V document A0097, July 2012.

- Y.-L. Chang, C.-L. Wu, Y.-P. Tsai and S. Lei, “CE5.h related: Depth oriented Neighboring Block Disparity Vector (DoNBDV) with virtual depth retrieval,” JCT3V document B0090, October 2012.