| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0083 |
| --- | --- |

| Title: | CE5 related: Results on Simplification on DLT-based Depth Intra Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Lu and Lu Yu Zhejiang University Zheda Road 38 Hangzhou, Zhejiang, China | Tel: Email: | +86 15968882677 yiqiu8932@gmail.com |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This proposal simplifies the depth intra coding performed on Depth Look-up Table (DLT) domain. By removing the transform process in DLT-based depth intra coding, the complexity of DLT-based depth intra coding can be reduced. Such a simplification makes DLT-based depth intra coding simpler than conventional intra coding.

# Problem Statement

In 6th JCT-3V meeting, contribution JCT3V-F0159[1] proposes an alternative residual generation method for DMM, DC, vertical, and horizontal modes.Instead of calculating residual as the difference of original pixel value and prediction pixel value directly, both original pixel value and prediction pixel value are firstly mapped to depth index through DLT, and then residual is generated as the difference of these two depth indexes. Let org[x], pred[x] and resi[x] be the original value, intra predicted value and residual at position x respectively, and let DLT[a] be the index of depth value a, equation (1) can be used to generate the residual of depth indexes:

resi_index[x] = DLT[org[x]] – DLT[pred[x]]                        (1)

At decoder, let rec[x] be the reconstructed value, and let IDLT[a] be the depth value of index a. the rec[x] is constructed as:

rec[x] = IDLT[ DLT[pred[x]] + resi_index[x] ]                        (2)

This method can achieve 0.1% and 0.2% coding gain under CTC and all intra cofiguration, respectively.

However, two additional looking-up table operation is needed for each pixel in PUs that use DMM, DC, vertical, and horizontal modes. Fig. 1 and Fig.2 give the flow chat of conventional intra prediction and DLT based intra prediction in decoding process. The looking-up table operations are included in the red retangles. Obviously, DLT based intra prediction is a little more complex than conventional intra prediction.

Fig. 1

Fig. 2

# Proposed Method

To solve the above problem, we propose to remove the transform and inverse transform process under the following considerations:

(1) The transform in HEVC is not designed for DLT indexes. When the residual signal represents for the index difference in DLT, the transform in HEVC may not be optimal any more.

(2) By removing the transform process, the DLT based intra prediction can be much simpler and some power could be saved both at the encoder and decoder.

Fig. 3(a) and Fig. 3(b) show the flow chat of DLT based intra prediction with and without transform.

Fig. 3(a)

Fig. 3(b)

# Experimental Results

Proposed method is implemented into HTM 9.0r1 software, and proposed method is compared with HTM 9.0r1. The test condition is CTC and All Intra Configuration.

Experimental results show that the coding loss of proposed simplification is very small (0.02% and 0.05% for CTC and all intra case), and even 0.06% and 0.02% coding gain can be achieved on dependent views.

The encoding and decoding time are similar compared to anchor.

## Results of CTC

Table 3-1: Experimental results for proposed method (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.02% | 0.07% | 0.00% | 0.05% | 0.06% | 107.4% | 87.3% |
| Kendo | 0.00% | -0.15% | -0.27% | -0.08% | 0.13% | 0.08% | 104.5% | 104.7% |
| Newspaper_CC | 0.00% | -0.09% | 0.08% | 0.01% | 0.11% | 0.08% | 105.5% | 99.6% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102.1% | 91.8% |
| Poznan_Hall2 | 0.00% | -0.20% | -0.05% | -0.05% | -0.04% | -0.05% | 105.9% | 99.6% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 103.9% | 98.0% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 104.6% | 96.3% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 103.6% | 103.2% |
| 1024x768 | 0.00% | -0.09% | -0.04% | -0.02% | 0.10% | 0.07% | 105.8% | 97.2% |
| 1920x1088 | 0.00% | -0.04% | -0.01% | -0.01% | -0.01% | -0.01% | 104.0% | 97.8% |
| average | 0.00% | -0.06% | -0.02% | -0.01% | 0.03% | 0.02% | 104.7% | 97.5% |

## Results of All Intra Configuration

Table 3-2: BD rate results for proposed method (All Intra)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 0.10% | 0.11% | 99.4% | 92.7% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | 0.14% | 0.11% | 99.6% | 95.2% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | 0.09% | 0.08% | 107.6% | 95.6% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 108.1% | 99.6% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.04% | 0.09% | 100.5% | 105.1% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 106.8% | 93.8% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102.4% | 92.1% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 103.2% | 98.0% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | 0.11% | 0.10% | 102.2% | 94.5% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.02% | 104.2% | 97.7% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.05% | 0.05% | 103.4% | 96.5% |

# Conclusion

This proposal simplifies the depth intra coding on DLT index domain. By removing the transform and inverse transform process, the DLT-based depth intra coding becomes simpler than conventional intra coding with negligible coding loss. We recommend adopting this simplification.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[1] Jin Heo, Jie. Jia and Sehoon Yea, “CE5: Fast depth lookup table application method to intra modes for depth data,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F0159, Oct. 2013.