This attachment is the modifications on the working draft by contribution “JCT3V-G0083 CE5 related: Results on Simplification on DLT-based Depth Intra Coding”. The modification is based on “JCT3V-F1001 3D-HEVC Draft Text 2” and in green highlight.

      - General decoding process for intra blocks

- ……

- Depending on SdcFlag[  xTb0 ][ xTb0 ], the following applies:

    - If SdcFlag[ xTb0 ][ yTb0 ] is equal to 0, following applies:

      - If DltFlag[ nuh_layer_id ] is equal to 1 and predModeIntra is equal to INTRA_DC, INTRA_ANGULAR10, INTRA_ANGULAR26, INTRA_DMM_WFULL, INTRA_DMM_WPREDTEX, INTRA_DMM_CPREDTEX, or INTRA_CHAIN, the variable transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is set equal to 1.

      - The scaling and transformation process as specified in subclause 8.6.2 is invoked with the luma location ( xTbY, yTbY ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nTbS as inputs, and the output is an (nTbS)x(nTbS) array resSamples.

[Ed. (GT):Meeting notes say: "Implement an enabling flag at (position t.b.d.)". However there seems to  be no decision on the position yet.]

      - If DltFlag[ nuh_layer_id ] is equal to 1 and predModeIntra is equal to INTRA_DC, INTRA_ANGULAR10, INTRA_ANGULAR26, INTRA_DMM_WFULL, INTRA_DMM_WPREDTEX, INTRA_DMM_CPREDTEX, or INTRA_CHAIN, the following applies, for i in the range of 0 to nTbS − 1, inclusive, and j in the range of 0 to nTbS − 1, inclusive:

          - idx = DepthValue2Idx[ predSamples[ i ][ j ] ] + resSamples[ i ][ j ]		(Error: Reference source not found-1)

          - SL[ xTb0 + i ][ yTb0 + j ] = 
	Idx2DepthValue[ clip3( 0, NumDepthValuesInDlt[ nuh_layer_id ] – 1, idx ) ]	(Error: Reference source not found-2)

      - Otherwise, the following applies:

        - The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the transform block location ( xTb0, yTb0 ), the transform block size nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.

    - Otherwise ( SdcFlag[ xTb0 ][ yTb0 ] is equal to 1) the following ordered steps apply:

      - The depth value reconstruction process as specified in subclause Error: Reference source not found is invoked with the location ( xTb0, yTb0 ), the transform size trafoSize set equal to nTbS, the (nTbS)x(nTbS) array predSamples, and the intra prediction mode predModeIntra, as the inputs and the output is a (nTbS)x(nTbS) array resSamples.

……