| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0084 |
| --- | --- |

| Title: | CE3 related: Modifications to motion hole filling in sub-PU level inter-view motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Hua Zhang1, Ce Zhu1, Yong-Bing Lin2, Jian-Hua Zheng2 1 School of Electronic Engineering University of Electronic Science and Technology of China, Chengdu, P. R. China,611731 2 HiSilicon technologies, Beijing, P. R. China, 100095 | Tel: Email: | +1-500-840-0575 jieruo@yeah.net +1-822-808-8676 eczhu@uestc.edu.cn +86-10-60612372 linyongbing@hisilicon.com |
| Source: | UESTC, HiSilicon |  |  |

_____________________________

- Abstract

This contribution proposes to modify the hole filling way in sub-PU level motion prediction process with two methods for cases that a sub-PU does not reach available motion from the reference block, mainly to enhance the ability of pipeline processing. Experimental results show that, average 0.02% and 0.00% BD-rate loss is observed on synthesized views with the first and second proposed methods, respectively.

- Introduction

In the following sub-sections, the merge list construction process for the dependent views in HTM9.0r1 and sub-PU level inter-view motion prediction method as proposed in JCT3V-E0184 [1] are briefly introduced, and motion hole filling process related to the sub-PU level inter-view motion prediction are also discussed.

- Merge candidate list construction process

The merge candidate list construction process for the dependent views of current 3D-HEVC is briefly described as below,

- Re-using existing MCL construction module in the HEVC.

- Insert the extra candidates into the HEVC MCL constructed in the above.

Following is a description of the inserting process for the extra candidates for the dependent views.

- Check VSP flags of spatial merging candidates (SMC) A1, B1, B0, A0 and B2 which are in the merge candidate list.

- If the current PU is in the depth map, the texture candidate (T) which is derived from the corresponding texture is identified. For T, if it is different from the A1 and B1, and derived to be available, is inserted to the head of the merge candidate list.

- The disparity vector is derived and the corresponding block of the current PU based on the disparity vector is identified.

- If the current PU is in the texture, for IvMC, if it is different from the A1 and B1, and derived to be available, is inserted to the head of the merge candidate list.

- If the current PU is in the depth map, if T is available ,for IvMC, if it is different from the T, and derived to be available, is inserted to the merge candidate list just behind the T; Otherwise, if T is not available, for IvMC, derived to be available, is inserted to the head of the merge candidate list.

- The inter-view disparity vector candidate (IvDC) which is converted from the disparity vector is inserted to the back of the last valid candidate from IvMC to B0, if it is different from the A1 and B1.

- If the BVSP is enabled for the current slice, the BVSP merging candidate is inserted to the back of the last valid candidate from IvMC to IvDC.

- If IvMC is derived to be the additional candidate, namely shifted IvMC, it is compared with the IvMC in step 3. If it is not equal to the IvMC in step 3 or if the additional candidate is a DSMV (when the additional IvMC is unavailable), the shifted IvMC candidate is inserted to the back of the last valid candidate from IvMC to B2.

- If IvDC is derived to be the additional candidate, namely shifted IvDC, it is compared with the IvDC in step 4. If it is not equal to the IvDC in step 4 , the shifted IvDC candidate is inserted to the back of the last valid candidate from IvMC to shifted IvMC.

- If the MCL is not full, the zero motion vectors are filled as a default.

During the merge candidate list construction process, the inter-view motion prediction candidate IvMC plays an important role in the prediction performance.

- Sub-PU level inter-view motion prediction

In last meeting, a sub-PU level inter-view motion prediction method is proposed in JCT3V-E0184 [1], which achieves coding gain by doing motion prediction with smaller block size instead of the full PU size.

As proposed, the inter-view merge candidate derivation process is performed on sub-PU level. In other words, each sub-PU may have its own motion information derived from a reference block in a reference view.

- Motion hole-filling process

In the sub-PU level inter-view motion prediction process, one set of temporary motion parameter is maintained and updated. For each sub-PU, if the reference block has available motion information, the temporary motion parameter is updated with the motion of the reference block. Otherwise, i.e., the reference block does not have available motion information, the current sub-PU will have a motion hole, the temporary motion parameter is copied to current sub-PU, and such a process is called the motion hole-filling process in this contribution.

According to the current design of motion hole-filling process, it is noted that there is dependency on the motion prediction process among sub-PUs within one PU, and therefore the current motion hole-filling process is not friendly to parallel processing. Specially, if the first sub-PU has a motion hole (or the beginning successive a few sub-PUs have motion holes), it (or they all) should wait to be filled by the first available temporary motion parameter, and therefore the current motion hole-filling process is not friendly to pipeline processing.

To address this problem, in last meeting, a simplification to sub-PU level inter-view motion prediction is proposed in JCT3V-F0127 [1]. The main idea is that for each sub-PU in the current PU, if the identified reference block does not contain available motion information, the followings are applied:

- If the original PU-level inter-view merge candidate is available, the corresponding motion information is inherited by the inter-view merge candidate associated with this sub-PU;

- Otherwise, zero motion (reference index is set as 0, motion vectors for both List0 and List1 are set as (0, 0)) is set for this sub-PU.

With the proposed idea, the dependency on the motion prediction process among sub-PUs within one PU is removed, and the sub-PU level motion prediction can be implemented in a pipeline or parallel fashion. However, because of the original PU-level inter-view merge candidate unavailable in the sub-PU level inter-view motion prediction process, JCT3V-F0127 [1] needs additional calculation for getting the original PU-level inter-view merge candidate, which increases additional complexity assuredly.

- Proposal

Under the premise of without any increase in complexity, to enhance the ability of pipeline processing of the current motion hole-filling process in sub-PU level inter-view motion prediction, two methods are proposed in this contribution as listed below:

Method #1:

For each sub-PU in the current PU, if the identified reference block does not contain available motion information, the followings are applied:

- If the sub-PU is the first one in the current PU, zero motion (reference index is set as 0, motion vectors for both List0 and List1 are set as (0, 0)) is set for this sub-PU;

- Otherwise, the corresponding motion information is inherited by the motion information of the previous sub-PU.

Method #2:

For each sub-PU in the current PU, if the identified reference block does not contain available motion information, the followings are applied:

- If the sub-PU is the first one in the current PU, DV information corresponding to current PU is set for this sub-PU;

- Otherwise, the corresponding motion information is inherited by the motion information of the previous sub-PU.

With the above proposed method, although the first sub-PU has a motion hole (or the beginning successive a few sub-PUs have motion holes), it (or they all) can directly get motion parameter. So the sub-PU level motion prediction can be implemented in a pipeline fashion. Meanwhile, DV information is more related to the inter-view motion parameter of the sub-PU, compared with method one, method two is more dominant in terms of prediction performance.

- Experimental results

The proposed methods are integrated based on the sub-PU inter-view motion prediction method as proposed in JCT3V-F0110 [2] and simulations have been implemented on top of HTM9.0r1 based on the common test conditions (CTC) JCT3V-F1100 [4]. Two tests, namely Test #1 and Test #2, are simulated in this section, as described below:

Test #1:

- Anchor: sub-PU level inter-view motion prediction, i.e. HTM9.0r1

- Tested: sub-PU level inter-view motion prediction with the proposed method one

Test #2:

- Anchor: sub-PU level inter-view motion prediction, i.e. HTM9.0r1

- Tested: sub-PU level inter-view motion prediction with the proposed method two

In above two tests, the sub-PU level inter-view motion prediction process is exactly the same as proposed in JCT3V-F0110 [2], and sub-PU size of 8x8 is always applied.

The coding performance of Test #1 is given in Table 1. As reported in Table 1, the overall average BD rate loss is 0.02% for the synthesized views.

Table 1: Coding results of Test #1 for 3-view case

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.06% | 0.11% | 0.03% | 0.01% | -0.07% | 99.4% | 100.3% | 100.0% |
| Kendo | 0.00% | -0.09% | 0.11% | 0.00% | 0.00% | 0.01% | 99.6% | 98.9% | 95.1% |
| Newspaper_CC | 0.00% | 0.07% | 0.14% | 0.04% | 0.04% | 0.06% | 100.2% | 109.8% | 101.2% |
| GT_Fly | 0.00% | 0.01% | -0.01% | 0.00% | -0.01% | 0.01% | 100.1% | 100.2% | 98.1% |
| Poznan_Hall2 | 0.00% | 0.20% | 0.23% | 0.09% | 0.08% | 0.15% | 100.1% | 92.0% | 99.1% |
| Poznan_Street | 0.00% | 0.02% | -0.04% | 0.00% | 0.00% | -0.01% | 100.6% | 98.1% | 101.6% |
| Undo_Dancer | 0.00% | -0.01% | 0.04% | 0.00% | 0.00% | 0.01% | 100.6% | 100.8% | 99.0% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% | 0.00% | 101.4% | 98.9% | 98.9% |
| 1024x768 | 0.00% | 0.01% | 0.12% | 0.02% | 0.01% | 0.00% | 99.7% | 103.0% | 98.7% |
| 1920x1088 | 0.00% | 0.05% | 0.04% | 0.02% | 0.01% | 0.03% | 100.6% | 98.0% | 99.3% |
| average | 0.00% | 0.03% | 0.07% | 0.02% | 0.01% | 0.02% | 100.2% | 99.9% | 99.1% |

The coding performance of Test #2 is given in Table 2. As reported in Table 2, the overall average BD rate loss is 0.00% for the synthesized views.

Table 2: Coding results of Test #2 for 3-view case

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.05% | 0.08% | 0.01% | 0.01% | -0.05% | 100.8% | 101.5% | 100.1% |
| Kendo | 0.00% | -0.04% | 0.08% | 0.02% | 0.05% | 0.02% | 99.7% | 101.6% | 94.2% |
| Newspaper_CC | 0.00% | 0.06% | 0.07% | 0.02% | 0.02% | -0.01% | 100.4% | 109.0% | 103.1% |
| GT_Fly | 0.00% | -0.07% | -0.08% | -0.01% | -0.01% | -0.01% | 101.0% | 102.2% | 99.9% |
| Poznan_Hall2 | 0.00% | 0.25% | 0.25% | 0.08% | 0.04% | 0.10% | 101.1% | 101.3% | 97.6% |
| Poznan_Street | 0.00% | 0.09% | -0.05% | 0.01% | 0.01% | 0.01% | 99.9% | 90.4% | 101.4% |
| Undo_Dancer | 0.00% | -0.07% | -0.14% | -0.02% | -0.03% | -0.05% | 99.3% | 102.2% | 98.6% |
| Shark | 0.00% | 0.05% | -0.09% | 0.00% | -0.01% | -0.01% | 100.7% | 99.6% | 100.5% |
| 1024x768 | 0.00% | -0.01% | 0.08% | 0.02% | 0.03% | -0.01% | 100.3% | 104.0% | 99.2% |
| 1920x1088 | 0.00% | 0.05% | -0.02% | 0.01% | 0.00% | 0.01% | 100.4% | 99.1% | 99.6% |
| average | 0.00% | 0.03% | 0.02% | 0.01% | 0.01% | 0.00% | 100.4% | 101.0% | 99.4% |

- Complexity analysis

Because zero motion can be directly obtained, and the DV information is also available in the sub-PU level motion prediction process, all the above methods do not increase the complexity.

- Patent rights declaration(s)

UESTC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

HiSilicon Technologies and Huawei technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

- J. An, K. Zhang, J.-L. Lin, S. Lei, “3D-CE3.h related: Sub-PU level inter-view motion prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E0184, 5th Meeting: Vienna, AU, July 27–Aug. 2, 2013.

- J. An, K. Zhang, J.-L. Lin, S. Lei, “3D-CE3: Sub-PU level inter-view motion prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F0110, 6th Meeting: Geneva, CH, Oct. 25– Nov. 1, 2013.

- X. Zhao, L. Zhang, Y. Chen, “CE3 related: Simplifications to sub-PU level inter-view motion prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F0127, 6th Meeting: Geneva, CH, Oct. 25– Nov. 1, 2013.

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F1100, 6th Meeting: Geneva, CH, Oct. 25– Nov. 1, 2013.