| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0068r2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0085r2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On parameter improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Signalling a flag general_cross_layer_aligned_idr_only_flag in vps_vui is proposed. This flag indicates the VCL NAL units of all IRAP pictures have the same value of nal_unit_type as IDR_W_RADL or IDR_N_LP ,and all IRAP pictures are cross-layer aligned. This indication is helpful for some legacy video streaming systems that support only closed-loop IDR pictures for random access.

# Parameter clean-ups of HEVC

- general_cross_layer_aligned_idr_only_flag is signalled in vps_vui. This flag indicates the VCL NAL units of all IRAP pictures have the same value of nal_unit_type as IDR_W_RADL or IDR_N_LP ,and all IRAP pictures are cross-layer aligned. This flag is beneficial for the video delivery and play systems that support only closed-loop IDR pictures for random access. Some legacy video systems may not correctly handle the CRA/BLA pictures and the associated leading pictures. If the RASL pictures associated with IRAP with NoRaslOutputFlag equal to 1 are decoded without any indication, it may result in the unintended behaviour of decoders. As far as we know, some legacy real-time video streaming systems or video-on-demand systems cannot support the CRA/BLA pictures or open-loop I picture handlings for random access. This flag is helpful for indicating whether only IDR is used for random access in a bitstream or not.

# Proposed specification text

| vps_vui( ){ | Descriptor |
| --- | --- |
| cross_layer_pic_type_aligned_flag | u(1) |
| if( !cross_layer_pic_type_aligned_flag ) |  |
| cross_layer_irap_aligned_flag | u(1) |
| if( cross_layer_irap_aligned_flag ) |  |
| general_cross_layer_aligned_idr_only_flag | u(1) |
| ... |  |
| } |  |

general_cross_layer_aligned_idr_only_flag equal to 1 indicates that within each access unit for which the VCL NAL units refer to the VPS, when one picture is an IRAP picture, all the pictures are IDR pictures and have the same value of nal_unit_type. general_cross_layer_aligned_idr_only_flag equal to 0 specifies that the above restriction may or may not apply.

- Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).