| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0071 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0088 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On auxiliary pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Signalling the auxiliary picture types used for a CVS in VPS extension is proposed for session negotiation and sub-bitstream extraction. For efficient layer-dependency signalling, if a layer of primary pictures and layers of its associated auxiliary pictures have the same layer dependency, only one representative dependency information for the grouped layers is signalled in VPS. Furthermore, some constraints for slice header parameters of auxiliary pictures are proposed.

# Auxiliary picture indication and grouping

The auxiliary picture types used for a CVS are identified with aux_mask_flag[ i ]. From VPS extension paring, what kinds of auxiliary pictures are present in the CVS shall be notified. This information is helpful for session negotiation and sub-bitstream extraction. A layer of primary pictures and layers of auxiliary pictures associated the primary pictures can be grouped, when they have the same layer dependencies. The layer dependency information of the grouped layers can be efficiently represented. vps_aux_picture_grouped_flag can decide if a primary picture layer and associated auxiliary picture layers are grouped or they are independently signaled in VPS and managed in DPB.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_aux_picture_use_flag | u(1) |
| if(vps_aux_picture_use_flag) { |  |
| for( i = 0, NumAuxTypes = 0; i < 16; i++ ) { |  |
| aux_mask_flag[ i ] | u(1) |
| NumAuxTypes += aux_mask_flag[ i ] |  |
| } |  |
| vps_aux_picture_grouped_flag | u(1) |
| } |  |
| NumGroupedLayers = 1 |  |
| if(vps_aux_picture_grouped_flag) |  |
| NumGroupedLayers += NumAuxTypes |  |
| for( i = 1; i <= MaxLayersMinus1; i+= NumGroupedLayers ) |  |
| for( j = 0; j < i; j+= NumGroupedLayers ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| … |  |
| direct_dep_type_len_minus2 | ue(v) |
| default_direct_dependency_flag | u(1) |
| if( default_direct_dependency_flag ) |  |
| default_direct_dependency_type | u(v) |
| else { |  |
| for( i = 1; i <= MaxLayersMinus1; i+= NumGroupedLayers ) |  |
| for( j = 0; j < i; j+= NumGroupedLayers ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| } |  |
| … |  |
| } |  |

vps_aux_picture_use_flag equal to 1 specifies that an auxiliary picture is present in a bitstream, and aux_mask_flag[i] is present in VPS. vps_aux_picture_use_flag equal to 0 specifiest hat an auxiliary picture is not present in a bitstream, and aux_mask_flag is inferred equal to 0.

aux_mask_flag [ i ] equal to 1 indicates that auxiliary pictures with the i-th auxiliay picture type are present in a bitstream. aux_mask_flag [ i ] equal to 0 indicates that auxiliary pictures with the i-th auxialiary picture type are not present in a bitstream.

vps_aux_picture_grouped_flag equal to 1 specifies that the nuh_layer_id of the i-th auxiliary pictures associated with a primary picture with nuh_layer_id equal to k is equal to (k+i). The auxiliary pictures have the same layer dependencies as the corresponding primary pictures. vps_aux_picture_grouped_flag equal to 0 specifies that the above restriction may or may not apply. If not present, vps_aux_picture_grouped_flag is inferred to be equal to 0.

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with index j may be a direct reference layer for the layer with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to MaxLayersMinus1, it is inferred to be equal to 0.

The variables NumDirectRefLayers[ i ] andRefLayerId[ i ][ j ]are derived as follows:

for( i = 0; i  <=  MaxLayersMinus1; i+= NumGroupedLayers ) 
for( k = 0; k < NumGroupedLayers; k++ ) {
	iNuhLId = layer_id_in_nuh[ i ] + k
	NumDirectRefLayers[ iNuhLId ] = 0
	for( j = k; j < i; j+= NumGroupedLayers )
		if( direct_dependency_flag[ i ][ j ] )
        RefLayerId[ iNuhLId ][ NumDirectRefLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ]
}

direct_dependency_type[ i ][ j ] indicates the type of dependency between the layer with nuh_layer_id equal layer_id_in_nuh[ i ] and the layer with nuh_layer_id equal to layer_id_in_nuh[ j ]. direct_dependency_type[ i ][ j ]equal to 0 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for both inter-layer sample prediction and inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ]equal to 1 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer sample prediction but not for inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ]equal to 2 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer motion prediction but not for inter-layer sample prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. The length of the direct_dependency_type[ i ][ j ] syntax element is direct_dep_type_len_minus2 + 2 bits. Although the value of direct_dependency_type[ i ][ j ] shall be in the range of 0 to 2, inclusive, in this version of this Specification, decoders shall allow values of direct_dependency_type[ i ][ j ] in the range of 3 to 232 − 2, inclusive, to appear in the syntax. When not present, the value of direct_dependency_type[ i ][ j ] is inferred to be equal to default_direct_dependency_type.

The variables VpsInterLayerSamplePredictionEnabled[ i ][ j ] and VpsInterLayerMotionPredictionEnabled[ i ][ j ] are derived as follows:

for( i = 0; i  <=  MaxLayersMinus1; i+= NumGroupedLayers ) 
for( k = 0; k < NumGroupedLayers; k++ ) {
	iNuhLId = layer_id_in_nuh[ i ] + k
	NumDirectRefLayers[ iNuhLId ] = 0
	for( j = k; j < i; j+= NumGroupedLayers )
   if( direct_dependency_flag[ i ][ j ] ) {
     VpsInterLayerSamplePredictionEnabled[ i + k ][ j ] = direct_dependency_type[ i ][ j ]& 0x1
     VpsInterLayerMotionPredictionEnabled[ I + k ][ j ] = direct_dependency_type[ i ][ j ]& 0x2
   }
}

# Constraints on auxiliary pictures

Constraint 1:

When present, the value of the slice segment header syntax elements pic_output_flag, no_output_of_prior_pics_flag, slice_pic_order_cnt_lsb, discardable_flag, cross_layer_bla_flag, poc_reset_flag, shall be the same in all slice segment headers of a primary coded picture and the associated auxiliary coded pictures.

Constraint 2:

For all slices of auxiliary pictures, slice_sao_luma_flag and slice_sao_chroma_flag shall both be equal to 0

Or,

When AuxId[ lId ] is equal to AUX_DEPTH, slice_sao_luma_flag and slice_sao_chroma_flag shall both be equal to 0

Constraint 3:

For all slices of auxiliary pictures, slice_deblocking_filter_disabled_flag  shall be equal to 0

Or,

When AuxId[ lId ] is equal to AUX_DEPTH, slice_deblocking_filter_disabled_flag  shall be equal to 0

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).