| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0079r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0092r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: comments on MV-HEVC WD 6 and SHVC WD 4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hahyun Lee Jung Won Kang Jinho Lee Jin Soo Choi 218 Gajeong-ro, Yuseong-gu Daejeon, Rep. of Korea | Tel: Email: | +82-42-860-6138 hanilee@etri.re.kr jungwon@etri.re.kr jinosoul@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

# Abstract

This document discuss the following issues in MV-HEVC draft text 6 and SHVC draft text 4 and proposes corresponding solutions: 1) derivation of the variable NumActiveRefLayerPics, 2) profile, tier and level information and 3) semantics of vps_number_layer_sets_minus1.

This revision provides a revised derivation process of the variable NumActiveRefLayerPics according to the discussion of HLS BoG.

# Proposals

## The derivation of the variable NumActiveRefLayerPics

In the previous meeting, semantic changes of the syntax element all_ref_layers_active_flag and sending the syntax element sub_layers_vps_max_minus1[] for each layer in the VPS extension are adopted into MV-HEVC/SHVC draft specification. And the derivation process of NumActiveRefLayerPics is changed as follows in the current MV-HEVC/SHVC draft text for these adoptions.

The variables numRefLayerPics and refLayerPicFlag[i] and refLayerPicIdc[j] are derived as follows:

for( i = 0, j = 0;  i < NumDirectRefLayers[ nuh_layer_id ]; i++ ) {
	refLayerIdx = LayerIdxInVps[ RefLayerId[ nuh_layer_id ][ i ] ]
	refLayerPicFlag[ i ] = ( sub_layers_vps_max_minus1[ refLayerIdx ] >= TemporalId )  && 
				( max_tid_il_ref_pics_plus1[ refLayerIdx ][ LayerIdxInVps[ nuh_layer_id ] ] > TemporalId )
	if( refLayerPicFlag[ i ] )
		refLayerPicIdc[ j++ ] = i
}

numRefLayerPics = j

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  NumDirectRefLayers[ nuh_layer_id ]  = =  0 )
	NumActiveRefLayerPics = 0
else if( all_ref_layers_active_flag )
	NumActiveRefLayerPics = numRefLayerPics
else if( !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = refLayerPicFlag[ 0 ] ? 1 : 0
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

When the syntax element “max_one_active_ref_layer_flag” is equal to 1 or NumDirectRefLayers [nuh_layer_id] is equal to 1, the value of NumActiveRefLayerPics depends on the value of refLayerPicFlag [0]. However, when “max_one_active_ref_layer_flag” is equal to 1, it specifies that at most one picture is used for inter-layer prediction for each picture in the CVS but does not guarantee that NumDirectRefLayers is always equal to 1. For example, when “max_one_active_ref_layer_flag” is equal to 1, the following case is possible;

- NumDirectRefLayerPics[nuh_layer_id] == 2

- refLayerPicFlag[ 0 ] == 0 and refLayerPicFlag[1] == 1

In this case, even though there is an actual reference picture that can be used in decoding of the current picture for inter-layer prediction, the variable NumActiveRefLayersPics is equal to be 0 by current derivation process.

Therefore, the derivation process of variable NumActiveRefLayerPics is proposed to be modified as follows

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0 | | NumDirectRefLayers[ nuh_layer_id ]  = =  0)
	NumActiveRefLayerPics = 0
else if( all_ref_layers_active_flag )
	NumActiveRefLayerPics = numRefLayerPics
else if( !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = numRefLayerPics > 0 ? 1: 0 
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

Or, the variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0 | | numRefLayerPics = =  0)
	NumActiveRefLayerPics = 0
else if( all_ref_layers_active_flag )
	NumActiveRefLayerPics = numRefLayerPics
else if( !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = 1 
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

The derivation process of the variable NumActiveRefLayersPics is modified according to the discussion of HLS BoG

The variables numRefLayerPics and refLayerPicIdc[j] are derived as follows:

for( i = 0, j = 0;  i < NumDirectRefLayers[ nuh_layer_id ]; i++ ) {
	refLayerIdx = LayerIdxInVps[ RefLayerId[ nuh_layer_id ][ i ] ]
	if (( sub_layers_vps_max_minus1[ refLayerIdx ] >= TemporalId )  && 
		( max_tid_il_ref_pics_plus1[ refLayerIdx ][ LayerIdxInVps[ nuh_layer_id ] ] > TemporalId ))
		refLayerPicIdc[ j++ ] = i
}

numRefLayerPics = j

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  numRefLayerPics  = =  0 )
	NumActiveRefLayerPics = 0
else if( all_ref_layers_active_flag )
	NumActiveRefLayerPics = numRefLayerPics
else if( !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = 1
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

## Clean-up of profile, tier and level information

In the previous meeting, JCTVC-O0109 indicated that the syntax element “profile_ref_minus1[i] is broken when i is equal to 1 and syntax element “vps_profile_present_flag[1]” is equal to 0. From BoG discussion, modification of the semantics of “profile_ref_minus[i]” to replace “shall be less than i” with “shall be less than or equal to i” was adopted into MV-HEVC/SHVC draft text as follows.

profile_ref_minus1[ i ] specifies that the profile and tier information for the i-th profile_tier_level( ) syntax structure is inferred to be equal to the profile and tier information for the ( profile _ref_minus1[ i ] + 1 )-th layer set. The value of profile_ref_minus1[i] + 1 shall be less than or equal to i.

However, the syntax element “profile_ref_minus1[i]” is broken as ever, when i equal to 1 and “vps_profile_present_flag[1]” is equal to 0.

Modified semantics of “profile_ref_minus1[i]” makes that profile and tier information for the 1st profile_tier_level() syntax structure is inferred to be equal to the profile and tier information for 1st profile_tier_level() syntax structure, when i is equal to 1 and “vps_profile_present_flag[1]” is equal to 0. Also, since no other layer in the same bitream can conform to any version 1 profile, profile and tier information for the 1 st profie_tier_level() syntax structure cannot be inferred to be equal to the profile and tier information for the 0th profile_tier_level() syntax structure. Therefore, the constraint is needed for semantics of “vps_profile_present_flag[i]” to solve this broken as follows, highlighted in turquoise

vps_profile_present_flag[i] equal to 1 specifies that the profile and tier information is present in the i-th profile_tier_level( ) syntax structure. vps_profile_present_flag[i] equal to 0 specifies that profile and tier information is not present in the i-th profile_tier_level( ) syntax structure and is inferred. The value of vps_profile_present_flag[1] for 1st profile_tier_level() syntax structure shall be equal to 1.

profile_ref_minus1[i] specifies that the profile and tier information for the i-th profile_tier_level( ) syntax structure is inferred to be equal to the profile and tier information for the ( profile _ref_minus1[i] + 1 )-th profile_tier_level() syntax structure. The value of profile_ref_minus1[i] + 1 shall be less than or equal to i.

## Semantics of vps_number_layer_sets_minus1

In MV-HEVC/SHVC draft text, syntax element “vps_number_layer_sets_minus1” signalled in VPS extension specifies the number of layer sets. And, the value of vps_number_layer_sets_minus1 shall be in the range of 0 to 1023. Since a layer set 0 consists of a layer with nuh_layer_id equal to 0 (i.e., the value of numLayersInIdList[0] is set equal to 1 and the value of layerSetLayerIdList[0][0] is set equal to 0) and MV-HEVC/SHVC bitstream has to have more than two layers, it is clear that, in MV-HEVC/SHVC profile, there should be more than two layer sets which means that value of vps_number_layer_sets_minus1 is always greater than 1. Therefore, it is asserted that the value of vps_number_layer_sets_minus1 shall be in the range of 1 to 1023.

vps_number_layer_sets_minus1 plus 1 specifies the number of layer sets that are specified by the VPS. The value of vps_number_layer_sets_minus1 shall be in the range of 1 to 1023, inclusive, and shall be equal to vps_num_layer_sets_minus1

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).