| depth_mode_parameters( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ¦ ¦ DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_CHAIN ) |  |
| edge_start_left_flag[ x0 ][ y0 ] | ae(v) |
| edge_start_position[ x0 ][ y0 ] | ae(v) |
| num_edge_codes[ x0 ][ y0 ] | ae(v) |
| for( k = 0; k <= num_edge_codes; k++ ) |  |
| edge_code[ k ] | ae(v) |
| if( DmmFlag[ x0 ][ y0 ] ¦ ¦ SdcFlag[ x0 ][ y0 ] ) { |  |
| dcNumSeg = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_PLANAR ) ? 1 : 2 |  |
| depth_dc_flag[ x0 ][ y0 ] | ae(v) |
| if ( depth_dc_flag[ x0 ][ y0 ] ) |  |
| for( i = 0; i < dcNumSeg; i ++ ) { |  |
| depth_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if ( depth_dc_abs[ x0 ][ y0 ][ i ]) |  |
| depth_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |

depth_intra_mode[ x0 ][ y0 ] specifies the depth intra mode of the current prediction unit. Table Error: Reference source not found-1 specifies the value of the variable depthIntraModeMaxLen depending on depthIntraModeSet and the value of the variable DepthIntraMode and the associated name depending on the on depth_intra_mode and depthIntraModeSet.

The variable SdcFlag[ x0 ][ y0 ] is derived as specified in the following:

- SdcFlag[ x0 ][ y0 ] =	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_PLANAR )  | |  	(Error: Reference source not found-1)
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_DMM_WFULL )

The variable DmmFlag[ x0 ][ y0 ] is derived as specified in the following:

- DmmFlag[ x0 ][ y0 ] =	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_WFULL )  | |  	(Error: Reference source not found-2)
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_CPREDTEX
                    DepthIntraMode [x0 ][ y0 ] = = INTRA_DEP_CHAIN )

Table Error: Reference source not found-1 – Specification of DepthIntraMode and associated name depending on depthIntraModeSet and depth_intra_mode and specification of and depthIntraModeMaxLen depending on depthIntraModeSet

|  | depthIntraModeSet | 0 | 1 | 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | depthIntraModeMaxLen | 1 | 32 | 34 |  |
| DepthIntraMode | Associated name | depth_intra_mode |  |  |  |
| 0 | INTRA_DEP_SDC_PLANAR | 0 | - | 0 |  |
| 1 | INTRA_DEP_NONE | 1 | 0 | 1 |  |
| 2 | INTRA_DEP_SDC_DMM_WFULL | - | - | 2 |  |
| 3 | INTRA_DEP_DMM_WFULL | - | 1 | 3 |  |
| 4 | INTRA_DEP_DMM_CPREDTEX INTRA_DEP_CHAIN | - | - 2 | 4 |  |

edge_start_left_flag[ x0 ][ y0 ] equal to 0 specifies that the start point of region boundary chain coding is located on the top row boundary of the current block. edge_start_left_flag[ x0 ][ y0 ] equal to 1 specifies that the start point of region boundary chain coding is located on the left column boundary of the current block.

edge_start_position[ x0 ][ y0 ] specifies the column position of the start point of region boundary chain coding when edge_start_left_flag[ x0 ][ y0 ] is equal to 0 and specifies the row position of the start point of region boundary chain coding when edge_start_left_flag[ x0 ][ y0 ] is equal to 1.

num_edge_codes[ x0 ][ y0 ] specifies the number of edges within the current block.

edge_code[ x0 ][ y0 ][ k ] shall be one of the values shown in Table Error: Reference source not found-2. edge_code[ x0 ][ y0 ][ k ] is used to derive the edge direction of the k-th edge when edge_intra_flag[ x0 ][ y0 ] is equal to 1.

- Table Error: Reference source not found-2 – Interpretation of edge_code[ x0 ][ y0 ][ k ]

| edge_code[ x0 ][ y0 ][ k ] | edge direction |
| --- | --- |
| 0 | 0° |
| 1 | 45° |
| 2 | −45° |
| 3 | 90° |
| 4 | −90° |

- H.8.4	Decoding process for coding units coded in intra prediction mode

- H.8.4.1General decoding process for coding units coded in intra prediction mode

The specifications in subclause 8.4.1 apply with the following modification:

–	All invocations of the process specified in subclause 8.4.2 are replaced with invocations of the process specified in subclause .

–	All invocations of the process specified in subclause 8.4.4.1 are replaced with invocations of the process specified in subclause Error: Reference source not found.

- H.8.4.2	Derivation process for luma intra prediction mode

Input to this process is a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top left luma sample of the current picture.

In this process, the luma intra prediction mode IntraPredModeY[ xPb ][ yPb ] is derived.

Table Error: Reference source not found-3 specifies the value for the intra prediction mode and the associated names.

Table Error: Reference source not found-3 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..34 | INTRA_ANGULAR2..INTRA_ANGULAR34 |
| 35 | INTRA_DMM_WFULL |
| 36 | INTRA_DMM_CPREDTEX INTRA_CHAIN |

IntraPredModeY[ xPb ][ yPb ] labelled 0..34 represents directions of predictions as illustrated in Figure 8 1.

- If DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_SDC_PLANAR, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_PLANAR.

- Otherwise, if DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_SDC_DMM_WFULL, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_DMM_WFULL.

- Otherwise, if DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_DMM_WFULL, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_DMM_WFULL.

- Otherwise if DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_DMM_CPREDTEX INTRA_DEP_CHAIN, IntraPredModeY[ xPb ][ yPb ] is set equal to INTRA_DMM_CPREDTEX INTRA_CHAIN.

- Otherwise ( DepthIntraMode[ xPb ][ yPb ] is equal to INTRA_DEP_NONE ), IntraPredModeY[ xPb ][ yPb ] is derived as the following ordered steps:

  - The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xPb − 1, yPb ) and ( xPb, yPb − 1 ), respectively.

  - For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

  - The availability derivation process for a block in z-scan order as specified in subclause 6.4.2 is invoked with the location ( xCurr, yCurr ) set equal to ( xPb, yPb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

  - The candidate intra prediction mode candIntraPredModeX is derived as follows:

  - If availableX is equal to FALSE, candIntraPredModeX is set equal to INTRA_DC.

  - Otherwise, if CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA or pcm_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeX is set equal to INTRA_DC,

  - Otherwise, if X is equal to B and yPb − 1 is less than ( ( yPb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ), candIntraPredModeB is set equal to INTRA_DC.

  - Otherwise, if candIntraPredModeX is larger than 34, candIntraPredModeX is set equal to INTRA_DC.

  - Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

  - The candModeList[ x ] with x = 0..2 is derived as follows:

  - If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

  - If candIntraPredModeA is less than 2 (i.e. equal to INTRA_PLANAR or INTRA_DC), candModeList[ x ] with x = 0..2 is derived as follows:

          - candModeList[ 0 ] = INTRA_PLANAR	(Error: Reference source not found-3)

          - candModeList[ 1 ] = INTRA_DC		(Error: Reference source not found-4)

          - candModeList[ 2 ] = INTRA_ANGULAR26	(Error: Reference source not found-5)

          - Otherwise, candModeList[ x ] with x = 0..2 is derived as follows:

          - candModeList[ 0 ] = candIntraPredModeA	(Error: Reference source not found-6)

          - candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(Error: Reference source not found-7)

          - candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(Error: Reference source not found-8)

          - Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

          - candModeList[ 0 ] and candModeList[ 1 ] are derived as follows:

          - candModeList[ 0 ] = candIntraPredModeA	(Error: Reference source not found-9)

          - candModeList[ 1 ] = candIntraPredModeB	(Error: Reference source not found-10)

          - If neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_PLANAR, candModeList[ 2 ] is set equal to INTRA_PLANAR,

          - Otherwise, if neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_DC, candModeList[ 2 ] is set equal to INTRA_DC,

          - Otherwise, candModeList[ 2 ] is set equal to INTRA_ANGULAR26.

  - IntraPredModeY[ xPb ][ yPb ] is derived by applying the following procedure:

  - If prev_intra_luma_pred_flag[ xPb ][ yPb ] is equal to 1, the IntraPredModeY[ xPb ][ yPb ] is set equal to  candModeList[ mpm_idx ].

  - Otherwise, IntraPredModeY[ xPb ][ yPb ] is derived by applying the following ordered steps:

- The array candModeList[ x ], x = 0..2 is modified as the following ordered steps:

  - When candModeList[ 0 ] is greater than candModeList[ 1 ], both values are swapped as follows:

( candModeList[ 0 ], candModeList[ 1 ] ) = Swap( candModeList[ 0 ], candModeList[ 1 ] )	(Error: Reference source not found-11)

  - When candModeList[ 0 ] is greater than candModeList[ 2 ], both values are swapped as follows:

( candModeList[ 0 ], candModeList[ 2 ] ) = Swap( candModeList[ 0 ], candModeList[ 2 ] )	(Error: Reference source not found-12)

  - When candModeList[ 1 ] is greater than candModeList[ 2 ], both values are swapped as follows:

( candModeList[ 1 ], candModeList[ 2 ] ) = Swap( candModeList[ 1 ], candModeList[ 2 ] )	(Error: Reference source not found-13)

- IntraPredModeY[ xPb ][ yPb ] is derived by the following ordered steps:

  - IntraPredModeY[ xPb ][ yPb ] is set equal to rem_intra_luma_pred_mode[ xPb ][ yPb ].

  - For i equal to 0 to 2, inclusive, when IntraPredModeY[ xPb ][ yPb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xPb ][ yPb ] is incremented by one.

- H.8.4.4.2	Intra sample prediction

- H.8.4.4.2.1	General intra sample prediction

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbS specifying the transform block size,

- a variable cIdx specifying the colour component of the current block.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The nTbS * 4 + 1 neighbouring samples p[ x ][ y ] that are constructed samples prior to the deblocking filter process, with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1, are derived as follows:

- The neighbouring location ( xNbCmp, yNbCmp ) is specified by:

    - ( xNbCmp, yNbCmp ) = ( xTbCmp + x, yTbCmp + y )	(8 27)

- The current luma location ( xTbY, yTbY ) and the neighbouring luma location ( xNbY, yNbY ) are derived as follows:

    - ( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTbCmp, yTbCmp ) : ( xTbCmp  <<  1, yTbCmp  <<  1 )	(8 28)

    - ( xNbY, yNbY ) = ( cIdx  = =  0 ) ? ( xNbCmp, yNbCmp ) : ( xNbCmp  <<  1, yNbCmp  <<  1 )	(8 29)

- The availability derivation process for a block in z-scan order as specified in subclause 6.4.1 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xNbY, yNbY ) as inputs, and the output is assigned to availableN.

- Each sample p[ x ][ y ] is derived as follows:

- If one or more of the following conditions are true, the sample p[ x ][ y ] is marked as "not available for intra prediction":

- The variable availableN is equal to FALSE.

- CuPredMode[ xNbY ][ yNbY ] is not equal to MODE_INTRA and constrained_intra_pred_flag is equal to 1.

- Otherwise, the sample p[ x ][ y ] is marked as "available for intra prediction" and the sample at the location ( xNbCmp, yNbCmp ) is assigned to p[ x ][ y ].

- When at least one sample p[ x ][ y ] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1 is marked as "not available for intra prediction", the reference sample substitution process for intra sample prediction in subclause 8.4.4.2.2 is invoked with the samples p[ x ][ y ] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1, nTbS, and cIdx as inputs, and the modified samples p[ x ][ y ] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1 as output.

- Depending on the value of predModeIntra, the following ordered steps apply:

- When cIdx is equal to 0 and predModeIntra is in the range of 0 to 34, the filtering process of neighbouring samples specified in subclause 8.4.4.2.3 is invoked with the sample array p and the transform block size nTbS as inputs, and the output is reassigned to the sample array p.

  - The intra sample prediction process according to predModeIntra applies as follows:

  - If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode specified in subclause 8.4.4.2.4 is invoked with the sample array p and the transform block size nTbS as inputs, and the output is the predicted sample array predSamples.

  - Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode specified in subclause 8.4.4.2.5 is invoked with the sample array p, the transform block size nTbS, and the colour component index cIdx as inputs, and the output is the predicted sample array predSamples.

  - Otherwise, if predModeIntra is in the range of INTRA_ANGULAR2..INTRA_ANGULAR34, the corresponding intra prediction mode specified in subclause 8.4.4.2.6 is invoked with the intra prediction mode predModeIntra, the sample array p, the transform block size nTbS, and the colour component index cIdx as inputs, and the output is the predicted sample array predSamples.

  - Otherwise, if predModeIntra is equal to INTRA_DMM_WFULL, the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the location ( xTbY, yTbY ), the sample array p and the transform block size nTbS as the inputs and the output are the predicted sample array predSamples.

  - Otherwise, if predModeIntra is equal to INTRA_DMM_CPREDTEX INTRA_CHAIN, the corresponding intra prediction mode specified in subclause  is invoked with the location ( xTbY, yTbY ), with the sample array p and the transform block size nTbS as the inputs and the output are the predicted sample array predSamples.

- H.8.4.4.2.8	Specification of intra prediction mode INTRA_DMM_CPREDTEX INTRA_CHAIN

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size,

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified by the following ordered steps:

  - The variable recTextPic is set equal to the array of the reconstructed luma picture samples of TexturePic..

  - The variable textThresh specifying a threshold for the segmentation of recTextPic is derived as specified in the following.

  - The variable sumTextPicVals is set equal to 0.

  - For x = 0..nTbS − 1 the following applies

  - For y = 0..nTbS − 1 the following applies

          - sumTextPicVals  +=  recTextPic[ xTb + x ][ yTb + y ]	(Error: Reference source not found-14)

          - The variable textThresh is set equal to ( sumTextPicVals  >>  ( 2 * log2( nTbS ) ) )

  - The variable wedgeletPattern[ x ][ y ] with x, y =0..nTbS − 1 specifying a binary partition pattern is derived as specified in the following.

  - For x = 0..nTbS − 1 the following applies

  - For y = 0..nTbS − 1 the following applies

          - wedgeletPattern[ x ][ y ] = ( recTextPic[ xTb + x ][ yTb + y ] > textThresh )	(Error: Reference source not found-15)

The depth partition value derivation and assignment process as specified in subclause Error: Reference source not found is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgeletPattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to depth_dc_flag[ xTb ][ yTb ], and the DC Offsets DcOffset[ xTb ][ yTb ][ 0 ], and DcOffset[ xTb ][ yTb ][ 1 ] as inputs and the output is assigned to predSamples[ x ][ y ].

Inputs to this process are:

- a sample location ( xTb, yTb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a variable nTbS specifying the transform block size,

Output of this process is:

- the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived as specified in the following ordered steps.

  - The start point of the chain in the current prediction unit is derived from edge_start_left_flag[ xTb ][ yTb ] and edge_start_position[ xTb ][ yTb ].

- If edge_start_left_flag[ xTb ][ yTb ] is equal to 1, the start point is set as s[ 0 ][ yS ] where yS is equal to edge_start_position[ xTb ][ yTb ].

- Otherwise, the start point is set as s[ xS ][ 0 ] where xS is equal to edge_start_position[ xTb ][ yTb ].

  - The number of edges in the current prediction unit is derived from num_edge_codes[ xTb ][ yTb ].

  - The direction of each edge is derived from edge_code[ xTb ][ yTb ][ k ], for k = 0.. num_edge_codes [ xTb ][ yTb ]. The direction of the edge is derived from edge_code as shown in Table Error: Reference source not found-2X.

  - The region boundary generated by connecting each edge separates the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 into two regions: the region rA that covers the top-left pixel (x0, y0) and the region rB that covers the remaining region.

For x, y = 0..nTbS − 1 the binary pattern edgePattern[ x ][ y ] is derived as follows:

- If (x, y) is covered by rA, edgePattern[ x ][ y ] = 0.

- Otherwise, edgePattern[ x ][ y ] = 1.

The depth partition value derivation and assignment process as specified in subclause Error: Reference source not found9 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern edgePattern[ x ][ y ], the transform size nTbS, the dcOffsetAvailFlag set equal to depth_dc_flag[ xTb ][ yTb ], and the DC offsets DcOffset[ xTb ][ yTb ][ 0 ], and DcOffset[ xTb ][ yTb ][ 1 ] as inputs and the output is assigned to predSamples[ x ][ y ].

- H.9.3.2 Initialization process

- H.9.3.2.1 General

The specifications in subclause 9.3.1 apply with the following modifications.

–	All invocations of the process specified in subclause 9.3.2.2 are replaced with invocations of the process specified in subclause .

–	All invocations of the process specified in subclause 9.3.2.4 are replaced with invocations of the process specified in subclause Error: Reference source not found.

- H.9.3.2.2 Initialization process for context variables

The specifications in subclause 9.3.2.2 apply with the following modifications.

- All references to the process specified in subclauses7.3.8.1 through 7.3.8.11 are replaced with references to the process specified in subclauses Error: Reference source not found to Error: Reference source not found.

- Table Error: Reference source not found-4 is appended to the end of Table 9-4.

- Error: Reference source not found to Error: Reference source not found are appended to the end of the subclause.

Table Error: Reference source not found-4 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) depth_mode_parameters( ) | depth_intra_mode | Table Error: Reference source not found-5 | 0..7 | 8..15 | 16..23 |
|  | wedge_full_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | depth_dc_flag | Table Error: Reference source not found-6 – Values of initValue for depth_intra_mode ctxIdx | 0 | 1 | 2 |
|  | depth_dc_abs | Error: Reference source not found | 0 | 1 | 2 |
|  | iv_res_pred_weight_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | ic_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | inter_sdc_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | inter_sdc_resi_abs_minus1 | Error: Reference source not found | 0 | 1 | 2 |
|  | inter_sdc_resi_sign_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | edge_start_left_flag | Table H-19-1 | 0 | 1 | 2 |
|  | num_edge_codes | Table H-19-1 | 0 | 1 | 2 |
|  | edge_code | Table H-19-2 | 0 | 1 | 2 |

Table Error: Reference source not found-5 – Values of initValue for depth_intra_mode ctxIdx

| Initialization variable | ctxIdx of depth_intra_mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| initValue | 0 | 0 | 64 | 168 | 168 | 124 | 154 | 0 |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 0 | 64 | 0 | 183 | 154 | 108 | 0 | 0 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| initValue | 64 | 0 | 154 | 154 | 168 | 109 | 0 | 0 |

Table Error: Reference source not found-6 – Values of initValue for depth_intra_mode ctxIdx

| Initialization variable | ctxIdx of depth_intra_mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| initValue | 153 | 162 | 0 | 153 | 139 | 153 | CNU | 0 |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 153 | 162 | 0 | 153 | 139 | 153 | CNU | 0 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| initValue | 153 | 162 | 0 | 153 | 139 | 153 | CNU | 0 |

Table Error: Reference source not found-19-1 – Values of initValue for edge_start_left_flag and num_edge_codes ctxIdx

| Initialization variable | ctxIdx of edge_start_left_flag and num_edge_codes |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 153 | 153 | 153 |

Table Error: Reference source not found-19-2 – Values of initValue for edge_code ctxIdx

| Initialization variable | ctxIdx of edge_code |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 153 | 153 | 153 |

- H.9.3.3 Binarization process

- G.9.3.3.1 General

The specifications in subclause 9.3.3.1 apply with the following modifications.

- Table Error: Reference source not found-7 is appended to the end of Table 9-32.

Table Error: Reference source not found-7 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_unit( ) | iv_res_pred_weight_idx | TR | cMax = 2, cRiceParam = 0 |
|  | ic_flag | FL | cMax = 1 |
|  | inter_sdc_flag | FL | cMax = 1 |
|  | inter_sdc_resi_abs_minus1 | UEG0 | [Ed. (GT) To be specified] |
|  | inter_sdc_resi_sign_flag | FL | cMax = 1 |
| depth_mode_parameters( ) | depth_intra_mode | TR | cMax = depthIntraModeMaxLen, cRiceParam = 0 |
|  | wedge_full_tab_idx | FL | cMax = wedgeFullTabIdxBits[ log2PbSize ] (defined in Table Error: Reference source not found-8) |
|  | depth_dc_flag | FL | cMax = 1 |
|  | depth_dc_abs | UEG0 | [Ed. (GT) To be specified] |
|  | depth_dc_sign_flag | FL | cMax = 1 |
|  | edge_start_left_flag | FL | cMax = 1 |
|  | edge_start_position | FL | cMax = edgeStartPositionLenBits[lob2PbSize] (defined in Table H-21-1) |
|  | num_edge_codes | H.9.3.3.11 | Defined in H.9.3.3.11 |
|  | edge_code | H-21-2 | Defined in H-21-4 |

Table Error: Reference source not found-8-1 –Values of edgeStartPositionLenBits[ log2PUSize ]

| Initialization variable | wedgeFullTabIdxBits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 | 6 |
| Value | 2 | 3 | 4 | 5 | 6 |

- H.9.3.3.11 Binarization process for num_edge_codes

- Input to this process is a request for the a syntax element num_edge_codes,

- Output of this process is the binarization of the syntax element.

The variable EdgeNumber is the number of edges obtained by num_edge_code.

The value of StartEdgeNum and BlkSize is defined in Table H-21-2 and Table H-21-3, respectively.

Table Error: Reference source not found-9-2 –Values of StartEdgeNum[ log2PUSize ]

| Initialization variable | wedgeFullTabIdxBits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 | 6 |
| Value | 2 | 4 | 8 | 16 | 32 |

Table Error: Reference source not found-10-3 –Values of BlkSize[ log2PUSize ]

| Initialization variable | wedgeFullTabIdxBits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 | 6 |
| Value | 4 | 8 | 16 | 32 | 64 |

The bin string is derived as follows:

- If ucMax is equal to BlkSize -1

- If ucMax < StartEdgeNum[log2PUSize] – 1, bin string = ucMax++

- Otherwise, bin string = ucMax + BlkSize[log2PUSize]

- Otherwise,

- If ucMax%2 is equal to 1, bin string = Blksize[log2PUSize] – 1 + ((ucMax+1)>>1)

- Otherwise, bin string = Blksize[log2PUSize] – 1 - (ucMax>>1)

- Table Error: Reference source not found-21-4 – Bin string of edge_code

| edge_code | Bin string |
| --- | --- |
| 0 | 00 |
| 1 | 01 |
| 2 | 10 |
| 3 | 110 |
| 4 | 111 |

- H.9.3.4.2 Derivation process for ctxTable, ctxIdx and bypassFlag

- H.9.3.4.2.1 General

The specifications in subclause 9.3.4.2.1 apply with the following modifications:

- Table Error: Reference source not found-11 is appended to the end of Table 9-37.

Table Error: Reference source not found-11 –Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| wedge_full_tab_idx | 0 | 0 | 0 | 0 | 0 | 0 |
| depth_dc_flag | 0 | na | na | na | na | na |
| depth_dc_abs | 0 | 0 | 0 | 0 | 0 | 0 |
| depth_dc_sign_flag | bypass | 0 | 0 | 0 | 0 | 0 |
| res_pred_flag | 0 | na | na | na | na | na |
| ic_flag | 0 | na | na | na | na | na |
| inter_sdc_flag | 0 | 0 | 0 | 0 | 0 | 0 |
| inter_sdc_resi_abs_minus1 | 0 | 0 | 0 | 0 | 0 | 0 |
| inter_sdc_resi_sign_flag | 0 | 0 | 0 | 0 | 0 | 0 |
| edge_start_left_flag | 0 | na | na | na | na | na |
| num_edge_codes | 0 | 0 | 0 | 0 | 0 | 0 |
| edge_code | 0 | 0 | 0 | na | na | na |