| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0095 |
| --- | --- |

| Title: | 3D-CE4.h related: An alternative disparity vector derivation method for ARP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jih-Sheng Tu Ching-Chieh Lin | Tel: Email: | +886-3-5915639 SuriseTu@itri.org.tw +886-3-5915758 JackLin@itri.org.tw |
| Source: | Industrial Technology Research Institute |  |  |

_____________________________

# Abstract

This contribution presents an alternative disparity vector derivation method for advanced residual prediction (ARP) in 3D-HEVC. The encoder can use DoNBDV and NBDV as DV candidates to find the best DV candidate in ARP process and then signal the index to the decoder. Experimental results show that the proposed method can achieve 0.1% BD-rate saving for the decoded/synthesized views in the common test condition.

# Introduction

In the current 3D-HEVC [1], advanced residual prediction (ARP) [2] is adopted to improve coding efficiency of inter-view residual prediction. A disparity vector from NBDV process locates an estimated block in the reference view. Then, a reference block in the reference view can be found by using the motion vector in the current block. Subtracting the reconstructed samples from the two blocks in the reference view generates ARP residual. These residuals are used to predict the residual of the current block in the current view. In addition, the encoder can find the best ARP residual with two weighting factors by rate-distortion optimization (RDO). The decision is written in the bitstream to signal the decoder. In the current ARP design, it is very important to find a good estimated block in the reference view to generate ARP residuals with less prediction error. However, the disparity vector from NBDV process is not precise enough in some situations.

# Proposed method

An alternative disparity vector derivation method is proposed to improve ARP coding efficiency. In our method, DoNBDV and NBDV are used as DV candidates in ARP. The encoder can find the best DV candidate used in ARP according to RDO criterion, and signal the result to the decoder. In our design, the 0.5 weighting option is replaced by using DoNBDV as DV candidate. Therefore, the DV list is identical to the current 3D-HEVC.

# Experimental results

The proposed method is integrated on HTM-9.0r1 under a common test condition [3]. The experimental result is shown in Table 1. It can be seen that the proposed method can achieve 0.1% BD-rate saving for the decoded and synthesized views in the common test condition.

TABLE 1: proposed method compared to HTM-9.0r1

# Conclusion

In this proposal, an alternative disparity vector derivation method is proposed to improve ARP coding efficiency. The proposed method improves the accuracy of the estimated DV. Experimental results report that the average bitrate reduction of 0.3% for two dependent views respectively, and 0.1% average bitrate reduction for video PSNR vs. video bitrate, video PSNR vs. total bitrate and synth PSNR vs. total bitrate in the common test condition.

# Patent rights declaration(s)

Industrial Technology Research Institute may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- L. Zhang, G. Tech, K. Wegner, S. Yea, “Test Model 6 of 3D-HEVC and MV-HEVC,” 6th JCT-3V meeting, JCT3V-F1005, Geneva, CH, October 2013.

- L. Zhang, Y. Chen, X. Li, M. Karczewicz, “CE4: Advanced residual prediction for multiview coding,” 4th JCT-3V meeting, JCT3V-D0177, Incheon, KR, April 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Geneva, CH, October 2013.