| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0098 |
| --- | --- |

| Title: | 3D-CE2 related: Performance evaluation on depth Merge mode candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

This contribution evaluates coding performance of depth Merge candidates. Currently, five 3D candidates, which consist of MPI, IVMV, DV, Shift IVMV and Shift DV, are added in the Merge lists in addition to HEVC candidates. However, it is observed that DV, Shift IVMV, and Shift DV have no coding gain at all. Also, IVMV has very small coding performance of less than 0.1%. Hence, this contribution proposes to remove these three candidates (DV, Shift IVMV, and Shift DV) in the depth Merge list construction and suggests evaluating the performance of the IVMV candidate in CE.

# Performance of 3D Depth Merge Candidate

For 3D depth Merge list construction, MPI, IVMV, DV, Shift IVMV and Shift DV candidates were added in the previous meetings. Tables 1 to 5 illustrate the performance of each candidate. MPI achieves the significantly high coding performance, while the remaining candidates do not have enough coding gain. Especially, some candidates have no coding gain at all. Based on the evaluation results, this contribution proposes to remove DV, Shift IVMV and Shift DV candidates in the depth Merge list.

Table 1. Performance of MPI

Table 2. Performance of IVMV

Table 3. Performance of DV

Table 4. Performance of Shift IVMV

Table 5. Performance of Shift DV

# Proposed Text

- H.8.5.3.2.1 Derivation process for luma motion vectors for merge mode

…

- 7. The variable availableFlagIvDCShift is set equal to 0, and when DepthFlag is equal to 0, availableFlagIvMCShift is equal to 0, and i is less than ( 5 + NumExtraMergeCand ), the derivation process for the shifted disparity merging candidate as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variables nPbW and nPbH, and the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N, of every candidate N being in extMergeCandList, extMergeCandList, and i as the inputs and the outputs are the flag availableFlagIvDCShift, the prediction utilization flags predFlagL0IvDCShift and predFlagL1IvDCShift, the reference indices refIdxL0IvDCShift and refIdxL1IvDCShift, and the motion vectors mvL0IvDCShift and mvL1IvDCShift.

…

- H.8.5.3.2.10 Derivation process for inter-view merge candidates

…

The availability flags availableFlagIvMC, availableFlagIvMCShift and availableFlagIvDC are set equal to 0, the variable refIdxLXIvMC, refIdxLXIvMCShift and refIdxLXIvDC are set equal to –1, and both components of mvLXIvMC, mvLXIvMCShift and mvLXIvDC are set equal to 0.

…

If DepthFlag is equal to 0, the shifted temporal inter-view motion vector merging candidate is derived by the following ordered steps.

…

If DepthFlag is equal to 0, the disparity inter-view motion vector merging candidate is derived by the following ordered steps.

…

- H.8.5.3.2.12 Derivation process for a disparity inter-view motion vector candidate

…

  - When PicOrderCnt( RefPicListX[ i ] ) is equal to the PicOrderCntVal, ViewIdx( RefPicListX[ i ] ) is equal to refViewIdx and availableFlagLXInterView is equal to 0 the following applies:

        - availableFlagLXInterView = 1 		(Error: Reference source not found-1)

        - mvLXInterView[ 0 ] = mvDisp[ 0 ]	(Error: Reference source not found-2)

        - mvLXInterView[ 1 ] = 0		(Error: Reference source not found-3)

refIdxLX = i

# Test Results

Based on the performance of each candidate in the depth Merge mode list construction, this contribution proposes to remove DV, Shift IVMV and Shift DV candidates in the depth Merge list. The result shows that these three candidates have no coding gain at all.

Table 6. Performance when DV, Shift IVMV and Shift DV candidates are removed

# Conclusion

All of the depth Merge candidates were evaluated on the top of 3D-HTM 9.0r1. Based on the evaluation results, this contribution recommends removing DV, Shift IVMV and Shift DV candidates in the depth Merge list.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).